| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0063-v1 |
| --- | --- |

| Title: | On MV-HEVC profiles |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Hongbin Liu2, Li Zhang1, Shaohui Jiao2, Chao Hu2, Jianyong Cui2, Guangbao Xu2 18910 University Center Lane San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Email: | {yekui.wang, liuhongbin.01, lizhang.idm, jiaoshaohui, huchao.jason, cuijianyong, xuguangbao} @bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

It is asserted that, although the syntax of MV-HEVC allows for an output layer set (OLS) in an MV-HEVC bitstream to have both texture layers and depth layers, the profile definitions do not allow this. Recently, applications have come up, e.g., virtual reality, that benefit from all the following three aspects simultaneously: 1) using inter-view prediction between different views, 2) having depths associated with the coded views to be available to the decoder and the renderer, and 3) using existing HEVC hardware encoder/decoder designs. It is also asserted that in such applications it is desirable that the bit depth of the depth component can be of up to 16 bits.

This contribution proposes the following:

- Update the definitions of the four new profiles, namely the Multiview Main 10, Multiview Monochrome, Multiview Monochrome 10, and Multiview Monochrome 12 profiles, to allow for an OLS containing a layer conforming to such a profile to have both texture layers and depth layers.

- Add a new profile, named the Multiview Monochrome 16 profile, where it is also allowed for an OLS containing a layer conforming to this profile to have both texture layers and depth layers.

The proposed changes, including some minor editorial changes not summarized above, are included in an attachment, with all the proposed changes marked relative to JVET-AE1006-v1.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).