| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0066-v1 |
| --- | --- |

| Title: | EE2: Test 2.9 Enable DBV in single tree |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Hongtao Wang, Vadim Seregin, Marta Karczewicz | Email: | hanhuang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution reports the test results of EE2 Test 2.9: Enable DBV in single tree.  Test 2.9a reports the following result {Y, U, V, EncT, DecT, VmPeak} under CTC:

RA {-0.02%, 0.02%, -0.03%, 99.9%, 100.7%, 100.0%}

Test 2.9b reports the following result when DualITree is set to 0:

AI {-0.21%, -0.09%, -0.12%, 100.9%, 100.7%, 100.0%}

# Introduction

In JVET-AE0098, proposes to enable chroma DBV mode in single-tree configuration and to remove the template matching based refinement in chroma DBV mode. It’s decided to study enabling chroma DBV mode in single-tree configuration in EE2 Test 2.9.

This contribution reports the test results of EE2 Test 2.9, wherein the constraint on DBV flag not being used for single tree case is removed.

# Test results

Table 1 shows the test result of Test 2.9a, wherein common test condition was used to conduct the test.

Table 1 Test results of Test 2.9a

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | 0.00% | -0.06% | -0.01% | 100.1% | 101.0% | 100.0% |
| Class A2 | -0.02% | -0.08% | -0.06% | 100.3% | 101.6% | 100.0% |
| Class B | -0.03% | 0.11% | -0.04% | 99.9% | 101.0% | 99.9% |
| Class C | -0.04% | 0.05% | -0.01% | 99.6% | 99.6% | 100.0% |
| Class E |  |  |  |  |  |  |
| Overall | -0.02% | 0.02% | -0.03% | 99.9% | 100.7% | 100.0% |
| Class D | -0.03% | -0.08% | -0.04% | 100.3% | 99.5% | 100.0% |
| Class F | -0.04% | -0.14% | -0.12% | 99.4% | 100.2% | 99.9% |
| Class TGM | -0.08% | -0.02% | -0.03% | 100.4% | 101.4% | 100.0% |

Table 2 shows the test result of Test 2.9b, wherein DualITree is set to 0 for both anchor and the test.

Table 2 Test result of Test 2.9b

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 DualITree off |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.04% | 0.07% | 0.23% | 100.5% | 100.8% | 100.0% |
| Class A2 | -0.27% | 0.17% | 0.33% | 100.6% | 100.2% | 100.0% |
| Class B | -0.24% | -0.05% | -0.35% | 100.7% | 100.2% | 100.0% |
| Class C | -0.23% | 0.01% | -0.05% | 101.2% | 101.7% | 100.0% |
| Class E | -0.25% | -0.71% | -0.59% | 101.4% | 100.5% | 100.0% |
| Overall | -0.21% | -0.09% | -0.12% | 100.9% | 100.7% | 100.0% |
| Class D | -0.07% | -0.27% | -0.20% | 100.3% | 100.4% | 100.0% |
| Class F | -0.14% | -0.25% | 0.07% | 101.5% | 101.1% | 100.0% |
| Class TGM | -0.12% | -0.09% | -0.09% | 101.0% | 99.1% | 100.0% |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).