| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0072-v1 |
| --- | --- |

| Title: | AHG12 : FIBC flag inherits IntraTMP-FLM flag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Menghu Jia, Ying Gao, Yaxian Bai Shaowei Xie, Cheng Huang, Ping Wu | Tel: Email: | jia.menghu@zte.com.cn gao.ying@zte.com.cn bai.yaxian@zte.com.cn xie.shaowei@zte.com.cn huang.cheng5@zte.com.cn ping.wu@zte.com.cn |
| Source: | ZTE Corporation |  |  |

_____________________________

- Abstract

This contribution proposes that FIBC flag in the IBC merge mode can inherit the IntraTMP-FLM flag. In the IBC merge mode, the FIBC flag is inferred from the merge candidate. When the merge candidate is come from the IntraTMP coded block, the FIBC flag can be inferred through IntraTMP-FLM flag.

The proposed method is implemented on top of ECM-10.0 software, it shows coding gains as follows:

AI:

Class F：   -0.02% / 0.04% / 0.02%, 100.4% / 99.8%

ClassTGM：-0.02% / -0.03% /-0.03%, 100.2% / 100.1%

Introduction

FIBC

Filtered IBC (FIBC) [1] mode is introduced in ECM-10.0, where a filter is applied to IBC predictor, which is derived by minimizing MSE between current and reference template.

Output of the filter is calculated as follows:

Figure 1: Spatial part of the filter

predLumaVal = c0C + c1N + c2S + c3E + c4W + c5P + c6B

The nonlinear term P is represented as power of two of the center sample C and scaled to the sample value range of the content:

P = ( C*C + midVal ) >> bitDepth

The bias term B represents a scalar offset between the input and output and is set to middle luma value (512 for 10-bit content).

The filter coefficients ci are calculated by minimising the MSE between the reference template and current template, very similar to the CCCM process.

One extra indication flag is introduced for the FIBC which is signaled conditioned on the IBC-LIC flag. Specifically, when the IBC-LIC flag is true, the flag is signaled and used to indicate whether the FIBC is applied to the current block or not. For IBC merge modes, this filtering mode is inherited when merge mode list is constructed, so there is no extra signaling. FIBC is only enabled in SCC sequences

However, in the IBC merge mode of ECM10.0, FIBC will be disabled when the merge candidate is come from the IntraTMP coded block,

IntraTMP-FLM

In ECM10.0, a linear filter is applied to the prediction of IntraTMP (IntraTMP-FLM) [2]. IntraTMP-FLM consists of a 5-tap plus sign shape spatial component and a bias term. The input to the spatial 5-tap component of the filter consists of a center (C) sample in the reference block which is at corresponding locations with the sample in the current block to be predicted and its above/north (N), below/south (S), left/west (W) and right/east (E) neighbors as Fig1.

The bias term B represents a scalar offset between the input and output and is set to middle luma value (512 for 10-bit content).

Output of the filter is calculated as follows:

predLumaVal = c0C + c1N + c2S + c3E + c4W + c5B

The filter coefficients ci are calculated by minimising the MSE between the reference template and current template, as shown in Fig2.

Figure 2: Reference area used to derive the filter coefficients

FIBC and IntraMTP-FLM are basically the same, except for one nonlinear term. Therefore, the FIBC flag can inherit the IntraTMP-FLM flag.

Proposed approach

In this contribution, it is proposed that the FIBC flag can be inherited from IntraTMP-FLM flag from IntraTMP candidates.

In the IBC merge mode, the FIBC flag is inferred from the merge candidate. When the merge candidate is come from the IntraTMP coded block, the FIBC flag can be inferred through IntraTMP-FLM flag.

- When IntraTMP-FLM flag is true, FIBC flag is true;

- When IntraTMP-FLM flag is false, FIBC flag is false;

Experimental results

The proposed method was implemented based on ECM-10.0 under common test condition [3]. Table 1 presents a summary of the results with all intra configurations.

Table 1. Simulation results of the proposed method

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.02% | 0.04% | 0.02% | 100.4% | 99.8% |
| Class TGM | -0.02% | -0.03% | -0.03% | 100.2% | 100.1% |

References

- H.-J. Jhu, et. al., “EE2-2.5: Filtered Intra Block Copy (FIBC),” JVET-AE-159, July 2023.

- J.-Y. Huo, et. al., “EE2-1.15a: Intra template matching (Intra TMP) based on linear filter model,” JVET-AD0112, April 2023.

- M. Karczewicz, et. al., “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, February 2022.

Patent rights declaration(s)

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).