| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0081-v1 |
| --- | --- |

| Title: | EE2-2.8: DBV improvement |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luhang Xu, Yue Yu, Haoping Yu, Jonathan Gan, Dong Wang (OPPO) | Tel: Email: | xuluhang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution reports the results of the EE2-2.8 test. In this test, the signalling of chroma DM mode and DBV mode is changed when one of the five corresponding luma positions is coded with either intraTMP or IBC. It is reported that the performance against ECM-10.0 is as follows:

AI: 		-0.00% (Y), 0.06% (U), 0.02% (V), 100.2% (EncT), 100.5% (DecT)

RA: 	-0.00% (Y), -0.01% (U), -0.05% (V), 99.9% (EncT), 100.1% (DecT)

Introduction

The DBV flag is signaled if one of the five corresponding luma positions is coded with either intraTMP or IBC.

Figure-1. the five corresponding luma positions (C, TL, TR, BL, BR)

If the current chroma block is coded with the DBV mode, the BV of the corresponding luma block is scaled and used to directly form a chroma prediction from the reconstructed chroma area. If the current chroma block is not coded with DBV and the corresponding luma block is coded with IBC or IntraTMP, it will be coded with DM, or DIMD, or one of the modes in the chroma intra mode list.

The chroma intra mode list is filled with planar, DC, horizontal and vertical modes by default. If DM or DIMD mode also uses one of these four default modes, the redundant mode in the list will be replaced by angular mode 67.

The corresponding binarization for the intra chroma mode when one of the five corresponding luma positions is coded with intraTMP or IBC is shown in Table-1.

Table-1 The binarization process for intra_chroma_pred_mode when one of the five corresponding luma positions is coded with intraTMP or IBC in ECM-10

| intra_chroma_pred_mode | bin string | chroma intra mode |
| --- | --- | --- |
| 0 | 11100 | list[0] |
| 1 | 11101 | list[1] |
| 2 | 11110 | list[2] |
| 3 | 11111 | list[3] |
| 4 | 110 | DIMD chroma |
| 5 | 10 | DM |
| 6 | 0 | DBV |

Proposed method

In this contribution, the signalling of chroma DM mode and DBV mode is changed. When one of the five corresponding luma positions is coded with either intraTMP or IBC and DBV mode is not true, DM flag will be skipped.

Table-2 The binarization process for intra_chroma_pred_mode when one of the five corresponding luma positions is coded with intraTMP or IBC in the proposed method

| intra_chroma_pred_mode | bin string | chroma intra mode |
| --- | --- | --- |
| 0 | 11100 | list[0] |
| 1 | 11101 | list[1] |
| 2 | 11110 | list[2] |
| 3 | 11111 | list[3] |
| 4 | 110 | DIMD chroma |
| 5 | 10 | DM |
| 65 | 0 | DBV |

Simulation results

The proposed method is implemented on top of the ECM-10.0 software [1], and tested based on the common test condition [2]. The test results are summarized in Table-3 and Table-4.

Table-3. The proposed method over ECM-10.0 (AI)

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over anchor |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.01% | 0.07% | 0.00% | 100.0% | 100.2% | 100.0% |
| Class A2 | -0.01% | 0.00% | 0.03% | 99.9% | 100.4% | 100.0% |
| Class B | 0.01% | 0.02% | 0.00% | 100.3% | 100.1% | 100.0% |
| Class C | -0.01% | 0.09% | 0.16% | 100.2% | 101.8% | 100.0% |
| Class E | 0.01% | 0.15% | -0.14% | 100.5% | 99.8% | 100.0% |
| Overall | 0.00% | 0.06% | 0.02% | 100.2% | 100.5% | 100.0% |
| Class D | 0.01% | -0.04% | -0.10% | 100.4% | 100.3% | 100.0% |
| Class F | -0.03% | 0.09% | -0.08% | 100.8% | 99.9% | 100.0% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! |

Table-4. The proposed method over ECM-10.0 (RA)

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over anchor |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | 0.02% | -0.11% | -0.02% | 100.1% | 99.9% | 99.3% |
| Class A2 | 0.02% | -0.16% | 0.02% | 99.8% | 99.7% | 100.2% |
| Class B | -0.02% | 0.17% | -0.01% | 99.9% | 99.9% | 100.1% |
| Class C | -0.02% | -0.03% | -0.17% | 99.7% | 100.6% | 100.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.00% | -0.01% | -0.05% | 99.9% | 100.1% | 99.9% |
| Class D | -0.02% | -0.03% | -0.10% | 100.0% | 100.2% | 100.0% |
| Class F | -0.08% | 0.15% | 0.16% | 99.9% | 100.3% | 100.1% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! |

Conclusions

This contribution reports the results of EE2-2.8. The reported performance change of the proposed method is 0.00% for AI configuration and 0.00% for RA configuration. It is suggested to adopt the proposed method in the next version of ECM reference software.

References

- ECM-10.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-10.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AD2017, April, 2023.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).