| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0089-v1 |
| --- | --- |

| Title: | AhG10: Lagrange multiplier and QP adaptation at CTU-level for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongwei Guo, Ce Zhu, Lei Luo, Junjie Chen (UESTC) Yongkai Huo, Yutain Liu (Transsion) | Email: | hwguo@uestc.edu.cn eczhu@uestc.edu.cn luolei1010@uestc.edu.cn 202221010107@std.uestc.edu.cn yongkai.huo@transsion.com yutian.liu@transsion.com |
| Source: | University of Electronic Science and Technology of China, Transsion Holdings |  |  |

_____________________________

- Abstract

This contribution proposes a Lagrange multiplier and QP adaptation at CTU-level for VVC. The proposed Lagrange multiplier and QP adaptation algorithm was implemented on VTM-21.2. Compared to the original VTM-21.2, the proposed method demonstrates coding efficiency improvements of 4.27%, 3.64%, X.XX%, and 0.25% in low delay B, low delay P, random access with GOP size of 32, and random access with GOP size of 16 configurations, respectively.

Introduction

VVC employs a hierarchical QP setting at frame level within a GOP for three test conditions: low delay B, low delay P, and random access. In comparison to a flat QP setting scheme, the hierarchical QP setting tends to perform better, as it enhances the coding quality of key frames that are frequently referenced. Likewise, we propose an optional CTU-level Lagrange multiplier and QP adaptation scheme, aiming to enhance the coding quality of regions within a frame that are frequently referenced.

Proposal

In this proposal, we first estimate a distortion propagation factor (DPF) for each 16×16 or 32×32 block in the frame to be encoded. Then, a weight coefficient of each CTU is calculated according to DPFs. Finally, the adaptive Lagrange multiplier and QP are determined by:

(1)

(2)

where ,  and are the Lagrange multiplier, weight coefficient and QP of jth CTU in the frame to be encoded, and  is the Lagrange multiplier of the frame to be encoded which is calculated by using the original scheme in VVC.

Furthermore, the weight coefficient is calculated by:

(3)

(4)

(5)

(6)

where  is the DPF of a block, which is obtained from subsection 2.1 or 2.2.  is the number of blocks in a frame, and  is the number of blocks in the jth CTU.

Estimating DPF in low delay configuration

The frame to be encoded is first pre-encoded with a 32×32 block integer pixel inter-prediction. The purpose of pre-encoding is to roughly obtain the coding distortion of a block and its motion compensation predicted (MCP) error, which will be used to calculate the DPF of a block. It is worth noting that the pre-encoding process can be easily implemented according to the function "EncSlice::precompressSlice( )" in VTM.

For B or P frame, if the temporal level of the frame to be encoded is 0 (i.e., the frame is a key frame), the DPF is calculated by:

(7)

where  and  are the coding distortion of a block and its MCP error, and  is the number of frames in the next GOP.

If the temporal level of the frame to be encoded is more than 0 (i.e., the frame is not a key frame), the DPF is calculated by:

(8)

where  is GOP size, and rPOC is relative picture order count (POC) in a GOP, which can be calculated by . Obviously,  represents the number of the subsequent frames in the current GOP.

Estimating DPF in random access configuration

In random access configuration, if the frame to be encoded is an Intra frame or its temporal level is equal to the maximum value (GOP size of 32 corresponds to 5, and GOP size of 16 corresponds 4), the frame does not need to be pre-encoded, and the DPF is directly set to 1.0. For other cases, the frame to be encoded is first pre-encoded with a 16×16 block integer pixel inter-prediction. Then, the DPF is calculated by:

(9)

Note that in low delay configuration, the block size is set as 32×32, and in random access configuration, the block size is set as 16×16.

Results

Based on the platform in VTM-21.2 [1], coding performance experiment was conducted following the common test conditions defined in [2]. For LDB configuration, experimental results are shown in Table 1. The proposed method can obtain -4.27%, -5.26%, and -3.91% BD-rate on average for Y, Cb, and Cr components, respectively.

Table 1. Performance of the proposed method on top of VTM-21.2 (LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-21.2 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -2.82% | -2.99% | -2.22% | 102% | 99% |
| Class C | -5.39% | -5.75% | -3.58% | 101% | 100% |
| Class E | -5.20% | -8.37% | -7.15% | 101% | 100% |
| Overall | -4.27% | -5.26% | -3.91% | 101% | 100% |
| Class D | -1.20% | 0.67% | -0.55% | 102% | 100% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

For LDP configuration, experimental results are shown in Table 2. The proposed method can obtain -3.64%, -4.23%, and -2.49% BD-rate on average for Y, Cb, and Cr components, respectively.

Table 2. Performance of the proposed method on top of VTM-21.2 (LDP)

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-21.2 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -2.14% | -2.08% | -1.33% | 101% | 100% |
| Class C | -5.00% | -4.68% | -2.13% | 101% | 99% |
| Class E | -4.33% | -7.22% | -4.88% | 100% | 99% |
| Overall | -3.64% | -4.23% | -2.49% | 101% | 99% |
| Class D | -1.14% | 0.08% | 1.58% | 101% | 100% |
| Class F | -4.46% | -6.93% | -4.37% | 102% | 100% |

For RA configuration with GOP size of 32, experimental results are shown in Table 3. The proposed method can obtain -X.XX%, -X.XX%, and -X.XX% BD-rate on average for Y, Cb, and Cr components, respectively.

Table 3. Performance of the proposed method on top of VTM-21.2 (RA with GOP size of 32)

|  | Random access Main10 GOP32 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-21.2 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.16% | -0.04% | 0.23% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.10% | -0.06% | 0.03% | 101% | 100% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

For RA configuration with GOP size of 16, experimental results are shown in Table 4. The proposed method can obtain -0.25%, 0.17%, and 0.18% BD-rate on average for Y, Cb, and Cr components, respectively.

Table 4. Performance of the proposed method on top of VTM-21.2 (RA with GOP size of 16)

|  | Random access Main10 GOP16 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-21.2 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | 0.72% | 0.01% | 101% | 99% |
| Class A2 | -0.56% | -0.18% | 0.17% | 101% | 99% |
| Class B | -0.27% | -0.05% | -0.03% | 101% | 99% |
| Class C | -0.12% | 0.28% | 0.59% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.25% | 0.17% | 0.18% | 101% | 99% |
| Class D | -0.08% | 0.12% | 0.36% | 101% | 100% |
| Class F | 0.05% | 0.22% | 0.55% | 102% | 100% |

It is noticed that the encoding time of the proposed method is almost the same as that of the original VTM-21.2. The reason is that the pre-encoding only uses integer-pixel inter-prediction with a fixed size of 32×32 or 16×16. The time complexity of pre-encoding is negligible compared with the time-consuming full encoding.

Conclusion

It has been shown that Lagrange multiplier and QP adaptation at CTU-level can provide coding performance benefits compared to the hierarchical QP setting at frame level. The proposed scheme can be easily implemented according to the function "EncSlice::precompressSlice( )" in VTM.

References

- VTM-21.2, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/releases/VTM-21.2

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “VTM common test conditions and software reference configurations for SDR video”, JVET-T2010, Meeting by Teleconference, October 2020

Patent rights declaration(s)

University of Electronic Science and Technology of China may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Shenzhen Transsion Holdings Co., Ltd. (Transsion) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).