| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0097-v1 |
| --- | --- |

| Title: | [AHG9] On description of picture 0 in source picture timing information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Junghak Nam Seung hwan Kim Jaehyun Lim | Email: | dr.hendry@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

It is asserted that the current description regarding picture 0 in the draft text of source picture timing information (SPTI) SEI message is not clear which may cause confusion. This contribution proposes some updates to the current text as follows:

- Specify that picture 0 for computation of SourcePictureTime[ 0 ] of an SPTI SEI message is the picture in the same access unit that contains the SEI message.

- Add a constraint that picture 0 shall not be a synthesized picture since it does not have corresponding source picture.

# Problem Statement

In the 31st JVET meeting, the first design of source picture timing information (SPTI) SEI message was agreed to be included in the Technologies under Consideration for future extensions of VSEI (JVET-AE2032). The semantics of SPTI SEI message includes description of picture n and its source picture timing computation as shown by excerpt below:

…

When picture n is a picture that is output having TemporalId less than or equal to i and is not the first picture of the bitstream that is output, the value of the variable SourcePictureTime[ n ] is derived as follows:

–	If temporalReversalFlag is equal to 0, SourcePictureTime[ n ] = SourcePictureTime[ previousPicInOutputOrder ] + SourcePictureInterval[ i

–	Otherwise, temporalReversalFlag is equal to 1, SourcePictureTime[ n ] = SourcePictureTime[ previousPicInOutputOrder ] − SourcePictureInterval[ i ]

where previousPicInOutputOrder is the last picture that is output having TemporalId less than or equal to i that precedes picture n in output order (if any). When the value of SourcePictureTime[ 0 ] is not provided by external means not specified in this document, the value of SourcePictureTime[ 0 ] is inferred to be 0.

…

It is asserted that in this design version, aspects regarding picture 0 is not clearly specified which may lead to the following problems:

- Picture 0 is supposed to be the first output picture whose association with source picture is provided by an SPTI SEI message but it is not clear which picture that is. The lack of this more detail description may cause confusion since the following pictures can be considered as picture 0: the first picture in the bitstream, the first picture in the CVS / CLVS, or the first picture associated with the SEI message.

- The current design includes the feature to identify whether picture(s) associated with an SPTI SEI message is synthesized pictures or not. Picture 0 should not be associated with a synthesized picture since synthesized picture does not have associated source picture. However, currently, this is not constrained yet.

# Proposal

We propose the following updates to the draft text of SPTI SEI message to overcome the asserted problems above:

- Specify that picture 0 for computation of SourcePictureTime[ 0 ] of an SPTI SEI message is the picture in the same access unit that contains the SEI message.

- Add a constraint that picture 0 shall not be a synthesized picture since it does not have corresponding source picture.

# Text implementation

…

Let picture 0 be picture that is the same access unit as the SPTI SEI message and picture n, for n greater than 0, be the n-th picture from picture 0. When picture n is a picture that is output having TemporalId less than or equal to i and is not the first picture of the bitstream that is output, the value of the variable SourcePictureTime[ n ] is derived as follows:

–	If temporalReversalFlag is equal to 0, SourcePictureTime[ n ] = SourcePictureTime[ previousPicInOutputOrder ] + SourcePictureInterval[ i ]

–	Otherwise, temporalReversalFlag is equal to 1, SourcePictureTime[ n ] = SourcePictureTime[ previousPicInOutputOrder ] - SourcePictureInterval[ i ]

where previousPicInOutputOrder is the last picture that is output having TemporalId less than or equal to i that precedes picture n in output order (if any). When the value of SourcePictureTime[ 0 ] is not provided by external means not specified in this document, the value of SourcePictureTime[ 0 ] is inferred to be 0.

spti_sublayer_synthesized_picture_flag[ i ], when present, equal to 1 indicates that decoded output pictures belonging to the ith temporal sublayer are synthesized and do not correspond to unmodified original source pictures. spti_sublayer_synthesized_picture_flag[ i ] equal to 0 provides no such indication. When not present, the value of spti_sublayer_synthesized_picture_flag[ i ] is inferred to be equal to 0.

It is constrained that picture 0 shall not be a picture that belongs to temporal sublayer that is synthesized.

…

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).