| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0100-v1 |
| --- | --- |

| Title: | [AHG9] On miscellaneous aspects of source picture timing information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Junghak Nam Seung hwan Kim Jaehyun Lim | Email: | dr.hendry@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution proposes several modifications that are asserted to provide improvement to some aspects of the source picture timing information (SPTI) SEI message design.

# On the computed value of ElementalSourcePictureInterval

## Problem statement

In the current text draft, the computed value of ElementalSourcePictureInterval could be equal to 0 when spti_num_units_in_elemental_source_picture_interval is equal to 0. This is asserted not correct since it will cause the value of SourcePictureInterval[ i ] also be equal to 0.

## Proposal

We propose two options to overcome the asserted problem above:

Option 1: specify value range for spti_time_scale and spti_num_units_in_elemental_source_picture_interval to ensure that the computation of ElementalSourcePictureInterval will be greater than 0 and shall not be equal to infinite value. The values of spti_time_scale and spti_num_units_in_elemental_source_picture_interval shall range from 1 to 232 – 1, inclusive.

Option 2: change spti_time_scale to spti_time_scale_minus1 and spti_num_units_in_elemental_source_picture_interval_minus1.

## Text implementation

### Option 1

…

spti_time_scale specifies the number of time units that pass in one second. The value of spti_time_scale shall not be equal to 0. For example, a time coordinate system that measures time using a 27 MHz clock has an spti_time_scale of 27,000,000. The value of spti_time_scale shall be in the range from 1 to 232 – 1, inclusive.

spti_num_units_in_elemental_source_picture_interval specifies the number of time units of a clock operating at the frequency spti_time_scale Hz that corresponds to the indicated elemental source picture interval of consecutive pictures in output order in the CLVS. The value of spti_num_units_in_elemental_source_picture_interval shall be in the range from 1 to 232 – 1, inclusive.

The indicated elemental source picture interval, also to be denoted by the variable ElementalSourcePictureInterval, in units of seconds, is equal to the quotient of spti_num_units_in_elemental_source_picture_interval divided by spti_time_scale. For example, to represent an elemental source picture interval equal to 0.04 seconds, spti_time_scale may be equal to 27,000,000 and spti_num_units_in_elemental_source_picture_interval may be equal to 1,080,000.

…

### Option 2

…

| source_picture_timing_info( payloadSize ) { | Descriptor |
| --- | --- |
| spti_cancel_flag | u(1) |
| if( !spti_cancel_flag ) { |  |
| spti_persistence_flag | u(1) |
| spti_source_picture_timing_type | u(8) |
| spti_source_timing_equals_output_timing_flag | u(1) |
| if( !spti_source_timing_equals_output_timing_flag ) { |  |
| spti_time_scale_minus1 | u(32) |
| spti_num_units_in_elemental_source_picture_interval_minus1 | u(32) |
| spti_max_sublayers_minus_1 | u(3) |
| for( i = 0; i <= spti_max_sublayers_minus1; i++ ) { |  |
| spti_sublayer_source_picture_interval_scale_factor[ i ] | ue(v) |
| spti_sublayer_synthesized_picture_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |

…

spti_time_scale_minus1 plus 1 specifies the number of time units that pass in one second. The value of spti_time_scale shall not be equal to 0. For example, a time coordinate system that measures time using a 27 MHz clock has an spti_time_scale of 27,000,000. The value of spti_time_scale shall be in the range from 0 to 232 – 1, inclusive.

spti_num_units_in_elemental_source_picture_interval_minus1 plus 1 specifies the number of time units of a clock operating at the frequency spti_time_scale Hz that corresponds to the indicated elemental source picture interval of consecutive pictures in output order in the CLVS. The value of spti_num_units_in_elemental_source_picture_interval shall be in the range from 0 to 232 – 1, inclusive.

The indicated elemental source picture interval, also to be denoted by the variable ElementalSourcePictureInterval, in units of seconds, is equal to the quotient of (spti_num_units_in_elemental_source_picture_interval_minus1 + 1) divided by (spti_time_scale_minus1 + 1). For example, to represent an elemental source picture interval equal to 0.04 seconds, spti_time_scale may be equal to 27,000,000 and spti_num_units_in_elemental_source_picture_interval may be equal to 1,080,000.

…

# On temporal reversal type and persistence of the SEI

## Problem statement

It is asserted that when an SPTI SEI message includes temporal reversal type (i.e., the value of temporalReversalFlag is equal to 1), it works only when it involves several pictures. Thus, it does not make sense if the value of spti_persistence_flag is equal to 0 in such case.

## Proposal

We propose to add a constraint such that when temporalReversalFlag is equal to 1, the value of spti_persistence_flag shall be equal to 1.

## Text implementation

…

The variable temporalReversalFlag is equal to ( spti_source_picture_timing_type & 0x10 )? 1 : 0.

When temporalReversalFlag is equal to 1, the value of spti_persistence_flag shall not be equal to 0.

…

# On synthesized temporal sublayer

## Problem statement

It is asserted that the two cases below do not make sense for synthesized temporal sublayer:

- Case 1: a temporal sublayer contains synthesized pictures has higher temporal sublayer that is not synthesized.

- Case 2: a bitstream only contains temporal sublayers that are synthesized

## Proposal

We propose the following constraints are added into the current text draft:

- Specifies a constraint such that when spti_sublayer_synthesized_picture_flag[ i ] is equal to 0 and i is greater than 0, it is constrained that the value of spti_sublayer_synthesized_picture_flag[ i – 1 ] shall not be equal to 1.

- Specifies a constraint such that there shall be at least one temporal sublayer containing non-synthesized pictures.

## Text implementation

…

spti_sublayer_synthesized_picture_flag[ i ], when present, equal to 1 indicates that decoded output pictures belonging to the ith temporal sublayer are synthesized and do not correspond to unmodified original source pictures. spti_sublayer_synthesized_picture_flag[ i ] equal to 0 provides no such indication. When not present, the value of spti_sublayer_synthesized_picture_flag[ i ] is inferred to be equal to 0. When spti_sublayer_synthesized_picture_flag[ i ] is equal to 0 and i is greater than 0, it is constrained that the value of spti_sublayer_synthesized_picture_flag[ i − 1 ] shall not be equal to 1. It is constrained that the value of spti_sublayer_synthesized_picture_flag[ i ] shall be equal to 0 for at least one value of i in the range of 0 to spti_max_sublayers_minus1, inclusive.

…

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).