| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0102-v1 |
| --- | --- |

| Title: | EE1-1.1.2 Complexity-performance trade off of decomposition |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | D. Rusanovskyy, Y.Li M. Karczewicz (Qualcomm) | Email: | dmytror@qti.qualcomm.com yli30@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

Test EE1-1.2 studies the complexity-performance trade-off optimization of the convolution decomposition in HOP, originally proposed in JVET-AE0164.

This test introduces reduced ranking of the two-components decomposition in residual blocks of the backbone. In sub-test EE1-1.1.2.a the change of the parameter C31 to 48 is tested. The complexity of the model is reduced by 11% to 426kMAC/pixel (block-wise 128x128). In sub-test EE1-1.1.2.b, order of the decomposition (horizontal-vertical or vertical-horizontal) is alternated every block in the backbone to mitigate the impact of the rank reduction on decomposition symmetry.

Comparing to the HOP anchor (nnIntra ON), the following results are reported:

Sub-test EE1-1.1.2.a provides {0.1%, 1.2%, 1.0%} and {0.2%, 2.5%, 1.0%} bd-rate penalty with 11% complexity reduction in kMAC/pixel terms. Run time is reportedly reduced to 85% and 75% on average for encoder and decoder, respectively.

Sub-test EE1-1.1.2.b provides {0.1%, 0.2%, 0.4%} and {0.1%, 1.0%, 0.4%} bd-rate penalty with 11% complexity reduction in kMAC/pixel terms. Run time is reportedly reduced to 83% and 68% on average for encoder and decoder, respectively.

# Introduction

A Unified Filter Architecture for the High-performance Operation Point (HOP), block diagram is shown in Figure 1, was defined at JVET-AD0380. Training strategy and intermediate results were presented in JVET-AE0191 and complete inference results for Stage3 HOP training are reported in JVET-AF0041.

Figure 1. HOP unified architecture

# Proposal

Sub-test EE1-1.1.2.a:

In proposed method tested in EE1-1.1.2.a, the ranking of the two-components decomposition in residue block of the backbone, C31 parameter, is reduced from 64 to 48.

Sub-test EE1-1.1.2.b:

To alleviate introduced decomposition asymmetry with reduced rank being used in horizontal direction only, in sub-test EE1-1.1.2.b, the order of the decomposition (horizontal/vertical) is being switched every residue block in the backbone. Two types of the residue block in backbone introduced. Type 1 (T1), shown in Figure 2.a, residue bock implements 3x1 convolution with C31=48, followed by 1x3 convolution and Type 2 (T2), shown in Figure 2.b, residue bock first performs 1x3 convolution with C31=48, followed by the 3x1 convolution.

- b.

Figure 2 Proposed backbone residue blocks: a. - Type 1, b. - Type 2.

Figure 3 shows integrated NN filter model with blocks T1 and T2 being switching in processing order.

Figure 3 Proposed model with switched order decompositions (Type 1 and Type2) and reduced rank, C31=48.

The model was trained following HOP Stage 3 procedure. Parameters of the training and inference testing for 2 proposed method are shown in Table 1 and Table 2, respectively.

Table1. Network information for the inference

| Network Information in Inference Stage |  |  |  |
| --- | --- | --- | --- |
|  | HW environment: |  |  |
|  | GPU Type |  | N/A |
|  | Framework: |  | SADL |
|  | Number of GPUs per Task |  | 0 |
|  | Total Parameter Number |  | 1.3M /model |
|  | Parameter Precision (Bits) |  | 32 or 16 |
|  | Memory Parameter (MB) |  |  |
|  | Multiply Accumulate (kMAC/pixel) block (frame) based |  | 426 (336 frame-wise) |
|  | Total Conv. Layers |  | 154 |
|  | Total FC Layers |  | 0 |
|  | Total Memory (MB) | Float |  |
|  |  | Int16 | 2.59/model |
|  | Batch size: |  | 1 |
|  | Patch size |  | 128×128, 256×256 |
|  | Changes to network configuration or weights required to generate rate points |  |  |
|  | Peak Memory Usage |  |  |
|  | Other information: |  |  |

Table 2. Network information for the training (HOP aligned)

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: NVIDIA Tesla A100 |
|  | Framework: | PyTorch v3.1 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 20 |
|  | Batch size: | 64 |
|  | Training time: | ~240h |
|  | Training data information: | HOP dataset |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | QP {22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional | Number of iterations |  |
|  | Patch size | 144x144 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

# Simulation results

Simulations are performed based on the common test conditions for neural network-based video coding technology [5]. The NNVC6.0 SW was used for inference test of the proposed filter model.

Summary of the comparative results vs. NNVC 6.0 HOP Anchor are shown in Table 3 and Table 4 for sub-testes EE1-1.1.2 a and b, respectively. NnIntra tool was enabled for both, anchor and test. Complete results in CTC template are provided along this document.

Table 3. Summary of the inference test results, sub-test EE1-1.1.2.a vs NNVC6.0 HOP anchor, nnIntra ON.

|  | Proposed a vs NNVC6.0 HOP anchor |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  |  |  | Random Access |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.12% | 1.07% | 0.55% | 87% | 76% | 0.17% | 3.42% | 0.03% | 83% | 78% |
| Class A2 | 0.20% | 1.76% | 2.03% | 89% | 78% | 0.32% | 3.40% | 5.67% | 84% | 77% |
| Class B | 0.01% | 1.14% | 0.89% | 91% | 77% | 0.20% | 2.45% | -0.67% | 85% | 77% |
| Class C | 0.09% | 0.60% | 0.71% | 85% | 67% | 0.26% | 1.25% | 0.40% | 81% | 73% |
| Class E | 0.19% | 1.37% | 0.97% | 78% | 68% |  |  |  |  |  |
| Overall | 0.11% | 1.15% | 1.00% | 86% | 73% | 0.24% | 2.51% | 1.02% | 83% | 76% |
| Class D | 0.03% | 1.09% | 1.13% | 88% | 67% | 0.16% | 1.08% | -0.40% | 87% | 76% |

Table 4. Summary of the inference test results, sub-test EE1-1.1.2.b vs NNVC anchor, nnIntra ON.

|  | Proposed a vs NNVC6.0 HOP anchor |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  |  |  | Random Access |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.16% | 0.74% | -0.20% | 84% | 69% | -0.11% | 1.11% | -0.72% | 80% | 70% |
| Class A2 | 0.16% | 0.53% | 1.63% | 88% | 71% | 0.22% | 1.83% | 4.73% | 85% | 72% |
| Class B | 0.04% | -0.31% | 0.05% | 88% | 70% | 0.11% | 0.68% | -1.24% | 85% | 73% |
| Class C | 0.11% | -0.39% | 0.25% | 82% | 60% | 0.22% | 0.75% | 0.00% | 78% | 67% |
| Class E | 0.11% | 0.86% | 0.78% | 73% | 61% |  |  |  |  |  |
| Overall | 0.11% | 0.18% | 0.44% | 83% | 66% | 0.12% | 1.02% | 0.39% | 82% | 71% |
| Class D | 0.06% | 0.66% | 1.16% | 85% | 61% | 0.11% | -0.67% | -1.18% | 81% | 68% |

# Conclusion

In sub-test 1.2.1, reduced ranking of the two-components decomposition in residual block of the backbone were tested. In sub-test EE1-1.1.2.a the change of the parameter C31 to 48 is tested. The complexity of the model is reduced by 11% to 426kMAC/pixel (block-wise 128x128). In sub-test EE1-1.1.2.b, order of the decomposition (horizontal-vertical or vertical-horizontal) is alternated every block in the backbone to mitigate the impact of the rank reduction on decomposition symmetry.

Sub-test EE1-1.1.2.b (int16) provides {0.1%, 0.2%, 0.4%} and {0.1%, 1.0%, 0.4%} bd-rate penalty with 11% complexity reduction in kMAC/pixel terms. Run time is reportedly reduced to 83% and 68% on average for encoder and decoder, respectively.

# References

- BoG report on NN-filter design unification, JVET-AD0380, E. Alshina, F. Galpin, April 2023

- AhG11: EE1-0 High Operation Point model, F. Galpin (InterDigital), S. Eadie, D. Rusanovskyy, Y. Li (Qualcomm), Y. Li, J. Li (ByteDance), L. Wang, R. Chang (Tencent), Z. Xie (Oppo), E. Alshina (Huawei), JVET-AE0191, 2023.

- AhG11: HOP full results, F. Galpin (InterDigital), D. Rusanovskyy, Y. Li (Qualcomm), Y. Li, J. Li (ByteDance), L. Wang, R. Chang (Tencent), Z. Xie (Oppo), E. Alshina (Huawei), JVET-AF0043, 2023.

- Common Test Conditions and evaluation procedures for neural network-based video coding technology, JVET-Z2016, E. Alshina, R.-L. Liao, S. Liu, A. Segall.

- EE1-1.2 Complexity-performance tradeoff of decomposition, D. Rusanovskyy, Y. Li, M. Karczewicz (Qualcomm), JVET-AE0164.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).