| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0105-v1 |
| --- | --- |

| Title: | AHG9: On color vision deficiency optimization type for encoder optimization info SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chul Keun Kim Hendry Jaehyun Lim Seung hwan Kim Dong-Gyu Gwak | Email: | chulkeun.kim@lge.com |
| Source: | LG electronics |  |  |

_____________________________

- Abstract

This contribution proposes an optimization type that describes optimization applied by encoder related to color modification for human viewing. This optimization may be intended for human viewing with color vision deficiency. When a video is optimized for individuals with color vision deficiency, it may be uncomfortable for those with normal color vision to watch, thus, such information provided in the SEI may be helpful. The proposed SEI message contains the following information:

- Indication whether the optimiztion for the picture(s) is for color vision deficiency.

Introduction

In the previous meeting, JVET-AE0064 proposed a new SEI message for signalling encoder optimization type that is applied to video bitstream and indication whether the optimization is targeting for human viewing and / or machine analysis , it has been adopted into TuC for VSEI extensions JVET-AE2032.

Table 1 The syntax proposed in JVET-AE0064

| encoder_optimization_info(payloadSize ) { | Descriptor |
| --- | --- |
| optimization_cancel_flag | u(1) |
| if( !optimization_cancel_flag ){ |  |
| optimization_persistence_flag | u(1) |
| optimization_for_human_viewing_flag | u(1) |
| optimization_for_machine_analysis_flag | u(1) |
| optimization_type | u(16) |
| } |  |
| } |  |

This contribution is a result of further study on optimization type for human viewing.

Proposal

It is a common practice that a video may be modified / optimized in certain way when it is intended to be viewed by users with color vision deficiency. In such case, optimization for human viewing may be for individuals with color vision deficiencies. The objectives of optimization could be as follows:

- Assistance in color perception for color vision deficiency.

  - The hue and saturation of pixels are transformed to enable individuals with color vision deficiencies to perceive differences in colors, allowing them to obtain information conveyed by colors.

- Efficient resource allocation for individuals with color vision deficiencies.

  - Sending information that cannot be perceived by the user may be unnecessary. For example, individuals with red-green color blindness may not obtain information from the V component(in the YUV color space), and optimization can be used to allocate fewer bits to the encoding of the video.

We propose the following:

- Add a new optimization type to indicate color vision deficiency. The new optimization type provides indication whether the optimiztion for the picture(s) is for color vision deficiency.

- The new optimization type shall be used only when the optimization_for_human_viewing_flag is equal to 1.

The application of optimization for color vision deficiencies is derived as follows:

( optimization_type & 0x20 )  !=  0

Table 2 – Definition of optimization_type

| Value | Interpretation |
| --- | --- |
| optimization_type = = 0 | May be used as determined by the application |
| optimization_type > 0 && ( optimization_type & 0x01 ) = = 0 | No object-based optimization |
| ( optimization_type & 0x01 ) != 0 | With object-based optimization; the pictures for which this SEI message persists have been pre-processed or encoded so that detected objects in the pictures are optimized with respect to other parts of the pictures for the indicated optimization purposes |
| optimization_type > 0 && ( optimization_type & 0x02 ) = = 0 | No temporal subsampling optimization |
| ( optimization_type & 0x02 ) != 0 | With temporal subsampling optimization |
| optimization_type > 0 && ( optimization_type & 0x04 ) = = 0 | No spatial subsampling optimization |
| ( optimization_type & 0x04 ) != 0 | With spatial subsampling optimization |
| optimization_type > 0 && ( optimization_type & 0x08 ) = = 0 | No temporal quality optimization |
| ( optimization_type & 0x08 ) != 0 | With temporal quality optimization |
| optimization_type > 0 && ( optimization_type & 0x10 ) = = 0 | No spatial quality optimization |
| ( optimization_type & 0x10 ) != 0 | With spatial quality optimization; the pictures for which this SEI message persists have been pre-processed or encoded to reduce unnecessary information or improve the quality of necessary information.(e.g to reduce the amount of noise and remove speckles at the picture-level) |
| optimization_type > 0 && ( optimization_type & 0x20 ) == 0 | No optimization for color vision deficiency |
| ( optimization_type & 0x20 ) != 0 | With color vision deficiency optimization; the pictures for which this SEI message persists have been pre-processed or encoded to reduce unnessary information for individuals with color blindness or to enhance their ability to distinguish colors.(e.g. Transforming indistinguishable colors into distinguishable ones or removing unperceivable color information) |

If the editorial change proposed by JVET-AFxxxx item 1 is agreed, the above table with the proposed new optimization type should be implemented as shown below:

| bitMask | Interpretation |
| --- | --- |
| 0x01 | Object-based optimization; the pictures for which this SEI message persists have been pre-processed or encoded so that detected objects in the pictures are optimized with respect to other parts of the pictures for the indicated optimization purposes |
| 0x02 | Temporal resampling optimization |
| 0x04 | Spatial resampling optimization |
| 0x08 | Temporal quality optimization |
| 0x10 | Spatial quality optimization; the pictures for which this SEI message persists have been pre-processed or encoded to reduce unnecessary information or improve the quality of necessary information.(e.g to reduce the amount of noise and remove speckles at the picture-level) |
| 0x20 | Color vision deficiency optimization; the pictures for which this SEI message persists have been pre-processed or encoded to reduce unnessary information for individuals with color blindness or to enhance their ability to distinguish colors.(e.g. Transforming indistinguishable colors into distinguishable ones or removing unperceivable color information) |

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).