| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0108-v1 |
| --- | --- |

| Title: | EE2-1.1: Non-square quadtree partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongjo Ahn Junghak Nam Naeri Park Jaehyun Lim 19 Yangjaedaero-11gil, Soecho-gu, Seoul 06772, South Korea Seunghwan Kim 10680 Treena St, San Diego, CA 92131, USA | Tel: Email: | +82-2-6912-6561 yongjo.ahn@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution reports results for EE2 Test 1.1 on non-square quadtree partitioning. In this test, quadtree partitioning for non-square blocks of which shapes are 2N×N and N×2N is proposed.  In the Test 1.1a, proposed non-square quadtree partition is tested. In the Test 1.1b, ECM-10.0 with maximum MTT depth plus 1 is tested as compared to Test 1.1a. The proposed partitioning method was implemented on top of ECM-10.0 and the summarized test results are shown as follows:

Test 1.1a:

AI: -0.11%/-0.20%/-0.25% (Y/U/V), 120.9% (EncT), 99.5% (DecT)

RA: -0.34%/-0.44%/-0.45% (Y/U/V), 121.9% (EncT), 100.7% (DecT)

LDB: -0.21%/ 0.08%/-0.24% (Y/U/V), 116.4% (EncT), 100.4% (DecT)

Test 1.1b:

AI: -0.43%/-0.67%/-0.74% (Y/U/V), 171.8% (EncT), 102.2% (DecT)

RA: -X.XX%/-X.XX%/-X.XX% (Y/U/V), XXX.X% (EncT), XXX.X% (DecT)

LDB: -0.77%/-1.05%/-1.37% (Y/U/V), 160.8% (EncT), 101.7% (DecT)

Introduction

In the current VVC and ECM, quadtree partitioning is applied to only square blocks (2N×2N) which are not split with binary or ternary tree while binary and ternary tree partitioning can be applied to both square and non-square blocks. In the JVET-AE meeting, JVET-AE0082 [1] was proposed to allow quadtree partitioning is also applied to non-square blocks of which shapes are 2N×N and N×2N. The proposed partitioning method was agreed to be included in the EE2 Test 1.1 for further study and evaluation.

Proposed methods

In the JVET-AE0082, quadtree partitioning for non-square blocks was proposed. Technical details of the proposed method are described in the following subsections:

Block shapes and sizes

The current VVC and ECM imposes restriction of allowing quadtree partitioning for any non-square blocks. The proposed partitioning method additionally allows quadtree partitioning for non-square blocks of which block shapes are 2N×N and N×2N.

Specifically, it is applied for block sizes which are greater than or equal to minimum block size (i.e. 16×8 or 8×16). The minimum block size depends on picture size and configuration. The proposed method doesn’t set the specific maximum block size for the non-square quadtree partitioning. The maximum block sizes can be decided according to the maximum block sizes for binary or ternary tree partitioning. Under the common test conditions, the largest block sizes for the non-square quadtree partitioning are 128×64 or 64×128 for Class A/B and 64×32 or 32×64 for other classes.

Figure 1. Block partitioning syntaxes of VVC and ECM with the proposed non-square quadtree partitions

Syntax signalling

In Figure 1, a basic idea and syntax structure of block partitioning for the proposed method are described. As shown in Figure 1, the proposed method has no additional syntax for non-square quadtree partitioning. The proposed partitioning method uses split_qt_flag, a conventional syntax for quadtree partitioning for square blocks, to signal whether the non-square quadtree partitioning is applied or not. There is no additional syntax for the proposed partitioning, but, this proposal adds a CABAC context for the cases when the non-square quadtree partition can be applied.

Constraint of the conventional quadtree partitioning for square blocks

In the current VVC and ECM, quadtree partitioning for square blocks cannot be applied to a CU if any of its ancestor CUs is split with binary or ternary tree partitions. The proposed partitioning method also keeps this constraint. Therefore, if any of its ancestor CUs is split with binary, ternary, or non-square quadtree, the CU cannot split with the quadtree for square blocks.

Fast encoding algorithms

In the JVET-AE meeting, the proposed method was requested to reduce encoding time increment in this round of EE. Hence, three fast encoding algorithms have been applied to the proposed partitioning method to reduce encoding time. The first fast encoding algorithm is related to constraints of partitioning depths considering that non-square quadtree (NQT) depth is included into multi-type-tree (MTT) depths. In addition, NQT partitioning is allowed only at once in MTT depths under the common test configurations. The second fast encoding algorithm is an early termination method based on coded block flag (CBF) of the previous coding block split with binary tree (BT) or ternary tree (TT) in rate-distortion optimization (RDO) process. The third fast encoding algorithm is an early termination method based on rd-cost comparison between NO_SPLIT and BT/TT blocks in RDO process.

Experimental results

The proposed partitioning method has been implemented in ECM-10.0 [2] and evaluated under all intra (AI), random access (RA), and low-delay B (LDB) common test configurations [3].

In this EE test, two simulations are tested on top of ECM-10.0 as below:

Test 1.1a: NQT with fast encoding algorithms

Test 1.1b: ECM-10.0 with maximum MTT depth plus 1

The results of Test 1.1a and Test 1.1b are tabulated in Table 1 and Table 2, respectively.

As shown in the below tables, the proposed partitioning method (Test 1.1a) provides 0.11%, 0.34%, and 0.21% BD-rate savings over ECM-10.0 under the AI, RA, and LDB configurations, respectively. Encoding time increments are 120.9%, 121.9%, and 116.4% and no meaningful decoding time is observed. Otherwise, Test 1.1b provides 0.43%, X.XX%, and 0.77% BD-rate savings over ECM-10.0 under the AI, RA, and LDB configurations, respectively. Encoding time increments are 171.8%, XXX.X%, and 160.8% and no meaningful decoding time is observed.

Table 1. Performance of Test 1.1a (NQT with fast encoding algorithms)

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.02% | -0.07% | -0.13% | 113.4% | 98.3% | 100.1% |
| Class A2 | -0.07% | -0.33% | -0.29% | 119.5% | 99.9% | 100.0% |
| Class B | -0.10% | -0.21% | -0.31% | 121.5% | 100.8% | 100.0% |
| Class C | -0.15% | -0.16% | -0.20% | 127.2% | 98.9% | 100.0% |
| Class E | -0.19% | -0.28% | -0.31% | 120.9% | 99.2% | 100.0% |
| Overall | -0.11% | -0.20% | -0.25% | 120.9% | 99.5% | 100.0% |
| Class D | -0.13% | -0.35% | -0.37% | 130.6% | 100.6% | 100.0% |
| Class F | -0.37% | -0.38% | -0.47% | 123.2% | 100.1% | 100.1% |
|  | Random Access Main 10 |  |  |  |  |  |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.36% | -0.47% | -0.29% | 120.2% | 99.9% | 100.1% |
| Class A2 | -0.34% | -0.65% | -0.35% | 120.5% | 100.6% | 99.9% |
| Class B | -0.34% | -0.40% | -0.50% | 121.8% | 100.9% | 100.2% |
| Class C | -0.34% | -0.32% | -0.60% | 124.3% | 101.1% | 100.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.34% | -0.44% | -0.45% | 121.9% | 100.7% | 100.1% |
| Class D | -0.34% | -0.25% | -0.49% | 123.6% | 100.8% | 100.0% |
| Class F | -0.39% | -0.40% | -0.55% | 118.1% | 99.5% | 99.8% |
|  | Low delay B Main 10 |  |  |  |  |  |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.19% | 0.48% | -0.18% | 115.8% | 100.2% | 100.1% |
| Class C | -0.29% | -0.63% | -0.68% | 119.7% | 101.2% | 100.0% |
| Class E | -0.16% | 0.36% | 0.28% | 112.9% | 99.9% | 100.0% |
| Overall | -0.21% | 0.08% | -0.24% | 116.4% | 100.4% | 100.1% |
| Class D | -0.14% | -0.44% | -0.38% | 118.5% | 100.0% | 100.0% |
| Class F | -0.33% | -0.38% | -0.13% | 115.2% | 99.8% | 100.0% |

Table 2. Performance of Test 1.1b (ECM-10.0 with maximum MTT depth plus 1)

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.09% | -0.03% | -0.25% | 143.9% | 101.0% | 100.1% |
| Class A2 | -0.23% | -0.70% | -0.73% | 162.9% | 102.6% | 100.2% |
| Class B | -0.43% | -0.72% | -0.87% | 176.1% | 102.0% | 100.0% |
| Class C | -0.62% | -0.73% | -0.83% | 197.7% | 102.2% | 100.0% |
| Class E | -0.70% | -1.09% | -0.95% | 172.5% | 103.5% | 100.0% |
| Overall | -0.43% | -0.67% | -0.74% | 171.8% | 102.2% | 100.1% |
| Class D | -0.52% | -0.62% | -0.81% | 203.1% | 102.9% | 100.0% |
| Class F | -1.30% | -1.70% | -1.66% | 180.7% | 101.2% | 100.1% |
|  | Random Access Main 10 |  |  |  |  |  |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -1.09% | -1.45% | -1.52% | 195.7% | 100.5% | 100.1% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class B | -1.01% | -1.71% | -1.71% | 186.8% | 101.0% | 100.4% |
| Class C | -0.80% | -1.12% | -1.38% | 166.5% | 101.8% | 100.0% |
| Class E |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class D | -0.73% | -1.45% | -1.19% | 160.2% | 102.3% | 100.0% |
| Class F | -1.55% | -1.65% | -1.61% | 163.3% | 102.0% | 99.9% |
|  | Low delay B Main 10 |  |  |  |  |  |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.69% | -1.06% | -1.23% | 165.3% | 100.9% | 100.4% |
| Class C | -0.83% | -1.41% | -1.47% | 167.4% | 102.5% | 100.0% |
| Class E | -0.81% | -0.56% | -1.49% | 145.7% | 102.0% | 100.0% |
| Overall | -0.77% | -1.05% | -1.37% | 160.8% | 101.7% | 100.2% |
| Class D | -0.69% | -1.83% | -1.38% | 152.9% | 102.1% | 100.0% |
| Class F | -1.93% | -2.82% | -2.07% | 155.3% | 102.6% | 100.0% |

Conclusion

This contribution reports the results for EE2 Test 1.1 on non-square quadtree partitioning. It is asserted that the results indicate reasonable trade-off between BD-rate gains and run time impacts. It is proposed to include the proposed partitioning from EE2 Test 1.1a into the next version of ECM.

Reference(s)

- Y. Ahn, J. Nam, N. Park, J. Lim, S. Kim, “AHG12: Non-square quadtree partitioning,” JVET-AE0082, 31st Meeting, Geneva, CH, 11-19 July 2023.

- ECM-10.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-10.0

- M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” JVET-AE2017, 31st Meeting, Geneva, CH, 11–19 July 2023.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).