| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0116-v3 |
| --- | --- |

| Title: | Non-EE2: slope adjustment for IBC LIC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Changyue Ma, Xiaoyu Xiu, Wei Chen, Hong-Jheng Jhu, Che-Wei Kuo, Ning Yan, Xianglin Wang | Email: | machangyue@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

- Abstract

This contribution proposes to improve IBC LIC with slope adjustment, where an offset parameter is utilized to adjust the slope parameter of IBC LIC. The proposed method is only applied to IBC AMVP CUs. The simulation results on top of ECM-10.0 are summarized as follows:

Screen Content:

AI:

Class F : {-0.21%, -0.14%, -0.26%, 100.9%, 99.6%, 100.0%}, Class TGM : {-0.12%, -0.13 %, -0.19%, 100.5%, 100.3%, 100.0%};

RA:

Class F : {-0.19%, 0.06%, -0.05%, 100.0%, 99.2%, 100.0%}, Class TGM : {-0.13%, -0.15%, -0.14%, 99.5%, 99.8%, 100.0%};

Natural Content :

AI : {xxx%, xxx%, xxx%, xxx%, xxx%}, RA : {xxx%, xxx %, xxx%, xxx%, xxx%};

Introduction

In ECM-10.0 [1], intra block copy with local illumination compensation (IBC-LIC) is a coding tool which compensates the local illumination variation within a picture between an IBC CU and its prediction block based on a linear model. The linear model parameters (i.e., scale and offset) are derived by following the same procedure of inter LIC except that the reference template is generated using the BV of the IBC block. IBC-LIC can be applied to IBC AMVP and IBC merge. For IBC AMVP, an IBC-LIC flag is signalled to indicate the use of IBC-LIC. For IBC merge, the IBC-LIC flag is inferred from the merge candidate. For IBC AMVP CUs, there are three template modes to derive the LIC parameters, i.e., IBC_LIC_TL (using both top and left templates), IBC_LIC_T (using only top template) and IBC_LIC_L (using only left template). Additionally, the multi-mode IBC-LIC is applied which divides the template samples into two groups based on their DC value and derives two LIC linear models separately. In the current design, the multi-mode IBC LIC is only applicable to IBC_LIC_TL CUs.

In ECM-10.0, slope adjustment is applied for the CCLM which utilizes an offset parameter to adjust the slope parameter of the linear model of CCLM, as shown in Fig. 1.

Figure 1. Left: model created in CCLM without slope adjustment. Right: model updated using slope adjustment.

Proposal

In this contribution, slope adjustment for IBC LIC is proposed, where an offset parameter is signaled to update the slope parameter of IBC LIC same to the slope adjustment for CCLM except being applied to luma component only. The proposed method is only applied in IBC AMVP mode. In addition, the proposed method is not applied to the CUs that apply multi-mode IBC-LIC.

Experimental results

The proposed method was implemented on top EMC-10.0 and tests were performed following ECM CTC [2]. The test results of the proposed method are summarized in the following table.

Table 1: Test result of slope adjustment for IBC LIC

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class F | -0.21% | -0.14% | -0.26% | 100.9% | 99.6% | 100.0% |
| Class TGM | -0.12% | -0.13% | -0.19% | 100.5% | 100.3% | 100.0% |
|  | Random Access Main 10 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class F | -0.19% | 0.06% | -0.05% | 100.0% | 99.2% | 100.0% |
| Class TGM | -0.13% | -0.15% | -0.14% | 99.5% | 99.8% | 100.0% |

Conclusion

The contribution proposes to utilize an offset parameter to adjust the slope parameter of IBC LIC. Simulation results show that the proposed method can provides noticeable coding benefits with minor enc/dec run-time impact. It is recommended to study the proposed method in the next round of EE.

References

[1] ECM-10.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-10.0.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluationJVET procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).