| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0118-v1 |
| --- | --- |

| Title: | Non-EE2: Regression-based GPM blending |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Bordes, Franck Galpin, Karam Naser, Fabrice Urban, Kevin Reuzé, Fabrice Le Léannec, Edouard Francois | Email: | philippe.bordes@interdigital.com, franck.galpin@interdigital.com, karam.naser@interdigital.com, fabrice.urban@interdigital.com, kevin.reuze@interdigital.com, fabrice.leleannec@interdigital.com, edouard.francois@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution presents an enhanced GPM mode where the splitting shape and the blending function are derived from the template. The proposed extended GPM mode was implemented on top of ECM-10.0, it reportedly provides {Y, U, V} BD-reduction with encoder and decoder runtimes as follows:

Method-A: RA: -0.07%, -0.11%, -0.14%, 100.8% (EncT), 99.9% (DecT), LDB: -0.11%, 0.21%, -0.25%, 101.0% (EncT), 99.9% (DecT).

Method-B: RA: -0.06%, -0.08%, -0.10%, 101.9% (EncT), 99.6% (DecT), LDB: -0.15%, -0.11%, -0.31%, 101.9% (EncT), 100.1% (DecT).

# Introduction

The Geometric partitioning mode (GPM) allows splitting a CU into two parts by a geometrically located straight line. Each part of a geometric partition in the CU is inter-predicted or intra predicted.

When GPM is selected, a geometric partition index indicating the partition mode of the geometric partition (angle and offset), and two merge indices (one for each partition) are further signaled.

Each inter-predicted geometric partition of a GPM CU can further decide whether to signal MVD or not. If MVD is signaled for a geometric partition, after a GPM merge candidate is selected, the motion of the partition is further refined by the signaled MVDs information.

The final prediction samples are generated by blending the two prediction signals using weighted average. Two integer blending matrices (W0 and W1) are used. The weights in the GPM blending matrices are derived from the ramp function based on the displacement from a predicted sample position to the GPM partitioning boundary. The blending area size can extend up to 8 samples on each side of the GPM partition split boundary. A CU level flag is coded to signal the selected blending area size.

Motion information for each geometric partition may be refined using template matching (TM). A CU-level flag is signaled to indicate whether TM is applied to both geometric partitions.

GPM with inter and intra prediction can be generated by weighting inter predicted samples and intra predicted samples for each GPM-separated region. The inter predicted samples are derived by inter GPM whereas the intra predicted samples are derived by an intra prediction mode (IPM) candidate list and a signaled index .

At last, template matching based reordering for GPM split modes is applied. Given the motion information of the current GPM block, the respective TM cost values of GPM split modes are computed. Then, GPM split modes are reordered in ascending ordering of TM cost values.

# Proposal

It is proposed an additional GPM implicit mode, where the two integer blending matrices (W0 and W1) are derived from the template (1 line above, 1 column left). The blending matrices are modelled as an affine linear function of the sample positions (x,y) in the current CU:

W0(x,y) = a.x + b.y + c  		and 	W1(x,y) = 1 - W0(x,y) 		(eq.1)

The parameters (a,b,c) are derived from the reference template using the same solver (MSE minimization) as the one used for CCCM, GLM or GL-CCCM. A list of pair of candidates is built from the regular GPM candidates and re-ordered with the template cost.

The GPM implicit mode is signaled by a CU-level flag (gpm_implicit_flag). If gpm_implicit_flag is true, a merge-idx is coded to signal the pair of GPM candidates to be used. If gpm_implicit_flag is false, the regular GPM syntax elements are signaled.

# Experimental results

The proposal has been implemented in ECM-10.0 and tested with CTC-RA/LDB conditions. One tested 2 methods to build the list of pair of GPM candidates:

- Method-A: the list of pair of GPM candidates contains at most the same number of candidates as the regular GPM list (numValidMergeCand) (Table 1 and Table 2).

- Method-B: the list of pair of GPM candidates may contain more candidates than the regular GPM list. Only the first numValidMergeCand reordered pairs of GPM candidates can be used (Table 3 and Table 4).

For screen-content with sharp edges, it is proposed the GPM-implicit mode may be disabled when sgpm_no_blend_flag is true.

Method-A:

Table 1: Performance results of proposal in RA (method-A).

|  | Random Access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0,03% | -0,11% | -0,14% | 100,1% | 99,5% | 100,0% |
| Class A2 | -0,06% | -0,08% | -0,09% | 100,2% | 100,4% | 99,6% |
| Class B | -0,09% | -0,13% | -0,23% | 101,4% | 100,4% | 100,1% |
| Class C | -0,07% | -0,12% | -0,07% | 101,2% | 99,1% | 100,1% |
| Class E |  |  |  |  |  |  |
| Overall | -0,07% | -0,11% | -0,14% | 100,8% | 99,9% | 100,0% |
| Class D | -0,05% | 0,02% | -0,15% | 100,9% | 99,6% | 100,1% |
| Class F | -0,08% | -0,20% | -0,19% | 100,9% | 100,6% | 99,9% |
| Class TGM | -0,01% | 0,00% | -0,01% | 101,0% | 100,1% | 100,0% |

Table 2: Performance results of proposal in LDB (method-A).

|  | Low Delay B Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |  |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0,11% | 0,45% | -0,48% | 101,0% | 99,8% | 99,9% |
| Class C | -0,18% | -0,17% | -0,47% | 100,9% | 99,8% | 100,1% |
| Class E | 0,00% | 0,34% | 0,44% | 101,1% | 100,1% | 100,1% |
| Overall | -0,11% | 0,21% | -0,25% | 101,0% | 99,9% | 100,0% |
| Class D | 0,03% | -0,23% | -0,26% | 100,7% | 100,3% | 100,1% |
| Class F | 0,03% | -0,11% | 0,78% | 100,8% | 100,1% | 100,0% |
| Class TGM | 0,11% | 0,01% | 0,11% | 100,8% | 99,6% | 100,6% |

Method-B:

Table 3: Performance results of proposal in RA (method-B).

|  | Random Access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0,04% | -0,14% | -0,13% | 101,2% | 99,5% | 100,4% |
| Class A2 | -0,01% | -0,07% | -0,15% | 101,9% | 100,0% | 99,9% |
| Class B | -0,11% | -0,03% | -0,09% | 102,2% | 99,1% | 100,1% |
| Class C | -0,07% | -0,10% | -0,04% | 102,0% | 100,1% | 100,1% |
| Class E |  |  |  |  |  |  |
| Overall | -0,06% | -0,08% | -0,10% | 101,9% | 99,6% | 100,1% |
| Class D | -0,07% | 0,11% | -0,08% | 101,7% | 99,7% | 100,1% |
| Class F | -0,10% | -0,30% | -0,28% | 101,8% | 100,0% | 100,0% |
| Class TGM | 0,03% | 0,02% | -0,01% | 102,1% | 99,8% | 100,0% |

Table 4: Performance results of proposal in LDB (method-B).

|  | Low Delay B Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT |  |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0,12% | 0,04% | -0,58% | 102,0% | 99,5% | 100,1% |
| Class C | -0,14% | -0,21% | -0,26% | 101,5% | 100,7% | 100,1% |
| Class E | -0,21% | -0,24% | 0,06% | 102,5% | 100,1% | 100,1% |
| Overall | -0,15% | -0,11% | -0,31% | 101,9% | 100,1% | 100,1% |
| Class D | 0,06% | 0,03% | 0,01% | 101,7% | 99,9% | 100,1% |
| Class F | 0,19% | 0,29% | 0,54% | 101,4% | 99,8% | 100,0% |
| Class TGM | 0,07% | 0,11% | 0,15% | 101,8% | 99,9% | 100,9% |

# Experimental results

This contribution proposes an enhanced GPM mode to improve the regular GPM. The results show that the proposed method can increase the compression performance of the GPM mode of ECM-10.0.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).