| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0132-v0 |
| --- | --- |

| Title: | Non-EE2: Optimization of TIMD blending mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chuan Zhou Zhuoyi Lv (vivo) | Email: | chuan.zhou@vivo.com zhuoyi.lv@vivo.com |
| Source: | vivo Mobile Communication Co., Ltd |  |  |

_____________________________

- Abstract

This contribution proposes an optimization method for reference lines selection in TIMD blending mode. For the second mode, the determination of which reference line to be used is not only based on the second mode, but also on the first mode.

On top of the ECM10.0, simulation results are reported as below:

AI: -0.03% (Y), -0.02% (U), -0.10% (V),  99.8% (EncT),  100.3% (DecT)

Introduction

For TIMD mode with blending in ECM10.0, the first mode uses reference line l. While for the second mode, whether to use reference line l or l+1 depends on the following conditions:

- If all of the following conditions are true, l+1 is chosen.

- the current block is not ISP block

- the second mode is angular prediction mode

- the second mode does not represent non-fractional angles

- Otherwise, l is chosen.

Proposed methods

For the second mode, the modified conditions of reference line determination are as follows:

- If all of the following conditions are true, l+1 is chosen.

- the current block is not ISP block

- both the first mode and second mode are angular prediction mode

- all of the following conditions are false

- abs(predModeIntra1 – predModeIntra2) is greater than Threshold. The value of Threshold is set to 8 or 4.

- (predModeIntra1 - EXT_HOR_IDX) * (predModeIntra2 - EXT_HOR_IDX) is less than 0.

- (predModeIntra1 - EXT_VER_IDX) * (predModeIntra2 - EXT_VER_IDX) is less than 0.

- Otherwise, l is chosen.

Simulation results

The proposed method is tested on top of ECM10.0 under the common test conditions [1].

Table 1. Simulation results of the proposed method

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.02% | 0.03% | -0.07% | 100.0% | 100.3% | 100.0% |
| Class A2 | -0.01% | -0.08% | -0.06% | 99.8% | 100.1% | 100.0% |
| Class B | -0.03% | 0.01% | -0.19% | 100.0% | 100.4% | 100.0% |
| Class C | -0.05% | -0.01% | 0.05% | 99.6% | 100.6% | 100.0% |
| Class E | -0.04% | -0.08% | -0.22% | 99.7% | 99.5% | 100.0% |
| Overall | -0.03% | -0.02% | -0.10% | 99.8% | 100.3% | 100.0% |
| Class D | -0.01% | 0.00% | -0.17% | 100.0% | 100.9% | 100.0% |
| Class F | -0.03% | -0.11% | -0.05% | 100.0% | 100.8% | 100.0% |

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |

References

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AE2017, Jul. 2023.

Patent rights declaration(s)

vivo Mobile Communication Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).