| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0134-v1 |
| --- | --- |

| Title: | AHG12: AMVP with SbTMVP mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao Jie Chen Yan Ye Xinwei Li | Tel: Email: | {ruling.lrl, jiechen.cj, yan.ye, sid.lxw}@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

# Abstract

The AMVP with SbTMVP mode was firstly proposed in JVET-AC0103, and was studied in EE2. The simulation result on top of ECM-8.0 was reported in JVET-AD0152, and the overall coding performance and runtime for {Y, U, V, EncT, DecT} was {-0.06%, -0.03%, -0.04%, 102%, 100%} for RA and {-0.22%, 0.06%, 0.04%, 102%, 100%} for LB. It was commented that the results did not provide attractive benefit in terms of compression, compared to the increased encoder.

In this contribution, the coding efficiency and complexity of the AMVP with SbTMVP mode are improved. Given a CU coded in AMVP with SbTMVP mode, the CU is predicted in a similar way as that of SbTMVP in merge mode except that the motion shift is signaled in the bitstream instead of being derived from neighboring blocks. It is reported that on top of ECM-10.0, the overall coding performance impact for {Y, U, V, EncT, DecT, VmPeak} is {-0.10%, -0.01%, -0.02%, 101.1%, 99.8%, 100.2%} for RA and {-0.36%, -0.21%, -0.29%, 103.2%, 99.7%, 100.1%} for LB.

# Introduction

The AMVP with SbTMVP mode was proposed in JVET-AC0103 [1] and JVET-AD0152 [2]. In JVET-AD0152, when a CU is coded using the AMVP with SbTMVP mode, the CU is split into 4x4 subblocks, and each subblock derives its own motion from a corresponding subblock in the collocated picture. The collocated picture is fixed to the same collocated picture used in VVC. The reference pictures of subblocks are selected using the same way as that of SbTMVP in VVC. The corresponding subblock is obtained using MVP with a signaled MVD. The number of MVD is set to 16 if a picture whose POC distance between current picture and collocated picture is larger than 2. Otherwise, if the picture is a low delay picture, the number of MVD is set to 32. Otherwise, the number of MVD is set to 64.

In some cases, the corresponding subblock in collocated picture is non-inter coded, that is, the temporal motion information is unavailable; in this case, the motion of center subblock of the CU is used instead. When the motion of the center subblock of the CU is also unavailable, the MVP with the signaled MVD is used instead.

Regarding the syntax design, when a CU is coded using AMVP mode, a flag is signaled to indicate whether the CU is predicted with SbTMVP. If the SbTMVP is used, another flag is signaled to indicate whether MVD exists, and an MVD index is signaled to indicate the motion direction and motion magnitude if MVD exists. The inter prediction direction and reference picture index are directly set to the collocated picture instead of being signaled in the bitstream for the proposed mode. Moreover, the adaptive motion vector resolution (AMVR), local illumination compensation (LIC) and multi-hypothesis prediction (MHP) are not applied to the proposed mode. The OBMC is always applied to the proposed mode without signaling.

# Proposed methods

To improve the coding efficiency and reduce the encoding complexity of the AMVP with SbTMVP mode, the following changes are made:

- For low delay pictures, one CU-level flag is signaled to indicated which one of the two collocated pictures is used to derive the motion. For non-low delay pictures, the collocated picture is fixed to the same collocated picture used in VVC.

- The reference pictures of subblocks are fixed to the one with reference picture index equal to 0.

- The number of MVD is always set to 16 which comprises 4 motion directions and 4 motion magnitudes (Figure 1).

- When picture resolution is larger than or equal to 1280x720 luma samples, the AMVR is enabled for the proposed mode. Two MVD resolutions are supported including 4-pel and 64-pel. When 4-pel MVD resolution is selected, the motion magnitudes are {4, 8, 12, 16}-pel. On the other hand, when 64-pel MVD resolution is selected, the motion magnitudes are {64, 128, 192, 256}-pel.

Figure 1. 16 MVD candidates for all pictures

# Simulation results

The proposed methods are implemented on top of ECM-10.0 and are tested under CTC for enhanced compression tool testing [3] with random access and low delay B configurations. Table 1 shows the simulation results of the proposed method.

Table 1. Simulation results of AMVP with SbTMVP mode

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.04% | 0.07% | -0.01% | 101.2% | 100.1% | 100.5% |
| Class A2 | -0.08% | 0.03% | -0.11% | 101.5% | 99.7% | 100.2% |
| Class B | -0.08% | 0.06% | -0.02% | 101.0% | 100.1% | 100.2% |
| Class C | -0.17% | -0.18% | 0.03% | 100.7% | 99.4% | 100.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.10% | -0.01% | -0.02% | 101.1% | 99.8% | 100.2% |
| Class D | -0.17% | 0.03% | -0.22% | 100.6% | 99.4% | 100.2% |
| Class F | -0.08% | -0.09% | -0.07% | 100.8% | 99.6% | 100.1% |

|  | Low delay B Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.13% | 0.22% | -0.29% | 103.7% | 100.2% | 100.0% |
| Class C | -0.21% | -0.31% | 0.08% | 102.3% | 99.1% | 100.2% |
| Class E | -0.93% | -0.79% | -0.79% | 103.6% | 99.6% | 100.1% |
| Overall | -0.36% | -0.21% | -0.29% | 103.2% | 99.7% | 100.1% |
| Class D | -0.42% | -0.60% | -0.52% | 102.7% | 98.1% | 100.2% |
| Class F | -0.13% | 0.29% | 0.31% | 101.9% | 98.5% | 100.1% |

# Conclusion

The contribution proposes several changes to improve coding efficiency and reduce complexity of the AMVP with SbTMVP mode. The proposed methods achieve 0.10% and 0.36% BD-rate reduction with 1% and 3% encoding runtime increase for RA and LB, respectively. It is recommended to study this proposal in next round of EE2.

# References

- R.-L. Liao, J. Chen, X. Li and Y. Ye, “Non-EE2: AMVP mode with subblock-based temporal motion vector prediction,” JVET-AC0103, Jan. 2023.

- R.-L. Liao, J. Chen, X. Li and Y. Ye, “EE2-2.2: AMVP with SbTMVP mode,” JVET-AD0152, Apr. 2023.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AE2017, Jul. 2023.

# Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).