| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13-20 October 2023 | Document: JVET-AF0145-v1 |
| --- | --- |

| Title: | AHG9: SEI message for text for generative AI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Arianne Hinds Stephan Wenger | Tel: Email: | ahinds@global.tencent.com swenger@global.tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

Text prompts serve as an important element to generative AI image and video generator applications. The sequence of images or video frames to be used as the source (inputs) visual basis for the resulting image or video generated outputs, from a generative AI process, can be carried in a coded video stream. Likewise, for the purposes of provenance, it may be important that such applications be able to record that generative AI operations were performed to create the output. This contribution proposes that a new SEI be created to store the text data for two use cases related to generative AI.

# Problem statement

## Use case 1: Text prompts

A number of AI image generators have already been introduced to the market [1,2,3]. Each of such generators utilizes a user-generated text prompt to guide the AI generation process. The video or image payload of a coded video stream may be used as the visual basis upon which the AI generation process will create the resulting output. A new SEI message to carry a UTF-8 encoded text prompt stream is needed to associate the text prompt to the imagery in the associated coded video stream for the purposes of guiding the generative AI process.

Figure 1 is an illustration of the generative AI image or video architecture.

Figure 1. Approximation of image/video AI generation process

In Figure 1, image or video of a person riding a bicycle in a city, and a text prompt to guide the AI generator process, are provided as input to an AI image or generator process. The image or video that are output from the AI image or video generator is the imagery generated from the AI process. In this example, image or video of the person riding a bicycle may be carried in a coded video stream, along with an SEI message to signal the desired text prompt.

Figure 2 is an expanded version of Figure 1, illustrating the encoding and decoding processes in the context of the generative AI use case.

Figure 2. Approximation of image/video AI generation process with encoder and decoder

In figure 2 a user creates a text prompt (panda riding bicycle in city) to guide a generative AI process to modify a video stream, shown as the person riding a bicycle. The text prompt and bicycle imagery are encoded into a video stream, that is carried over a network to a decoding process. The decoder reconstructs the imagery and makes the text prompt data (panda riding bicycle in city) available to a generative AI process. The generative AI process creates the output comprised of imagery of a panda riding a bicycle in a city. The output from the generative AI process may be encoded by yet another video coding process.

## Use case 2: Provenance with AI operations performed

Lately, the provenance of imagery that has been created or manipulated by a generative AI process has been a topic of concern, e.g., as one concern of the actors in the Screen Actors’ Guild that generative AI processes may exploit their likenesses in ways that are potentially harmful to the actors; photos entered into photography contests that were created with a generative AI process.

Whereas a number of metadata can be stored directly into still imagery, for video there is no single metadata source to document the use of AI operations that were used to create the video.

Figure 3 illustrates a use case for a new SEI that stores messages, e.g., “Created with AI generation process” for the purpose of maintaining a record of provenance for the resulting imagery.

Figure 3. AI generation process with message inserted for provenance.

In figure 3, a user uploads a video sequence and corresponding text prompt to an AI image or video generator in a cloud. Note that in this figure, the method used to upload the data into the cloud is not illustrated. The AI process creates an output video sequence along with a string of text indicating that the sequence was created with the help of an AI process. The generated video sequence and corresponding message are then input into an encoder. Output from the encoder includes the coded video stream along with an SEI message to indicate that an AI process was involved in the creation of the output video stream.

# Proposed approach

An AI text container SEI message is proposed to address the above use cases. A byte to signal the purpose of the text message indicates which of the above use cases the text addresses, i.e, to carry the text to guide an AI image or video generator process, or to signal that an AI process was used in the creation of the coded video.

Approaches for carriage of AI text data within an SEI include:

- The text payload is carried entirely as a “container” within an SEI message that is made active according to the semantics of a “persistence” flag.

- The text payload is carried entirely as a “container” within an SEI message that is made active or cancelled for the duration of the bitstream until another such SEI message is encountered, i.e., via the semantics of a cancel flag.

# Proposal

## AI text data SEI message syntax

| ai_text_data( payload_size ) | Descriptor |
| --- | --- |
| ait_data_purpose | u(8) |
| ait_data_cancel_flag | u(1) |
| if (!ait_data_cancel_flag) { |  |
| ait_data_persistence_flag | u(1) |
| ait_data_string | st(v) |
| } |  |
| } |  |

## AI text data SEI message semantics

The AI text data SEI message indicates text prompts encoded as specified by [4].

ait_data_purpose specifies the purpose of the AI text as follows:

- If ait_data_purpose is equal to 0, the remaining SEI payload bytes shall be a string containing text prompts to guide an image or video application, e.g., an artificial intelligence image or video generator, to modify one or more coded pictures in the bitstream to produce imagery that attempts to modify the pictured according to the guidance in the text prompts.

- Otherwise if ait_data_purpose is equal to 1, the remaining SEI payload bytes shall be a string containing text that indicates that an application (e.g., generative artificial intelligence application) was used to create the pictures for which this SEI message is associated.

All other values of ait_data_purpose are reserved.

ait_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous AI text data SEI message in output order. ait_cancel_flag equal to 0 indicates that AI text data information follows.

ait_data_persistence_flag specifies the persistence of the AI text data SEI message for the current layer.

ait_data_persistence_flag equal to 0 specifies that the AI text data SEI message applies to the current decoded picture only.

ait_data_persistence_flag equal to 1 specifies that the AI text data SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with an AI text data SEI message is output that follows the current picture in output order.

ait_data_string shall be a string containing text according to ait_data_purpose.

# References

[1] https://openai.com/dall-e-2

[2] https://www.adobe.com/sensei/generative-ai/firefly.html

[3] https://www.canva.com/ai-image-generator/

[4] ISO/IEC 10646: Information technology — Universal Coded Character Set (UCS)

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).