| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0148-v1 |
| --- | --- |

| Title: | [AHG9] Large SEI message signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gilles Teniou, Stephan Wenger | Email: | (teniou,swenger)[at]global.tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In nowadays video coding standards SEI messages serve multiple purposes e.g., improving the rendering quality or helping the receiver to apply the correct post-processing techniques.... Those SEI messages, particularly the most recent ones, defined in VSEI have a significant size impacting the signalling of the associated header. The present contribution proposes a complementary approach suitable for large SEI messages.

# Problem statement

In the HEVC [1] and VVC [2] specifications the SEI messages are signalled as follow:

### Supplemental enhancement information message syntax

| sei_message( ) { | Descriptor |
| --- | --- |
| payloadType = 0 |  |
| do { |  |
| payload_type_byte | u(8) |
| payloadType += payload_type_byte |  |
| } while( payload_type_byte = = 0xFF ) |  |
| payloadSize = 0 |  |
| do { |  |
| payload_size_byte | u(8) |
| payloadSize += payload_size_byte |  |
| } while( payload_size_byte = = 0xFF ) |  |
| sei_payload( payloadType, payloadSize ) |  |
| } |  |

This means that the header uses several bytes for signalling the payload type (the identifier of the SEI message) and the payload size (the size of the SEI message).

For SEI messages which payloadType is between 0 and 255, the size of the payload type is represented by 1 byte. An additional byte is required if the payloadType is 256 and 511 (which is not the case yet in the current specifications defining SEI messages)

For the PayloadSize of the SEI messages, it uses one byte every 256 bytes of data. This may end up with a significant number of bytes to signal large SEI messages as illustrated in the table below:

For example, a large SEI message containing 64kB of data in its payload would require 256 Bytes for signalling its size.

Those new SEI messages requiring large payload would benefit from an optimized representation of the corresponding header.

# Proposed solution

It is proposed to define a large SEI message header representing:

- the payload size in a fixed length 16-bits syntax element allowing to signal any payload up to 64kB.

- the signalling of the large SEI position that can serve the same purpose of the currently distinguished prefix and suffix SEI NAL unit types, meaning that only one NAL unit type would be needed. (1 bit would be enough but 2 bits would allow the definition of future cases)

- The SEI message relevance that would indicate how important the SEI message is for a given application. The use of 2 bits would permit to signal 4 levels of relevance (0 being the least relevant and 3 the most)

- For byte alignment, 4 reserved bits could be used in the future. For example, those bits could be used for signalling payload sizes greater than 64kB.

# Proposal

## VVC

### 3.1.1a Supplemental enhancement information message syntax

| lsei_message( ) { | Descriptor |
| --- | --- |
| lsei_position | u(2) |
| lsei_relevance | u(2) |
| lsei_reserved | u(4) |
| lsei_payload_type_byte | u(8) |
| lsei_payload_size_16bits | u(16) |
| lsei_payload( lseiPayloadType, lseiPayloadSize ) |  |
| } |  |

### 7.4.7a Large Supplemental enhancement information message semantics

Each Large SEI message consists of the variables specifying the type payloadType and size payloadSize of the large SEI message payload. Large SEI message payloads are specified in Annex D. The derived Large SEI message payload size payloadSize is specified in bytes and shall be equal to the number of RBSP bytes in the Large SEI message payload.

NOTE – The NAL unit byte sequence containing the Large SEI message might include one or more emulation prevention bytes (represented by emulation_prevention_three_byte syntax elements). Since the payload size of a Large SEI message is specified in RBSP bytes, the quantity of emulation prevention bytes is not included in the size payloadSize of a Large SEI payload.

lsei_position indicates if the SEI message corresponds to the PREFIX_SEI_NUT and SUFFIX_SEI_NUT.  lsei_position equal 0 indicates that the SEI message is treated as PREFIX_SEI_NUT. lsei_position equal 1 indicates that the SEI message is treated as SUFFIX_SEI_NUT. Values 3 and 4 of lsei_position are reserved for future use and shall be ignored.

lsei_relevance indicates the relevance of the SEI message for the target application. lsei_relevance ranges from 0 to 3, 0 being the least relevant and 3 being the most relevant.

NOTE – The relevance of an SEI message is an arbitrary decision and its use is to be specified by the target application.

lsei_reserved is revered for future use and shall be ignored.

lsei_payload_type_byte is a byte of the payload type of a large SEI message. payloadType = lsei_payload_type_byte.

payload_size_16bits is the payload size in bits of a large SEI message. payloadSize = payload_size_16bits.

# References

[1]	Recommendation ITU-T H.265 (V8) High efficiency video coding, Aug. 2021.

[2]	Recommendation ITU-T H.266 | ISO/IEC 23090-3, Versatile video coding.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).