| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0154-v1 |
| --- | --- |

| Title: | EE1-1.2.3: Input and output rotation of model for NNVC in-loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Renjie Chang Liqiang Wang Xiaozhong Xu Shan Liu | Email: | renjiechang@tencent.com liqiangwang@tencent.com xiaozhongxu@tencent.com shanl@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In this contribution, the test results of JVET-AE-EE1-1.2.3 are reported. In JVET-AE-EE1-1.2.3, the rotation operations are applied to the inputs and outputs of NNVC in-loop filter network to further improve the coding performance. In the rotation process, all the input samples including image information and side information are rotated and then fed into network to perform the inference. After that, the output of network is rotated back to restore the original order of filtered samples. Based on NNVC-6.0, the test results are shown in order of RA and LDB configurations as follows.

Compared with NNVC-6.0 HOP anchor:

Test 1 (Independent rotation):

RA:  -x.xx%	-x.xx%	  -x.xx%		EncT: xxx%	DecT: xxx%

LDB: -x.xx%	-x.xx%	  -x.xx%		EncT: xxx%	DecT: xxx%

Test 2 (Rotation combined with base QP adjustment)

RA: -0.01%	-0.22%	  -0.75%		EncT: 99%	DecT: 99%

LDB: -x.xx%	-x.xx%	  -x.xx%		EncT: xxx%	DecT: xxx%

Introduction

In NNVC-6.0, two neural-network based in-loop filters including unified HOP filter [1] and unified LOP filter are developed as additional in-loop filters to improve the coding performance. The network architectures of unified HOP filter and unified LOP filter are shown in Fig. 1 and Fig. 2, respectively. The inputs of both HOP and LOP network contain reconstruction, prediction, boundary strength, sliceQP, baseQP and IPB. The outputs of both HOP and LOP network contain luma and chroma reconstruction images which are denoted by Rec Y and Rec UV respectively.

Fig. 1. Network architecture of HOP filter

Fig. 2. Network architecture of LOP filter

Proposed method

In this contribution, two kinds of input-output rotation methods are proposed to improve the coding performance. In Section 2.1, a rotation method is proposed to be independent of both base QP adjustment and adaptive/fixed scale factor blending methods which are already intergrated into NNVC software. The rotation combined with base QP adjustment method is introduced in Section 2.2.

Independent rotation

In this proposed method, all the input samples can be rotated by 90 degrees to the left/right and then fed into the network. If rotation is performed on the inputs, the output of network would be rotated back to restore the original order of the filtered samples. The operations of rotating left/right are shown in Fig. 2.

Specifically, a slice-level flag is signaled to indicate whether the rotation type is no rotation type, rotation to the left type or rotation to the right type. All CTUs in a slice would be applied the same rotation operation before and after network inference.

Besides, to reduce the encoding complexity, encoding fast algorithms similar to JVET-AE-EE1-1.2.6 are used in the RDO process of selecting the best rotation type. That is, only distortion of partial CTUs in a slice are calculated to evaluate the rotation types for the best one at encoder side.

Fig. 2: Rotation to the left/right

Rotation combined with base QP adjustment

To promote the consistency between NNLF training and testing, the base QP adjustment method [1] is proposed and integrated into NNVC reference software, which can adaptively adjust base QP value of network input. Considering that the rotation of network input and output is essentially to seek consistency between training and testing, the rotation to the left is combined with the base QP adjustment to further obtain coding gains.

Specifically, in the RDO process of base QP adjustment, the rotation to the left will be applied to the network input by default if based QP value is adjusted and the network output would be also rotated back. Therefore, there are no flags needed to be signaled and no fast algorithms are added additionally here.

Inference stage

The inference information is identical to NNVC-6.0 HOP filter.

| - Network Information in Inference Stage |  |  |  |
| --- | --- | --- | --- |
| Mandatory | HW environment: |  |  |
|  | GPU Type | CPU only |  |
|  | Framework: | SADL |  |
|  | Number of GPUs per Task | 0 |  |
|  | Number of Parameters (Each Model) | 1.45 M/model |  |
|  | Total Number of Parameters (All Models) | 1.45 M/model |  |
|  | Parameter Precision (Bits) | 16 |  |
|  | Memory Parameter (MB) | 2.8 MB/model |  |
|  | Multiply Accumulate (kMAC/pixel) | 477 |  |
|  | Calculation Method | block basis |  |
| Optional |  |  |  |
|  | Total Conv. Layers |  |  |
|  | Total FC Layers | 0 |  |
|  | Total Memory (MB) |  |  |
|  | Batch size: | 1 |  |
|  | Patch size | 128x128 |  |
|  | Changes to network configuration or weights required to generate rate points |  |  |
|  | Peak Memory Usage (Total) |  |  |
|  | Peak Memory Usage (per Model) |  |  |
|  | Border handling |  |  |
|  | Other information: |  |  |

Results

All the experiments are performed based on the common test conditions in [3]. The proposed method is implemented on top of NNVC-6.0 unified HOP and LOP filter. The results are shown as follows.

Compared with NNVC-6.0 HOP anchor (NNIntra + HOP)

Table 1 RA Performance of EE1-1.2.3-test 1

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0_NNIntra+HOP |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

Table 2 LDB Performance of EE1-1.2.3-test 1

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0_NNIntra+HOP |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

Table 3 RA Performance of EE1-1.2.3-test 2

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0_NNIntra+HOP |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.01% | -0.02% | -0.90% | 100% | 99% |
| Class A2 | 0.02% | 0.14% | -0.12% | 100% | 99% |
| Class B | 0.00% | -0.02% | -0.65% | 99% | 99% |
| Class C | -0.05% | -0.89% | -1.23% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.22% | -0.75% | 99% | 99% |
| Class D | -0.13% | -1.19% | -1.20% | 100% | 100% |
| Class F | -0.09% | -0.54% | -0.87% | 99% | 103% |

Table 4 LDB Performance of EE1-1.2.3-test 2

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0_NNIntra+HOP |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | 100% | 99% |
| Class C | -0.35% | -2.57% | -2.34% | 99% | 99% |
| Class E | -0.27% | -0.27% | 1.13% | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | 99% | 99% |
| Class D | -0.43% | -2.93% | -3.73% | 98% | 102% |
| Class F | -0.29% | -1.25% | -1.17% | 100% | 99% |

Compared with NNVC-6.0 (only HOP)

Table 9 RA Performance of EE1-1.2.3-test 1

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 (only HOP) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

Table 10 LDB Performance of EE1-1.2.3-test 1

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 (only HOP) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

Table 11 RA Performance of EE1-1.2.3-test 2

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 (only HOP) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.06% | -0.21% | -0.88% | 99% | 98% |
| Class A2 | 0.04% | 0.18% | 0.01% | 100% | 98% |
| Class B | 0.02% | -0.05% | -0.76% | 99% | 98% |
| Class C | -0.03% | -0.83% | -0.97% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.25% | -0.69% | 99% | 98% |
| Class D | -0.15% | -1.11% | -1.14% | 99% | 98% |
| Class F | 0.01% | -0.48% | -0.73% | 99% | 101% |

Table 12 LDB Performance of EE1-1.2.3-test 2

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 (only HOP) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.31% | -2.64% | -2.37% | 100% | 99% |
| Class C | -0.31% | -2.32% | -2.23% | 100% | 98% |
| Class E | -0.37% | -1.36% | 1.06% | 100% | 98% |
| Overall | -0.33% | -2.21% | -1.47% | 100% | 99% |
| Class D | -0.45% | -2.38% | -3.71% | 100% | 99% |
| Class F | -0.31% | -1.30% | -1.51% | 99% | 102% |

Conclusions

In this contribution, two kinds of input-output rotation methods are proposed to further improve the coding performance. Experimental results demonstrate the effectiveness of the proposed method.

References

- E. Alshina, F. Galpin. BoG report on NN-filter design unification. JVET-AD0380, 30th Meeting, Antalya, TR, 21–28 April 2023.

- E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-AE2016, 31st Meeting, Geneva, CH, 11–19 July 2023.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).