| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0162-v2 |
| --- | --- |

| Title: | AHG12: Fixes to template matching |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Chen Ru-Ling Liao Xinwei Li Yan Ye | Tel: Email: | jiechen.cj@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

This contribution reports and fixes the issues in the design of template matching in ECM-10.0. It is reported that the proposed fixes give {-0.01%, 0.05%, 0.03%} in RA and {-0.05%, 0.09%, -0.08%} in LD overall coding gain with no encoding and decoding time increase.

Problem statement

Template matching (TM) was adopted in ECM software to refine the motion for merge mode with decoder side motion search. Two issues of TM are reported in the contribution.

First, in the current design of TM in ECM-10.0, for each search position, only the weighted sum of absolute difference (SAD) between the template of current block and the template of predicted block is used as the TM cost for the current position. The motion vector (MV) cost which is the difference between the original MV and the refined MV is calculated in the software but ignored in the TM cost calculation.

Second, for bi-prediction block, the two MVs are refined with an iteration process as follows:

- Step 1: MV0 is refined with TM to get MV0’ and the corresponding TM cost is obtained as CostUni0

- Step 2: MV1 is refined with TM to get MV1’ and the corresponding TM cost is obtained as CostUni1.

- Step 3:

if CostUni1 >= CostUni0, MV0’ is fixed and used to further refine MV1’ to get MV1’’, the corresponding TM cost is obtained as CostBi, and CostUni is set to CostUni0;

otherwise MV1’ is fixed and used to further refine MV0’ to get MV0’’, the corresponding TM cost is obtained as CostBi, and CostUni is set to CostUni1.

if CostBi > 1.125*CostUni or this is not a low delay picture, goes to step 4; otherwise finish refinement

- Step 4:

if CostUni1 >= CostUni0 (i.e., MV1’ is refined to obtain MV1’’ in step 3), MV1’’ is fixed and used to further refine MV0’ to get MV0’’, the corresponding TM cost is obtained as CostBi, and CostUni is set to CostUni1;

otherwise (i.e., MV0’ is refined to obtain MV0’’ in step 3) MV0’’ is fixed and used to further refine MV1’ to get MV1’’, the corresponding TM cost is obtained as CostBi, and CostUni is set to CostUni0.

After refinement, CostBi is compared with CostUni. If CostBi > 1.125*CostUni, the current block is converted into a uni-prediction block. However, according to the above process, there is no guarantee CostUni is the smaller one between CostUni0 and CostUni1, and the bi-prediction TM cost should be compared with the best one of the uni-prediction TM cost to decide whether to convert the current bi-prediction block into a uni-prediction block.

Proposed fixes

To solve the above-mentioned issues, the contribution proposes the following two changes.

First, it is proposed to consider MV cost in TM cost calculation. It is implemented by changing the code

cost = xGetTempMatchError<tplSize>(mvCand);

to

cost += xGetTempMatchError<tplSize>(mvCand);

Second, it is proposed to always compare CostBi with the smaller one of CostUni0 and CostUni1 for lowdelay picture. If CostBi > 1.125* min (CostUni0, CostUni1), convert the current bi-prediction block into a uni-prediction with the corresponding refined MV.

Simulation results

The proposed methods are implemented on top of ECM-10.0 and are tested under CTC for enhanced compression tool testing [1] with random access and low delay B configurations. Table 1 shows the simulation results of the proposed changes.

Table 1. Simulation results of the proposed method

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | 0.02% | 0.03% | 0.13% | 99.9% | 98.9% | 100.1% |
| Class A2 | 0.02% | -0.02% | 0.02% | 99.6% | 99.6% | 100.0% |
| Class B | 0.00% | 0.18% | -0.05% | 99.4% | 99.5% | 100.2% |
| Class C | -0.06% | -0.04% | 0.07% | 99.2% | 99.2% | 100.0% |
| Class E |  |  |  |  |  |  |
| Overall | -0.01% | 0.05% | 0.03% | 99.5% | 99.3% | 100.1% |
| Class D | -0.03% | 0.11% | 0.02% | 98.9% | 99.3% | 100.0% |

|  | Low delay B Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.01% | 0.50% | -0.33% | 100.0% | 99.2% | 99.9% |
| Class C | -0.11% | -0.25% | 0.07% | 100.8% | 100.6% | 100.0% |
| Class E | -0.04% | -0.13% | 0.12% | 100.1% | 99.7% | 100.0% |
| Overall | -0.05% | 0.09% | -0.08% | 100.3% | 99.8% | 99.9% |
| Class D | -0.05% | 0.09% | -0.09% | 99.5% | 99.6% | 100.0% |

Conclusion

Two issues in the design of TM in ECM-10.0 software are reported and fixed in this contribution. The fixes only involve very few lines of code change. Some coding gain was observed by fixing the issues and no impact on running time and memory usage. It is suggested to adopt the proposed fixes into next release of ECM software.

References

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AE2017, Jul. 2023.

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).