| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0171 |
| --- | --- |

| Title: | Non-EE2: Sample distance-based weighting for CIIP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Zhao Seung-Hwan Kim 10680 Treena St. St# 550 San Diego, CA 92131, USA | Tel: Email: | +1-858-635-5226 jie.zhao@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

# Abstract

In this contribution, sample distance-based weighting for CIIP is proposed for CIIP angular modes. Weights of intra and inter prediction are calculated based on sample distance to the boundary, intra mode angle, as well as block size.

Experimental results show that there is BD rate gain of 0.0x%, 0.0x%, 0.0x% in RA with coding time of xxxx% and xxx% for encoding and decoding.

# Background

In ECM’s CIIP mode, the prediction samples are generated by weighting an inter prediction signal predicted using CIIP-TM merge candidate and an intra prediction signal predicted using TIMD derived intra prediction mode [1]. The TIMD derivation method is used to derive the intra prediction mode in CIIP.

If the derived intra prediction mode is an angular mode, the weights (wIntra, wInter) for the two predictions are: For near-horizontal modes (2 <= angular mode index < 34), the current block is vertically divided as shown in Figure 1(a); for near-vertical modes (34 <= angular mode index <= 66), the current block is horizontally divided as shown in Figure 1(b). The (wIntra, wInter) for different sub-blocks are shown in Table 1.

Table 1. The weights used for CIIP angular modes.

| The sub-block index | (wIntra, wInter) |
| --- | --- |
| 0 | (6, 2) |
| 1 | (5, 3) |
| 2 | (3, 5) |
| 3 | (2, 6) |

# Proposed methods

In current ECM’s sub-block based weighting, the weight change at the boundary of sub-blocks are not gradual, so it may cause blocking artefacts, higher residuals and less compression efficiency. In addition, categorizing angular modes to only near-horizontal and near-vertical modes are too rough to capture the detailed pixel value variations. Therefore it is proposed to use sample distance and finer representation of the angles to derive the weights.

In our test, the angles are categorized as near-horizontal, near-vertical and near-diagonal three cases. Here, 34 is the diagonal mode index.

- near –horizontal : (2 <= angular mode index < 34 - 6),

- near-vertical modes (34+6 < angular mode index <= 66),

- near diagonal (34-6 <= angular mode index <= 34+6),

The weighs wIntra and wInter are derived as:

- near –horizontal : wIntra = ((width - x) * 64 + 32) / width – 4

- near-vertical modes:

  - wIntra = ((height - y) * 64 + 32) / height – 4

- near diagonal modes:

  - wIntra = ((height - y + width - x) * 64 + 32) / (width + height) - 4;

wIntra is clipped to within [4, 60], and wInter = 64-wIntra.

In the above formula, offset 32/width – 4, 32/ height – 4, and 32/ (width + height) – 4 are added. Its effect is to make the weight adaptive to the block size. So for smaller block, it gives more weight to intra. Experiments show that adding this offset has slightly better performance than without using this offset.

# Experimental results

Experiments are performed based on ECM common test condition [2]. In Random Access test case, the proposed method has average gain of xxx%, xxx%, xxx%.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.04% | 0.09% |  |  |
| Class A2 | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class B | -0.04% | 0.11% | 0.06% |  |  |
| Class C | -0.04% | -0.03% | -0.02% |  |  |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class D | -0.01% | 0.16% | 0.00% |  |  |
| Class F | -0.03% | -0.01% | -0.04% |  |  |
| Class TGM | -0.01% | -0.02% | -0.01% |  |  |

# Conclusion

In this contribution, sample distance-based weighting is tested for CIIP. Results show that with more accurate representation of the weights, there are an average gain of 0.0x% in RA test conditions. It is proposed to further study this approach in EE.

# References

- M. Coban, R.-L. Liao, K. Naser, J. Ström, L. Zhang, “Algorithm description of Enhanced Compression Model 9 (ECM 9)”, JVET-AD2025, ,  April 2023.

- M. Karczewicz and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing”, JVET-AE2017, April 2023.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).