| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0181-v1 |
| --- | --- |

| Title: | EE1-1.1.4.a: Simplified feature extraction for HOP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Li, Kai Zhang, Li Zhang | Tel: Email: | yue.li@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution presents the test result of EE1-1.1.4.a. In this test, simplified feature extraction is applied to HOP. The proposed change leads to significant worst-case inference memory reduction, as well as kMAC/pixel reduction. Compared with existing HOP filter, worst-case inference memory is reduced by 75%, kMAC/pixel is reduced by 4.2%, reportedly. BD-rate changes on top of NNNC-6.0 anchor (NN intra + LOP) and NNVC-6.0 HOP anchor (NN intra + HOP) are reportedly summarized as below:

Compared with NNVC-6.0 anchor: RA: -5.16%, -18.49%, -19.50%, AI: -3.22%, -14.53%, -15.52%

Compared with NNVC-6.0 HOP anchor: RA: 0.24%, 0.86%, 0.52%, AI: 0.09%, 0.22%, 0.36%

Introduction

HOP filter extract features from inputs with convolutional layers. Inputs of more importance, such as reconstruction and prediction, are allocated with more channels as shown in Fig. 1. It can be observed that the first layer extracts 192 feature maps from reconstruction at original resolution, causing the worst-case inference memory in the HOP.

Proposed method

Network structure

To reduce the worst-case inference memory consumption, it is proposed to set the stride of convolution as 2 for all input feature extraction layers as shown in Fig. 2 and accordingly remove the stride of 2 in the transition layer. In this way, all the internal computations operate in the reduced resolution. The worst-case inference memory is reduced significantly, i.e., by 75%, and kMAC/pixel is also reduced, i.e. by 4.2%.

Fig. 1. Existing HOP model.

Fig. 2. Proposed HOP model.

Inference

SADL is used for performing the inference of the proposed NN filters in VTM. Both weights and feature maps are represented with int16 precision using a static quantization method. The network information in the inference stage is provided in Table 1.

Table 1. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | same as HOP |
|  | Parameter Precision (Bits) | 16 (I) |
|  | Memory Parameter (MB) | same as HOP |
|  | Multiply Accumulate (kMAC/pixel) | 457 |
| Optional | Total Conv. Layers | 154 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 144×144, 272×272 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Other information: |  |

Training

HOP stage 3 training is applied. The network information in the training stage is provided in Table 2.

Table 2. Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: A100-SXM-80GB |
|  | Framework: | PyTorch v1.9 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 20 |
|  | Batch size: | 64 |
|  | Training time: | 200h |
|  | Training data information: | DIV2K, BVI-DVC, TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | NNVC, QP {22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional | Number of iterations |  |
|  | Patch size | 144×144, 272×272 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |

Experimental results

The proposed method is integrated into NNVC-6.0 and tested according to the common test conditions defined in [1].

Compared with NNVC-6.0 anchor (nn intra + lop), the proposed method brings on average {-5.16%, -18.49%, -19.50%} and {-3.22%, -14.53%, -15.52%} BD-rate changes for {Y, Cb, Cr} under RA and AI configurations.

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 Anchor |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |  |
| Class A1 | -5.30% | -17.22% | -23.01% | 132% | 495% |  |
| Class A2 | -5.42% | -18.53% | -15.25% | 128% | 529% |  |
| Class B | -4.89% | -19.27% | -19.65% | 131% | 497% |  |
| Class C | -5.20% | -18.43% | -19.89% | 122% | 464% |  |
| Class E |  |  |  |  |  |  |
| Overall | -5.16% | -18.49% | -19.50% | 128% | 494% |  |
| Class D | -5.64% | -19.32% | -20.16% | 120% | 416% |  |
| Class F | -3.65% | -13.64% | -13.76% | 143% | 156% |  |
|  | All Intra Main10 |  |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 Anchor |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT |  | DecT CPU |  |
| Class A1 | -2.41% | -15.01% | -16.52% | 123% |  | 498% |  |
| Class A2 | -2.94% | -14.44% | -12.32% | 112% |  | 474% |  |
| Class B | -2.91% | -14.49% | -15.29% | 112% |  | 478% |  |
| Class C | -3.51% | -12.85% | -15.58% | 106% |  | 409% |  |
| Class E | -4.44% | -16.47% | -18.02% | 113% |  | 462% |  |
| Overall | -3.22% | -14.53% | -15.52% | 113% |  | 462% |  |
| Class D | -2.88% | -13.22% | -15.45% | 106% |  | 341% |  |
| Class F | -3.13% | -11.90% | -12.68% | 107% |  | 444% |  |

Compared with NNVC-6.0 HOP anchor (nn intra + hop), the proposed method brings on average {0.24%, 0.86%, 0.52%} and {0.09%, 0.22%, 0.36%} BD-rate changes for {Y, Cb, Cr} under RA and AI configurations.

|  | Random access Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 HOP Anchor |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT |  | DecT CPU |  |
| Class A1 | 0.13% | 0.68% | -1.33% | 96% |  | 86% |  |
| Class A2 | 0.31% | 0.73% | 4.81% | 96% |  | 89% |  |
| Class B | 0.18% | 0.68% | -0.97% | 95% |  | 87% |  |
| Class C | 0.34% | 1.30% | 0.55% | 95% |  | 82% |  |
| Class E |  |  |  |  |  |  |  |
| Overall | 0.24% | 0.86% | 0.52% | 95% |  | 86% |  |
| Class D | 0.21% | 0.19% | -0.15% | 95% |  | 85% |  |
| Class F | 0.11% | 0.41% | -0.32% | 92% |  | 85% |  |
|  | All Intra Main10 |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 HOP Anchor |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |  |
| Class A1 | 0.17% | 0.53% | 1.77% | 95% | 90% |  |
| Class A2 | 0.19% | 0.97% | 1.65% | 98% | 88% |  |
| Class B | -0.03% | -0.09% | -0.56% | 98% | 90% |  |
| Class C | 0.12% | -0.31% | -0.39% | 100% | 89% |  |
| Class E | 0.08% | 0.38% | 0.17% | 96% | 85% |  |
| Overall | 0.09% | 0.22% | 0.36% | 98% | 89% |  |
| Class D | 0.10% | 0.29% | 0.85% | 102% | 84% |  |
| Class F | -0.32% | 0.12% | -0.18% | 98% | 88% |  |

Conclusions

This contribution presents a simplified feature extraction method for HOP. The proposed filtering method brings a significant worst-case inference memory reduction, as well as kMAC/pixel reduction. It is therefore suggested to adopt the proposed design into the HOP of NNVC.

References

- E. Alshina, R. -L. Liao, S. Liu, A. Segall, Common test conditions and evaluation procedures for neural network-based video coding technology, JVET-AE2016, Geneva, CH, July 2023.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).