| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32th Meeting, Hanover, DE, 13–20 October 2023 | Document: JVET-AF0187-v1 |
| --- | --- |

| Title: | Compression of Gaming Contents |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zehui Lin Kangying Cai Yin Zhao Elena Alshina | Tel: Email: | caikangying@huawei.com |
| Source: | Huawei Technologies |  |  |

_____________________________

- Abstract

With the widely adoption of cloud gaming, efficient compression of gaming content becomes more and more important. The characteristics of gaming content are quite different from those of the TGM sequences. Performance on several gaming sequences for ECM, VTM and HM are tested. This contribution proposes to extend test condition by adding gaming contents and develop specific compression solution for gaming contents.

Introduction

Cloud gaming is widely adopted by industry. Examples include Google Stadia, Xbox cloud gaming and NVidia’s GeForce Now. Efficient compression of gaming sequences is one of the key factors for the success of cloud gaming. Nowadays, typical bitrates for high quality distribution of gaming sequences could go up to 8Mbps~20Mbps, which is much higher than natural video sequences. Therefore, it is necessary to improve the compression performance of gaming contents. Requirements for gaming sequence coding comprise

- Resolution: 1080p, 720p…

- Frame rate: 60fps, 30fps

- Low latency: end to end latency < 100ms

Test dataset and compression performance

Test dataset

It is lacking of gaming contents in current JVET test dataset. As show in Figure 1, typical gaming sequence is very different from the TGM class in JVET.

Moreover, as mentioned by JVET-Y0041 [1], gaming sequences usually could be accompanied by auxiliary information such as depth, albedo and motion vector, which could be used to improve the compression efficiency. Those auxiliary information should also be provided with the corresponding test contents.

- (b)

Figure 1 (a) TGM class in JVET. (b) Typical gaming conent: Genshin Impact CREDIT MIHOYO. Source: https://www.forbes.com/sites/davidthier/2020/10/05/genshin-impact-feels-destined-to-change-the-gaming-industry/?sh=5bfb2d4f68b0

Compression performance of available codecs

Figure 2 shows the compression performance of HM, VTM and ECM on two gaming-like sequences (30fps). Although ECM could achieve significant gain over VTM and HM, its much longer coding time makes it unsuitable for cloud gaming scenario.

Figure 2 Compression performance (low delay P mode).

Recommendation

Given insufficient gaming content sequences in JVET test dataset, it is recommended to extend test condition by adding sufficient gaming sequences. The following auxiliary information, which could be provided by game engine, should also be provided along with the gaming contents

- Camera information and depth map

- Motion vector

- Any other auxiliary information can be retrieved from the rendering pipeline

To meet the requirements for cloud gaming industry, it is also recommended to develop specific compression solutions for gaming content in low delay mode.

References

[1] G. Martin-Cocher, M.Badawi, T. Poirier, S. Puri, K. Nasser, “Proposed new class of sequences with depth and optical flow information”, JVET-Y0041, January 2022

Patent rights declaration(s)

Huawei Inc. does not have any current or pending patent rights relating to the technology described in this contribution.