| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0208-v1 |
| --- | --- |

| Title: | EE1-2.1: Deep Reference Frame Generation for Inter Prediction Enhancement |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weijie Bao, Xin Chen, Jianghao Jia, Yuantong Zhang, Zhenzhong Chen Wuhan University Zizheng Liu, Xiaozhong Xu, Shan Liu Tencent | Email: | {baoweijie, cinched, jiajh2021, yuantongzhang, zzchen} @whu.edu.cn {zizhengliu, xiaozhongxu, shanl} @tencent.com |
| Source: | Wuhan University, Tencent |  |  |

_____________________________

- Abstract

This contribution reports the EE1-2.1 results of NN-based inter prediction. Based on JVET-AE0112 [1], this contribution has simplified the network architecture to reduce computational complexity and has accelerated float32 SADL inference. It is reported that this method can achieve X.XX%/X.XX%/X.XX% bitrate savings for the Y/U/V components under the NNVC-RA configuration when compared to the anchor VTM-11.0_NNVC-6.0 (LOP-1 filter and NN-Intra are enabled by default).

Introduction

JVET-AE0112 [1] proposes a deep reference frame generation method to enhance VVC inter prediction. During the encoding and decoding process, the network receives two reconstructed frames from the decoded picture buffer as inputs and generates a new frame. The generated frame will be put into the reference picture lists as an additional reference for the current encoding picture, which enhances the inter prediction.

In this contribution, the network architecture is simplified, and the float32 SADL inference is accelerated. The main modifications are summarized as:

- Remove the flow-mask operation during flow-warp operation in the float32 SADL inference process, while retaining it during the training process.

- Accelerate float32 SADL inference by enhancing the SADL extension features [2] and incorporating SIMD support (AVX2 and AVX512).

Proposed Method

Framework

Fig. 1: The framework of the proposed DRF method

The framework of the proposed DRF method is depicted in Figure 1. In this approach, two reconstructed frames chosen from the DPB are inputted into the deep reference frame generation network to synthesize a new frame. This synthesized frame is then added to both reference picture list0 and list1, using the same POC as the current encoding frame. It serves as an additional reference frame for subsequent encoding processes.

Network architecture

Fig. 2: The architecture of the DRF network

Figure 2 illustrates the architecture of the proposed DRF network. In this contribution, the flow-mask operation has been removed to reduce computational complexity with minimal loss, thus reducing the number of layers in the SADL model from 914 to 803 layers.

SADL implementation

This contribution accelerates the float32 SADL inference of DRF. Specifically, optimizations have been made to certain operators within the extended SADL library [2], including GridSample, Resize, and Conv2dTranspose. Support for AVX2 and AVX512 has been added, and AVX512 was utilized in experiments.

Network information in training stage

Table 1: Network information for the NN-based video coding tool testing in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GTX 3090 x 2 x 24GB |
|  | Framework | PyTorch v1.9.0 |
|  | Number of GPUs per Task | 2 |
|  | Epoch | 200 |
|  | Batch size | 3Kx24 |
|  | Loss function | L1 |
|  | Training time (for 1 model) | 60h |
|  | Training data information | Vimeo90K |
|  | Training configurations for generating compressed training data (if different to VTM CTC) | VTM-11.0_NNVC-5.0, QP{22, 27, 32, 37, 42} |
| Optional | Number of iterations |  |
|  | Patch size | 256x256 |
|  | Learning rate | 1e-4 |
|  | Learning rate update strategy |  |
|  | Optimizer | ADAM |
|  | Preprocessing |  |
|  | Mini-batch selection process |  |
|  | Training data update strategy |  |
|  | Other information |  |

Network information in inference stage

Table 2: Network information for the NN-based video coding tool testing in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment |  |
|  | GPU Type | CPU only |
|  | Framework | SADL |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 3800K |
|  | Total Number of Parameters (All Models) | 3800K |
|  | Parameter Precision (Bits) | 32 (F) |
|  | Multiply Accumulate (kMAC/pixel) | 504 |
| Optional | Total Conv. Layers | 101 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size | 1 |
|  | Patch size | 256x256 (with 16 boundary padding) |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information |  |

Results

The method has been implemented on the VTM-11.0_NNVC-6.0, and the experimental results of the proposed DRF method under NNVC-RA configuration are presented in Table 3. As depicted in Table 3, the proposed method demonstrates a BD-rate reduction of X.XX%/X.XX%/X.XX% for the Y/U/V components compared to the NNVC-6.0 anchor.

Table 3: Performance of the proposed method (NNVC-RA, full length)

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_NNVC-6.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -1.71% | -5.28% | -4.48% | -2.09% | -6.55% | -5.65% | 138% | 1195% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -1.90% | -3.59% | -3.68% | -2.75% | -6.09% | -6.02% | 138% | 1236% |
| Class C | -2.89% | -3.17% | -3.72% | -4.39% | -6.14% | -6.79% | 125% | 1189% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -2.70% | -1.87% | -1.64% | -3.93% | -6.53% | -6.24% | 124% | 1053% |
| Class F | -0.97% | -0.99% | -0.64% | -1.58% | -2.32% | -2.39% | 157% | 1240% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Conclusions

This contribution presents a deep reference frame generation (DRF) method for VVC inter prediction enhancement. Implemented on the VTM-11.0_NNVC-6.0, the proposed method can bring X.XX%/X.XX%/X.XX% coding efficiency improvements with a computational complexity of 504 kMAC/pixel under NNVC-RA configuration.

Reference

[1] W. Bao, W. Meng, J. Jia, Y. Zhang, H. Wang, Z. Chen, Z. Liu, X. Xu, and S. Liu. “EE1-5.1: Deep Reference Frame Generation for Inter Prediction Enhancement”. JVET-AE0112, 31st Meeting, Geneva, CH, 11–19 July 2023.

[2] W. Bao, D. Ding, J. Wang, J. Jia, and Z. Chen. “AHG14: The extension of SADL library”. JVET-AD0365, 30th Meeting, Antalya, TR, 21–28 April 2023.

Patent rights declaration(s)

Wuhan University does not have any current or pending patent rights relating to the technology described in this contribution.

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).