| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0235-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AF0131 (EE2-2.2: DIMD with filtered template) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jie Chen | Tel: Email: | jiechen.cj@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

This contribution is a crosscheck report of test 2.2a and test 2.2b of EE2 which is reported in JVET-AF0131[1]. It is confirmed that the results match with the results reported in JVET-AF0131.

Introduction

JVET-AF0131 reported tests results of EE2-2.2a and EE2-2.2b. In the test, the template is filtered using a 3x3 filter operator before obtaining the gradient histogram. In EE2-2.2a, the gradient histogram derivation uses 3x2 and 2x3 gradient operators, respectively. In EE2-2.2b, the same 3x3 Sobel gradient operators as in ECM are used.

Crosscheck results

The tests are conducted under CTC[2] and results are confirmed to be identical to those reported in JVET-AF0131.

- Test 2.2a

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.09% | -0.01% | -0.11% | 103.4% | 99.7% | 100.0% |
| Class A2 | -0.07% | 0.11% | 0.01% | 102.6% | 100.0% | 100.0% |
| Class B | -0.08% | 0.05% | 0.02% | 103.0% | 99.6% | 100.0% |
| Class C | -0.06% | 0.06% | 0.15% | 103.1% | 98.8% | 100.0% |
| Class E | -0.09% | -0.02% | -0.25% | 103.6% | 99.5% | 100.0% |
| Overall | -0.08% | 0.04% | -0.02% | 103.1% | 99.5% | 100.0% |
| Class D | -0.04% | 0.07% | -0.09% | 102.7% | 99.4% | 100.0% |
| Class F | -0.06% | -0.05% | 0.30% | 102.2% | 99.9% | 100.0% |

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.05% | -0.05% | -0.04% | 100.8% | 99.1% | 100.2% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class B | -0.08% | 0.08% | 0.08% | 100.5% | 99.9% | 100.2% |
| Class C | -0.02% | 0.07% | -0.06% | 100.6% | 99.4% | 100.0% |
| Class E |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class D | -0.02% | 0.00% | 0.19% | 100.6% | 99.9% | 100.0% |
| Class F | -0.06% | 0.06% | -0.09% | 100.6% | 98.7% | 99.8% |

- Test 2.2b

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.11% | -0.13% | 0.01% | 103.1% | 100.8% | 100.0% |
| Class A2 | -0.09% | -0.04% | -0.04% | 102.6% | 100.2% | 100.1% |
| Class B | -0.07% | 0.00% | -0.06% | 102.8% | 98.6% | 100.0% |
| Class C | -0.06% | 0.02% | 0.03% | 102.8% | 98.9% | 100.0% |
| Class E | -0.06% | 0.21% | -0.16% | 103.6% | 99.6% | 100.0% |
| Overall | -0.07% | 0.01% | -0.04% | 103.0% | 99.5% | 100.0% |
| Class D | -0.03% | -0.06% | -0.02% | 102.2% | 100.1% | 100.0% |
| Class F | -0.08% | 0.15% | 0.12% | 101.6% | 99.6% | 100.0% |

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class B | -0.08% | 0.18% | -0.11% | 100.5% | 99.8% | 100.3% |
| Class C | -0.04% | -0.04% | -0.11% | 101.0% | 99.5% | 100.0% |
| Class E |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class D | -0.06% | -0.22% | -0.03% | 100.7% | 99.9% | 100.0% |
| Class F | 0.06% | 0.30% | 0.40% | 100.4% | 98.8% | 99.9% |

Conclusion

This contribution is a crosscheck report of EE2-2.2a and EE2-2.2b. It is confirmed that the results match with the results reported in JVET-AF0131.

References

C. Zhou, Z. Lv, “EE2-2.2: DIMD with filtered template”, JVET-AF0131, Oct. 2023

M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AE2017, Jul. 2023.