| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0261-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AF0190 (EE2-Test4.1: Enabling template-based inter tools for the RPR) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Z. Zhang | Tel: Email: | zhizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the results of crosschecking JVET-AF0190 proposed by Kwai Inc. JVET-AF0190 proposes to allow use TM cost to sort the inter merge candidates, affine MMVD candidates, BCW index, reference list index and MMVD sign and magnitude-suffix prediction when reference picture has a difference resolution compared to the current picture. The proposed method also allows LIC to be used when reference picture has a difference resolution compared to the current picture. The crosscheck reports the performance of RPR setting in RA and LDB configurations over ECM-10. The simulation results match the reported results in JVET-AF0190 in term of BDR performance.

# Verification

Kwai had provided the software and results of their proposed methods in JVET-AF0190. The code was updated according to the description in the proposals. The proposed methods were implemented on top of ECM-10.

The BD-rate results match the results reported. The simulation results are as follows:

## Test 4.1a TM tools enabled to RPR setting of 1.5x

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10 RPR 1.5x |  |  |  |  |  |  |  |  |  |
|  | PSNR1 |  |  |  |  | PSNR2 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | -1.10% | -1.27% | -1.03% | 101.9% | 102.0% | -0.98% | -1.21% | -0.95% | -0.98% | -1.21% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.95% | -0.51% | -0.95% | 101.5% | 101.2% | -0.68% | -0.50% | -0.91% | 101.5% | 101.2% |
| Class F |  |  |  |  |  |  |  |  |  |  |

|  | Low delay B Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10 RPR 1.5x |  |  |  |  |  |  |  |  |  |
|  | PSNR1 |  |  |  |  | PSNR2 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | -0.88% | -0.99% | -0.95% | 102.4% | 103.0% | -0.86% | -1.10% | -1.03% | 102.4% | 103.0% |
| Class E | -0.62% | -0.43% | 0.09% | 102.7% | 103.7% | -0.64% | -0.52% | 0.06% | 102.7% | 103.7% |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.48% | 0.26% | 0.13% | 102.1% | 102.5% | -0.49% | 0.23% | 0.15% | 102.1% | 102.5% |
| Class F |  |  |  |  |  |  |  |  |  |  |

## Test 4.1a TM tools enabled to RPR setting of 2.0x

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10 RPR 2.0x |  |  |  |  |  |  |  |  |  |
|  | PSNR1 |  |  |  |  | PSNR2 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | -1.07% | -1.33% | -1.32% | 101.7% | 102.8% | -0.83% | -1.16% | -1.18% | 101.7% | 102.8% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -1.01% | -0.85% | -0.90% | 101.4% | 101.2% | -0.60% | -0.70% | -0.69% | 101.4% | 101.2% |
| Class F |  |  |  |  |  |  |  |  |  |  |

|  | Low delay B Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10 RPR 2.0x |  |  |  |  |  |  |  |  |  |
|  | PSNR1 |  |  |  |  | PSNR2 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | -0.87% | -0.74% | -0.58% | 103.6% | 103.6% | -0.76% | -0.57% | -0.47% | 103.6% | 103.6% |
| Class E | -0.59% | -0.28% | -0.08% | 102.4% | 102.1% | -0.53% | -0.53% | -0.31% | 102.4% | 102.1% |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.57% | 0.01% | -0.04% | 101.3% | 103.3% | -0.40% | 0.06% | -0.02% | 101.3% | 103.3% |
| Class F | -0.95% | -0.41% | -0.50% | 102.0% | 100.8% | -0.73% | -0.15% | -0.76% | 102.0% | 100.8% |

## Test 4.1b TM tools and LIC enabled to RPR setting of 1.5x

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10 RPR 1.5x |  |  |  |  |  |  |  |  |  |
|  | PSNR1 |  |  |  |  | PSNR2 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | -1.32% | -1.41% | -1.10% | 102.5% | 103.5% | -1.13% | -1.31% | -0.99% | 102.5% | 103.5% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -1.11% | -1.10% | -1.04% | 101.6% | 102.2% | -0.79% | -1.10% | -0.95% | 101.6% | 102.2% |
| Class F |  |  |  |  |  |  |  |  |  |  |

|  | Low delay B Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10 RPR 1.5x |  |  |  |  |  |  |  |  |  |
|  | PSNR1 |  |  |  |  | PSNR2 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | -1.10% | -1.18% | -1.23% | 102.9% | 104.2% | -1.07% | -1.28% | -1.34% | 102.9% | 104.2% |
| Class E | -0.87% | -0.09% | -0.10% | 103.0% | 105.7% | -0.92% | -0.14% | -0.18% | 103.0% | 105.7% |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.61% | 0.03% | -0.01% | 101.7% | 100.8% | -0.58% | 0.06% | -0.13% | 101.7% | 100.8% |
| Class F |  |  |  |  |  |  |  |  |  |  |

## Test 4.1b TM tools and LIC enabled to RPR setting of 2.0x

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10 RPR 2.0x |  |  |  |  |  |  |  |  |  |
|  | PSNR1 |  |  |  |  | PSNR2 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | -1.23% | -1.31% | -1.06% | 102.0% | 104.0% | -0.94% | -1.12% | -0.88% | 102.0% | 104.0% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -1.18% | -0.94% | -1.11% | 101.5% | 102.2% | -0.73% | -0.86% | -0.88% | 101.5% | 102.2% |
| Class F |  |  |  |  |  |  |  |  |  |  |

|  | Low delay B Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10 RPR 2.0x |  |  |  |  |  |  |  |  |  |
|  | PSNR1 |  |  |  |  | PSNR2 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | -0.91% | -0.75% | -0.17% | 104.3% | 106.0% | -0.77% | -0.70% | -0.08% | 104.3% | 106.0% |
| Class E | -0.62% | -0.11% | -1.40% | 102.1% | 103.8% | -0.57% | -0.21% | -1.53% | 102.1% | 103.8% |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.51% | 0.29% | -0.22% | 102.4% | 104.2% | -0.41% | 0.44% | 0.05% | 102.4% | 104.2% |
| Class F | -1.20% | -1.20% | -0.60% | 102.4% | 101.3% | -0.78% | -0.69% | -0.98% | 102.4% | 101.3% |

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).