| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0266-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AF0153 (EE1-1.1.1: Optimization for complexity-performance trade-off of HOP network) test1 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Zhihuang Xie | Tel: Email: | xiezhihuang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution reports the cross-check results of JVET-AF0153, which is EE1-1.1.1. Only test 1 in this EE test was crosschecked, and both training and inference were conducted.

The performance difference between proponent’s and crosschecker’s is summarized as below:

{Y: 0.12%  U: -0.13%  V: -0.22%} for AI;

{Y: 0.12%  U: 0.21%  V: 0.17%} for RA;

Introduction

JVET-AF0153[1] proposes further optimization with new residual blocks to improve the complexity-performance trade-off of HOP network. Specifically, based on the group convolution and channel reduction, two kinds of residual blocks are proposed to reduce the computational complexity while maintaining performance as much as possible. In Test 1, the group convolution is integrated into HOP residual block to improve the complexity-performance trade-off.

Crosscheck status

The dataset of HOP stage3-int16 was used to train EE1-1.1.1 test1 in this crosscheck. Default setting of HOP training process was kept unchanged.

After 20 training epochs was completed, the last model was converted to SADL integer 16 bits and did the inference crosscheck.

Results

The simulation was conducted under the NNVC CTC[2], which means that HOP filter and NN intra were enabled for both anchor and test.

Over proponent’s results

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over proponent’s results |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.03% | -0.02% | 0.18% | 109% | 146% |
| Class A2 | 0.14% | -0.29% | -0.78% | 103% | 139% |
| Class B | 0.09% | -0.17% | -0.12% | 100% | 144% |
| Class C | 0.12% | 0.07% | -0.04% | 164% | 217% |
| Class E | 0.27% | -0.28% | -0.46% | 165% | 179% |
| Overall | 0.12% | -0.13% | -0.22% | 124% | 163% |
| Class D | 0.10% | -0.06% | -0.16% | 159% | 230% |
| Class F | -0.26% | -0.01% | -1.05% | 136% | 166% |

|  | Rondam Access 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over proponent’s results |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -0.13% | -0.84% | 0.64% | 117% | 145% |
| Class A2 | 0.28% | -0.03% | -0.11% | 96% | 117% |
| Class B | 0.17% | 0.80% | 0.13% | 142% | 174% |
| Class C | 0.14% | 0.43% | 0.09% | 133% | 167% |
| Class E |  |  |  |  |  |
| Overall | 0.12% | 0.21% | 0.17% | 124% | 153% |
| Class D | 0.15% | 1.03% | 1.00% | 134% | 174% |
| Class F |  |  |  |  |  |

Please note that the running time is not reliable.

Conclusion

The performance gap between crosschecker’s model and proponent’s is minor.

Reference

[1] R. Chang, et al. EE1-1.1.1: Optimization for complexity-performance trade-off of HOP network. JVET-AF0153, 32nd Meeting, Hannover DE. Oct. 13-20 2023.

[2] E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-AE2016, 31th Meeting, Geneva, 11–20 July 2023