| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0267-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AF0192 (EE1-1.1.5: Combination test of EE1-1.1.1 and EE1-1.1.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Zhihuang Xie | Tel: Email: | xiezhihuang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution reports the cross-check results of JVET-AF0192, which is EE1-1.1.5, the combination test of EE1-1.1.1 and EE1-1.1.5. Both training and inference crosscheck were conducted in this crosscheck.

The performance difference between proponent (Tencent)’s and crosschecker’s is summarized as below:

{Y: 0.00%  U: 0.49%  V: 0.16%} for AI;

{Y: 0.04%  U: -0.07%  V: -0.21%} for RA.

Introduction

JVET-AF0192[1], which is also known as EE1-1.1.5, proposes to combines EE1-1.1.1 and EE1-1.1.5 to improve the complexity-performance trade-off of HOP network. Specifically, a new residual block with group convolution is proposed in EE1-1.1.1 to reduce the computational complexity without affecting the coding performance. In EE1-1.1.2, reduced ranking of the two-components decomposition is introduced and order of the decomposition is alternated every block in the backbone to mitigate the impact of the rank reduction on decomposition symmetry.

Crosscheck status

The dataset of HOP stage3-int16 was used to train EE1-1.1.5 in this crosscheck. Default setting of HOP training process was kept unchanged.

After 20 training epochs was completed, the last model was converted to SADL integer 16 bits and did the inference crosscheck.

Results

The simulation was conducted under the NNVC CTC[2], which means that HOP filter and NN intra were enabled for both anchor and test.

Over proponent’s results

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over proponent’s results |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.02% | -0.01% | 0.02% | 179% | 146% |
| Class A2 | -0.03% | 0.20% | 0.07% | 164% | 138% |
| Class B | -0.02% | 0.84% | 0.01% | 184% | 171% |
| Class C | -0.03% | 0.15% | 0.47% | 90% | 96% |
| Class E | 0.07% | 1.17% | 0.22% | 91% | 94% |
| Overall | 0.00% | 0.49% | 0.16% | 136% | 128% |
| Class D | -0.02% | -0.05% | -0.21% | 91% | 98% |
| Class F |  |  |  |  |  |

|  | Rondam Access 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over proponent’s results |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.33% | 0.27% | -0.97% | 79% | 79% |
| Class A2 | -0.05% | -1.29% | 0.08% | 78% | 78% |
| Class B | 0.02% | 0.26% | -0.35% | 115% | 116% |
| Class C | -0.07% | 0.16% | 0.32% | 97% | 110% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | -0.07% | -0.21% | 95% | 98% |
| Class D | -0.09% | 1.18% | 0.95% | 103% | 134% |
| Class F |  |  |  |  |  |

Please note that the running time is not reliable.

Conclusion

The performance gap between proponent’s model and crosschecker’s model is minor.

Reference

[1] R. Chang, et al. EE1-1.1.5: Combination test of EE1-1.1.1 and EE1-1.1.2. JVET-AF0192, 32nd Meeting, Hannover DE. Oct. 13-20 2023.

[2] E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-AE2016, 31th Meeting, Geneva, 11–20 July 2023