| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0292 |
| --- | --- |

| Title: | Considerations on LCEVC performances in relation to input document JVET-AF0075/m65560 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Simone Ferrara, Lorenzo Ciccarelli, Guendalina Cobianchi, Guido Meardi 20 Eastbourne Terrace London, United Kingdom | Tel: Email: | {simone.ferrara, lorenzo.ciccarelli, guendalina.cobianchi, guido.meardi} @v-nova.com |
| Source: | V-Nova |  |  |

# Executive Summary

This document is produced to provide constructive feedback on the methodology and the conclusions of input document JVET-AF0075 and a summary of the expected performance of MPEG-5 Low Complexity Enhancement Video Coding (LCEVC) on objective metrics vs. formal MOS, according to tens of extensive cross-checked evaluations performed over the course of LCEVC’s collaborative development phase as well as during its Verification Tests.

Most of the hypotheses and results presented in JVET-AF0075 had already been evaluated, expanded, discussed at length, and resolved during the development and verification of LCEVC, so this document aims to provide useful reminders for experts that may have been less involved in the standardization of LCEVC. Over the course of 11 MPEG meetings (MPEG 124 to MPEG 134), there have been over 30 documents evaluating the performances of LCEVC in terms of PSNR, VMAF, and formal subjective assessments, including LCEVC’s Verification Tests (m54455) and several evaluations of LCEVC using the same sequences as in JVET-AF0075 (such as N19160 , m53796, m52997 and m53523. A good summary of LCEVC tests preceding Verification Tests can be found in N19571.

All the above test results, including those from Verification Tests, are aligned with results in JVET-AF0075 when it comes to PSNR. In previous tests, VMAF results complemented PSNR on objective metrics, as requested by the CTC (following evidence presented in m51438 which proved the lack of correlation between PSNR and subjective performance and a higher correlation between VMAF and formal subjective performances) and, more importantly, ITU-R BT.500 formal MOS results consistently disprove some of the conclusions in JVET-AF0075 with regards to the correlation of objective metrics, and PSNR in particular, with visual quality. In this document we report a selection of previous conclusions.

Furthermore, the tests in JVET-AF0075 have been configured using the LTM at bitrates that are not consistent with the LCEVC CTC, according to which LCEVC encodings should have been “designed in order to achieve bitrates compatible with mass market distribution and quality levels amenable to subjective testing” as outlined in m52997. Testing very high data rates (e.g., up to 100 Mbps) using LTM default configurations originally aimed at operating points below 20 Mbps may lead to questionable conclusions, which therefore pertain to the specific settings and implementation of the current LTM, rather than to the LCEVC format per se. For instance, due to the intended range of LTM’s default configurations, the LCEVC quantization stepwidths used at very high bitrate was much higher (like almost an order of magnitude higher, as shown by configurations used in past tests at very high bitrates highlighted in N19571 et al.) than it is necessary for said very high levels of quality, consequently producing suboptimal results. Furthermore, LTM default configurations were calibrated for VTM 11 and should be adapted to the latest VTM.

In addition to LTM tests, this document presents the results obtained with commercial implementations of LCEVC (within ffmpeg) using two different base codecs – namely HEVC (in its x265 implementation) and VVC (in its beta MainConcept implementation) on the same set of contents used in JVET-AF0075.  Results are presented in terms of a wide range of objective metrics (including PSNR, VMAF, MS-SSIM and VMAF_NEG) with some sample screenshots to support the metrics. Once again, results are in line with previous tests, showing a very different behaviour (from very good to very bad) depending on the metric and on luma vs. chroma. It is thus very risky to draw conclusions by focusing on a single metric, in particular if that metric is PSNR, which for LCEVC consistently demonstrated lack of correlation with ground-truth formal MOS results, for reasons that were largely identified and clarified during the LCEVC standardization process.

This document also provides data about the possible luma and chroma settings to be used with LCEVC. These examples demonstrate the effects of different luma-chroma configurations on both visual quality and BD-rate gains for the sequences tested.  Additionally, the document confirms the potential validity of luma-based metrics like VMAF for LCEVC evaluations, consistent with other video coding schemes and contrary to the conclusions made in JVET-AF0075. LCEVC can indeed encode also chroma residuals (up to lossless if needed), and the default luma-only configuration is just a minor design choice aimed at optimising the visual quality impact of the LCEVC enhancement.  In any case, even when LCEVC residuals are applied with equal quantization step-width to both luma and chroma planes, the negative impact on the BD-rate of luma metrics, such as VMAF, is very small (< 1 percentage point) and does not materially affect the results.

Finally, this document also provides a reminder of the importance of the Low Complexity requirements of LCEVC, which provide uniquely impactful benefits in many of its real-world use cases.

# Introduction

More details can be found in the MPEG document, m65556. Considering that presented results are related to codecs from both, JVET and WG04, V-Nova would like to request discussing JVET-AF00xxx observations and results at a joint meeting between JVET and WG04.