| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0301-v1 |
| --- | --- |

| Title: | AHG12: SbTMVP with MMVD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lien-Fei Chen, Roman Chernyak, Xin Zhao, Xiaozhong Xu, Shan Liu 2747 Park Boulevard Palo Alto, CA 94306, U.S.A | Email: | lienfeichen@global.tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In this proposal, it is proposed to apply the MMVD on SbTMVP candidate in subblock MMVD merge list. The syntax element, MMVD index, is signaled to indicate the MMVD offset for the SbTMVP merge candidate, and this MMVD offset is used to derive the offset value of the displacement vector (DV) for each SbTMVP-MMVD candidate. The final DV for each SbTMVP-MMVD candidate is calculated from the DV of SbTMVP merge candidate with the selected offset. Different subblock-based motion field could be obtained by using the different DV offset at different MMVD offset position. In this proposal, the maximum candidate list size is 16. The concept of MMVD offset index in this proposal is also utilized in JVET-AF0134. Compared with JVET-AF0134, signaling these SbTMVP-MMVD index at subblock merge mode is the major difference, other design aspects in JVET-AF0134 could be harmonized in this proposal. The overall simulation results of our proposal on top of ECM-10.0 reference software are reported as below.

- RA: -0.xx%/-0.xx %/-0.xx % with xxx.x% EncT and xxx.x% DecT

- LB: -0.xx%/-0.xx %/-0.xx % with xxx.x% EncT and xxx.x% DecT

# Introduction

In ECM 10[1], affine MMVD mode is proposed to improve the coding gain by using subblock-based motion information with affine parameter. Although, SbTMVP is one of the candidates in subblock-based merge list, MMVD is not supported for SbTMVP in current ECM design; And the derived SbTMVP from the neighboring block may not be the optimal subblock-based motion field in ECM 10[1]. In order to improve the SbTMVP, the SbTMVP with MMVD was proposed in JVET-AC0213[2] and was adopted in EE2-2.3 test. In this meeting, the further improvement of SbTMVP with MMVD in AMVP mode is proposed in JVET-AF0134. In this proposed, it is proposed to apply SbTMVP-MMVD methodology in subblock merge mode instead of AMVP mode on top of ECM 10. The further improvement of SbTMVP with MMVD in JVET-AF0134 is also applicable to our signaling method.

# Proposed Methodology

In subblock MMVD merge list, it is proposed to apply the MMVD on SbTMVP candidate in subblock MMVD merge list. The syntax element, MMVD index, is signaled to indicate the MMVD offset for the SbTMVP merge candidate, and this MMVD offset is used to derive the offset value of the displacement vector (DV) for each SbTMVP-MMVD candidate. The final DV for each SbTMVP-MMVD candidate is calculated from the DV of SbTMVP merge candidate with the selected offset. By using the different DV offset, different subblock-based motion field could be obtained. In this proposal, the spiral scanning order is used to scan all MMVD candidate and the available candidate, which has the valid center motion vector, will be put into the candidate list in sequence. Regarding to the MMVD offset, the step size in test is designed as {4, 8, 12, 16, 20} and the unit in the step size is the integer pixel unit, and the number of directions is kept as 8 which is used in affine MMVD as well. Like the affine MMVD, the total number of the SbTMVP-MMVD candidates is less than or equal to 16. In order to improve the coding gain and runtime, some aspects are utilized in this proposal, the detailed is listed as below.

- Picture level flag of SbTMVP-MMVD mode is used in RA case. This flag is used to indicate whether the SbTMVP-MMVD is enabled or not when both of the SPS flag of SbTMVP-MMVD and the SPS control flag in picture header for SbTMVP-MMVD are true. The picture level enabling and disabling is determined based on the temporal ID and the SbTMVP-MMVD is only enabled at the highest two temporal layers for RA case.

- The constraint of the size of collocated position is extended from one CTU row to three CTU rows. This extension is applied for SbTMVP-MMVD only, and there is no size extension for TMVP and SbTMVP.

# Simulation Results

The proposed method is implemented on top of ECM-10.0 reference software[3] and tested by following the common test conditions (CTC) as described in [4].

Table 1 shows the overall performance for both of RA and LDB cases on top of ECM-10.0[3]. The simulation results may not be reliable because we don’t have time to run the anchor and the test at the same time and at the same cluster.

Table 1: simulation results on top of ECM-10.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-10 6d48808f |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.08% | -0.08% | -0.07% | 101.6% | 102.6% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.03% | 0.00% | -0.07% | 101.1% | 101.8% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-10 6d48808f |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.30% | -0.54% | -0.85% | 103.1% | 111.1% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Conclusion

In this proposal, it is proposed to apply the MMVD on SbTMVP candidate in subblock MMVD merge list. According to our simulation results, our method could achieve -0.xx%/-0.xx%/-0.xx% coding gain for Y/U/V channels in RA case and achieve -0.xx%/-0.xx%/-0.xx% coding gain in LB case. We recommend adopting this proposal for the EE study in next JVET meeting.

# Reference

- M. Coban, F. L. Léannec. R.-L. Liao, K. Naser, J. Ström, and L. Zhang, “Algorithm description of Enhanced Compression Model 10 (ECM 10)”, ISO/IEC JTC1/SC29 JVET-AE2024, July 2022.

- L.-F. Chen, R. Chernyak, X. Zhao, X. Xu, and S. Liu, “Non-EE2: SbTMVP with MMVD”, ISO/IEC JTC1/SC29 JVET-AC0213, Jan. 2023.

- ECM-10.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

- M. Karczewicz, and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, ISO/IEC JTC1/SC29 JVET-AE2017, July 2023.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).