| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0310-v1 |
| --- | --- |

| Title: | AHG9: Combined text for scope/persistence changes and addition of po_id to the SEI processing order SEI message and addition of the processing order nesting SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal, Information |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Technologies, Finland Gary J. Sullivan Dolby Laboratories, United States Yago Sanchez Fraunhofer HHI, Germany Ye-Kui Wang Bytedance Inc., United States Lulin Chen MediaTek Inc., United States | Email: | miska.hannuksela@nokia.com gary.sullivan@dolby.com yago.sanchez@hhi.fraunhofer.de yekui.wang@bytedance.com lulin.chen@mediatek.com |
| Source: | Nokia, Dolby Labs, Fraunhofer HHI, Bytedance Inc., MediaTek Inc. |  |  |

_____________________________

# Abstract

In response to the review and agreements in the JVET track meeting on 14 Oct 2023, this contribution proposes a combination of aspects from contributions JVET-AF0049, JVET-AF0061, JVET-AF0174, and JVET-AF0189 related to the scope/persistence of the SEI processing order SEI message, and the addition of po_id from contributions in JVET-AF0061, JVET-AF0067, and JVET-AF0174. The aspects proposed in this contribution can be summarized as follows:

- Change the persistence scope of the SEI processing order SEI message as follows: Each SEI message identified within the SEI processing order SEI message has the same persistence scope as if the SEI message was carried outside of the SEI processing order SEI message and not identified within an SEI processing order SEI message. The number of SEI messages and the type codes of the SEI messages indicated within the SEI processing order SEI message persist in decoding order from the first access unit in decoding order until the end of the CVS in output order.

- Remove of the capability of wrapping SEI messages within the SEI processing order SEI message and add of a processing order nesting SEI message that includes one or more SEI messages having the same persistence and are used in an indicated position within the processing order.

- Add processing order ID, po_id, as a u(8) syntax element.

- Replace the requirement

"When there are multiple SEI processing order SEI messages present in a CVS, they shall have the same content"

with

"When more than one SEI processing order SEI message with a particular value of po_id is present in a CVS, the values of po_num_sei_messages_minus2 and, for each value of i, the values of po_sei_wrapping_flag[ i ], po_sei_prefix_flag[ i ], po_sei_importance_flag[ i ], po_sei_payload_type[ i ], po_sei_processing_order[ i ] shall be the same as in the other SEI processing order SEI messages in the CVS with the same value of po_id."

- To cope with multiple PO SEI messages with different po_id values, relax the requirement of po_sei_processing_order[ i ] from being equal or incremented by 1 compared to the po_sei_processing_order[ i − 1 ] to being greater than or equal to po_sei_processing_order[ i − 1 ].

- To cope with multiple PO SEI messages with different po_id values, include one or more pon_target_po_id[ i ] values in the processing order nesting SEI message to identify associated PO SEI messages.

# Proposed text

Note: In the text below, the syntax elements referring to prefix bits or bytes may follow JVET-AE2027 (prefix bytes) or agreements of the JVET-AF meeting (prefix bits) in an inconsistent manner. This will be fixed during the integration of all related adoptions to the output text of the JVET-AF meeting.

# In subclause D.2.2, make the following changes:

...

Table D.1 – Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| Buffering period | The remainder of the bitstream |
| Picture timing | The AU containing the SEI message |
| DU information | The AU containing the SEI message |
| Scalable nesting | Depending on the scalable-nested SEI messages. Each scalable-nested SEI message has the same persistence scope as if the SEI message was not scalable-nested. |
| SEI manifest | The CVS containing the SEI message |
| SEI prefix indication | The CVS containing the SEI message |
| Subpicture level information | The CVS containing the SLI SEI message and up to but not including the next CVS, in decoding order, that contains an SLI SEI message with different content |
| Constrained RASL encoding indication | The CVS containing the SEI message |
| SEI processing order | Depending on the SEI messages indicated within the syntax. Each SEI message identified within the SEI processing order SEI message has the same persistence scope as if the SEI message was carried outside of the SEI processing order SEI message and not identified within an SEI processing order SEI message. For each value of po_id, the number of SEI messages and the type codes of the SEI messages indicated within the SEI processing order SEI message persist in decoding order from the first access unit in decoding order until the end of the CVS in output order. The CVS containing the SEI message |
| Processing order nesting | Depending on the processing-order-nested SEI messages. Each processing-order-nested SEI message has the same persistence scope as if the SEI message was not nested. |

...

# Add clause D.11 as follows:

## D.11 SEI processing order SEI message

D.11.1 SEI processing order SEI message syntax

| sei_processing_order( payloadSize ) { | Descriptor |
| --- | --- |
| po_id | u(8) |
| po_num_sei_messages_minus2 | u(8) |
| for( i = 0, i < po_num_sei_messages_minus2 + 2; i++) { |  |
| po_sei_wrapping_flag[ i ] | u(1) |
| po_sei_importance_flag[ i ] | u(1) |
| if( po_sei_wrapping_flag[ i ] ) { |  |
| reserved_alignment_6bits | u(6) |
| sei_message( ) |  |
| } else { |  |
| po_sei_prefix_flag[ i ] | u(1) |
| po_sei_payload_type[ i ] | u(13) |
| if( po_sei_prefix_flag[ i ]) { |  |
| po_num_prefix_bytes[ i ] | b(8) |
| for( j = 0; j < po_num_prefix_bytes[ i ]; j++ ) |  |
| po_prefix_byte[ i ][ j ] | b(8) |
| } |  |
| } |  |
| po_sei_processing_order[ i ] | u(8) |
| } |  |
| } |  |

### D.11.2 SEI processing order SEI message semantics

The SEI processing order SEI message carries information indicating the preferred processing order, as determined by the encoder (i.e., the content producer), for a group of SEI messages that may be present in a CVS.

NOTE 1 – In the semantics of this SEI message, two different types of SEI messages may have the same SEI payloadType value but are differentiated by some syntax elements in the SEI payload. For example, two neural-network post-filter characteristics (NNPFC) SEI messages with different nnpfc_id values are considered as having two different SEI message types.

When an SEI processing order SEI message with a particular value of po_id is present in any access unit of a CVS, an SEI processing order SEI message with a particular value of po_id shall be present in the first access unit of the CVS in decoding order. The number of SEI messages and the type codes of the SEI messages indicated within each SEI processing order SEI message with the same value of po_id persist in decoding order from the current access unit until the end of the CVS in output order.

It is a requirement of bitstream conformance that, within an SEI processing order SEI message, there shall be at least two pairs of the syntax elements po_sei_payload_type[ i ] and po_sei_processing_order[ i ], and there shall be at least two values of po_sei_processing_order[ i ] that are not equal.

The SEI processing order SEI message can carry one or more SEI prefix indications of a particular payloadType. Each SEI prefix indication is a byte bit string that follows the SEI payload syntax of that value of payloadType and contains a number of complete syntax elements starting from the first syntax element in the SEI payload, and may be followed by bits that do not represent any complete syntax element of the SEI payload.

[Ed per JVET-AD0386: Open issues -- Should the SEI prefix be required to consist of a number of complete syntax elements only? If yes, should the SEI prefix length be indicated in bytes or bits?]

These SEI prefix indications should provide sufficient information to determine the specific processing order for SEI messages having the same value of payloadType but different preferred processing order.

po_id contains an identifying number that may be used to identify an SEI processing order SEI message indicating a preferred processing order for a group of SEI messages.

A group of SEI messages is indicated by a set of SEI messages for which preferred processing order information is provided in an SEI processing order SEI message with each SEI message being identified by the syntax elements po_sei_payload_type[ i ], po_sei_wrapping_flag[ i ], po_sei_processing_order[ i ] and, when present, po_num_bits_in_prefix_indication_minus1[ i ] and po_prefix_data_bit[ i ][ j ].

For each picture, there may be multiple persisting or activated SEI messages belonging to one or more groups of SEI messages.

NOTE X – Groups of SEI messages could be alternative to each other, i.e., at most one group is chosen to be applied, or could be complementary, i.e., more than one group is chosen and applied separately, with each group generating one output.

po_num_sei_messages_minus2 plus 2 indicates the number of SEI message types for which the preferred order of processing is that have a processing order indicated in the SEI processing order SEI message.

po_sei_wrapping_flag[ i ] equal to 1 specifies that one or more processing order nesting SEI messages with all of the following constraints should be present:

- pon_target_po_id[ j ] with any value of j is equal to po_id.

- The payloadType of the nested SEI message is equal to po_sei_payload_type[ i ].

- pon_processing_order[ j ] with any value of j is equal to po_sei_processing_order[ i ].

If po_sei_wrapping_flag[ i ] is equal to 0, an SEI message with payloadType equal to po_sei_payload_type[ i ] should be present outside of the SEI processing order processing order nesting SEI message with payloadType equal to po_sei_payload_type[ i ]. However, if po_sei_wrapping_flag[ i ] is equal to 0 and no SEI message is present with payloadType equal to po_sei_payload_type[ i ], the following applies:

- If po_sei_importance_flag[ i ] is equal to 1, the decoder should ignore the entire SEI processing order SEI message.

- Otherwise, the decoder should ignore all data associated with the loop variable value of i.

NOTE – po_sei_wrapping_flag[ i ] equal to 1 enables SEI messages to be carried within the SEI processing order processing order nesting SEI message to prevent such SEI messages from being incorrectly interpreted by decoders that do not process the SEI processing order SEI message. Thus, po_sei_wrapping_flag[ i ] equal to 1 is intended to be used when po_sei_wrapping_flag[ i ] equal to 0 can lead to unintended results being produced by such decoders.

…

po_sei_processing_order[ i ] indicates the preferred order of processing of the i-th SEI message type for which preferred processing order information is provided in the SEI processing order SEI message. For any two different integer values of m and n, po_sei_processing_order[ m ] less than po_sei_processing_order[ n ] indicates that the SEI message indicated with index m should be processed before the SEI message indicated with index n, and po_sei_processing_order[ m ] equal to po_sei_processing_order[ n ] indicates that there is no preferred order of processing between the associated SEI messages (e.g. they may indicate different properties that are both applicable at that stage or alternative processes that can be applied or one may indicate a property and the other may indicate a process).

po_sei_processing_order[ 0 ] shall be equal to 0, and for i greater than 0, po_sei_processing_order[ i ] shall be equal to po_sei_processing_order[ i − 1 ] or po_sei_processing_order[ i − 1 ] + 1.

For i greater than 0, po_sei_processing_order[ i ] shall be greater than or equal to po_sei_processing_order[ i − 1 ].

The value of po_sei_processing_order[ po_num_sei_messages_minus2 + 1 ] shall not be equal to 0.

When more than one SEI processing order SEI message with a particular value of po_id is present in a CVS, the values of po_num_sei_messages_minus2 and, for each value of i, the values of po_sei_wrapping_flag[ i ], po_sei_prefix_flag[ i ], po_sei_importance_flag[ i ], po_sei_payload_type[ i ], po_sei_processing_order[ i ] shall be the same as in the other SEI processing order SEI messages in the CVS with the same value of po_id.

# Add clause D.12 as follows [Ed. highlighting not used, since the entire D.12 is proposed in this document]:

## D.12 Processing order nesting SEI message

D.12.1 Processing order nesting SEI message syntax

| processing_order_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| pon_num_po_ids_minus1 | u(8) |
| for( i = 0; i <= pon_num_po_ids_minus1; i++ ) |  |
| pon_target_po_id[ i ] | u(8) |
| pon_num_seis_minus1 | u(8) |
| for( i = 0; i <= pon_num_seis_minus1; i++ ) { |  |
| pon_processing_order[ i ] | u(8) |
| sei_message( ) |  |
| } |  |
| } |  |

D.12.2 Processing order nesting SEI message semantics

The processing order nesting SEI message includes one or more SEI messages that should be applied only as parts of the processing chain implied by an associated SEI processing order SEI message and should not be applied in a manner that would contradict with the processing chain implied by the associated SEI processing order SEI message.

The SEI messages contained in the processing order nesting SEI message are also referred to as the processing-order-nested SEI messages.

The persistence of all the SEI messages included in the same processing order nesting SEI message shall be the same.

pon_num_po_ids_minus1 plus 1 specifies the number of the SEI processing order SEI messages SEI associated with this processing order nesting SEI message. The value of pon_num_po_ids_minus1 shall be in the range of 0 to 255, inclusive.

pon_target_po_id[ i ] indicates the po_id of the i-th associated SEI processing order SEI message.

pon_num_seis_minus1 plus 1 specifies the number of the processing-order-nested SEI messages that are included in this SEI message. The value of pon_num_seis_minus1 shall be in the range of 0 to 255, inclusive.

pon_processing_order[ i ] specifies the position of the i-th processing-order-nested SEI message within the processing order defined by the associated SEI processing order SEI message. When i is greater than 0, pon_processing_order[ i ] shall be greater than or equal to pon_processing_order[ i − 1 ].

For each associated SEI processing order SEI message there shall be at least a value of i in the range of 0 to pon_num_seis_minus1, inclusive, in the processing order nesting SEI message for which the associated SEI processing order SEI message has an entry idx for which all of the following are true:

–	po_sei_processing_order[ idx ] is equal to pon_processing_order[ i ]

–	po_sei_payload_type[ idx ] is equal to the payloadType value of the i-th processing-order-nested SEI message.

–	When po_sei_prefix_flag[ idx ] is equal to 1, po_prefix_byte[ idx ][ j ] for j in the range of 0 to po_num_prefix_bytes[ idx ] − 1, inclusive, contain the same content as the po_num_prefix_bytes[ idx ] initial bytes of the SEI message payload of the i-th processing-order-nested SEI message.

The i-th processing-order-nested SEI message should be applied as the idx-th loop entry of the associated SEI processing order SEI message.

# Patent rights declaration(s)

Nokia, Dolby Laboratories, Fraunhofer HHI, Bytedance Inc., and MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).