| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0014-v2 |
| --- | --- |

| Title: | JVET AHG report: NNVC software development AhG14 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | F. Galpin, Y. Li, Y. Li, J. N. Shingala L. Wang, Z. Xie | Tel: Email: | franck.galpin@interdigital.com yue.li@bytedance.com yli30@qti.qualcomm.com jay.shingala@ittiam.com liqiangwang@tencent.com xiezhihuang@oppo.com |
| Source: | AHG14 on NNVC software development |  |  |

# Abstract

This report summarizes the activities of the AHG14 on NNVC software development that have taken place between the 32nd and 33rd JVET meetings.

# Introduction

The mandates for the AhG are as follows:

- Coordinate development of the NNVC software and associated configuration files.

- Prepare and deliver NNVC-7.0 and NNVC-7.1 software versions and the reference configuration encodings according to the NNVC common test conditions as described in JVET-AF2016.

- Investigate combinations of tools included in the NNVC software, prepare and release anchor data for all configurations of the software, including anchors for High and Low Operation Point (HOP/LOP) configurations.

- Study and maintain the SADL (Small Adhoc Deep-Learning Library). Identify gaps in functionality and develop improvements as needed.

- Coordinate with NNVC algorithm and software description (JVET-AF2019) editors to identify any mismatches between software and description document, suggest further updates to the description document as appropriate.

- Coordinate with AHG11 on items related to NNVC activities.

# Software development

## Location

NNVC repository is located at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM

NNVC software is based on VTM-11.0 with enabled MCTF including the update from JVET-V0056, GOP32, and enabling deblocking in the RDO.

NNVC-7.1 anchor at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM is used for NNVC performance evaluation.

## Software changes

Several commits were merged in the NNVC repository.

The following changes were integrated:

- HOP related

  - JVET-AF0155/AF0180: luma/chroma balance

  - JVET-AF0296: learning rate

  - JVET-AF0150: batch size

  - improved documentation

- LOP related:

  - JVET-AF0043: LOP.2 training and model

- LOP/HOP:

  - Training speed-up

  - Unification of code, training scripts and configuration files

  - JVET-AF0172: cleaning scale flag

  - JVET-AF0205: padding policy

  - JVET-AF0085: residual offset adjustment

  - JVET-AF0193: decoder complexity optimization (default off)

- NN intra lower complexity: JVET-AF0139

- JVET-AF0152: SADL v7

- fixes:

  - build issues fixes.

  - default to AVX2 for SADL

  - remove deprecated code

## Software version

NNVC-7.1 was tagged November 29th, 2023

NNVC-7.0 was tagged November 3rd, 2023

NNVC-6.1 was tagged September 25th, 2023 (fix)

NNVC-6.0 was tagged September 6th, 2023.

NNVC-5.1 was tagged July 19th, 2023.

NNVC-5.0 was tagged May 11th, 2023.

NNVC-3.0 (a.k.a VTM-11.0_nnvc3.0) was tagged December 1st 2022.

NCS-1.0 (a.k.a NNVC-3.0wip2) was tagged September 4th 2022 (first release containing the FilterSets, using NNVC 2.0 as a base).

VTM-11.0_nnvc-2.0 was tagged August 4th 2022 (add deblocking in RDO).

VTM-11.0_nnvc-1.0 was tagged May 6th 2021 (VTM-11.0 base with MCTF enabled).

# CTC performance

See configurations section for naming convention.

## Comparison to VTM

### NNVC-6.0 VTM vs NNVC-7.1 VTM

NNVC-7.1 in VTM mode performance are the same as the ones in NNVC-5.0/NNVC-6.0.

### NNVC-7.1 VTM vs NNVC-7.1 anchor

The NNVC-7.1 anchor includes LOP.2 filter and Intra Prediction tools activated.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-7.0 == 7.1 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -7.91% | -10.09% | -10.61% | -8.48% | -11.15% | -10.66% | 129% | 9031% |
| Class A2 | -6.79% | -12.68% | -9.74% | -7.00% | -11.83% | -7.35% | 126% | 8426% |
| Class B | -6.57% | -13.94% | -12.70% | -6.78% | -12.92% | -11.88% | 127% | 9076% |
| Class C | -6.58% | -14.87% | -14.43% | -7.15% | -13.24% | -12.86% | 120% | 7892% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -6.89% | -13.17% | -12.15% | -7.26% | -12.43% | -10.99% | 126% | 8606% |
| Class D | -7.17% | -14.84% | -15.00% | -6.49% | -13.17% | -13.07% | 119% | 7530% |
| Class F | -3.71% | -9.49% | -9.41% | -4.23% | -10.19% | -9.77% | 136% | 3684% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-7.0 == 7.1 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | -5.29% | -13.38% | -13.25% | -6.09% | -14.72% | -17.51% | 120% | 8941% |
| Class C | -5.53% | -13.74% | -14.88% | -7.36% | -15.51% | -16.98% | 113% | 7756% |
| Class E | -6.13% | -8.86% | -12.10% | -7.76% | -10.47% | -13.50% | 136% | 9072% |
| Overall | -5.58% | -12.37% | -13.51% | -6.93% | -13.92% | -16.33% | 121% | 8558% |
| Class D | -6.54% | -13.06% | -15.42% | -7.34% | -14.20% | -17.88% | 110% | 7563% |
| Class F | -3.14% | -8.73% | -8.67% | -4.24% | -9.89% | -13.21% | 129% | 4227% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-7.0 == 7.1 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -8.62% | -12.69% | -12.29% | -9.00% | -14.28% | -12.61% | 187% | 6908% |
| Class A2 | -7.15% | -12.70% | -11.26% | -7.47% | -12.25% | -9.23% | 186% | 5880% |
| Class B | -7.35% | -13.02% | -13.55% | -7.37% | -13.33% | -13.41% | 181% | 5701% |
| Class C | -7.53% | -13.81% | -14.47% | -7.93% | -14.27% | -14.56% | 168% | 4342% |
| Class E | -10.71% | -14.15% | -15.12% | -10.94% | -13.61% | -14.11% | 176% | 6176% |
| Overall | -8.13% | -13.28% | -13.42% | -8.38% | -13.56% | -12.95% | 179% | 5644% |
| Class D | -7.65% | -12.72% | -15.05% | -7.48% | -13.58% | -15.42% | 160% | 4159% |
| Class F | -5.27% | -10.79% | -10.39% | -5.08% | -11.26% | -11.78% | 141% | 4847% |

Note: Results from Tencent, crosschecked by InterDigital for gains.

### NNVC-7.1 VTM mode vs NNVC-7.1 HOP.2

The NNVC-7.1 where LOP.2 filter is replaced by HOP.2 and Intra Prediction tools activated.

|  | Random access Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-7.1 VTM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR |  | V-PSNR |  | Y-MSIM |  | U-MSIM |  | V-MSIM |  | EncT |  |  | DecT CPU |  |
| Class A1 | -15.29% | -8.16% |  | -18.63% |  | -16.62% |  | -14.19% |  | -19.81% |  | 444% |  |  | 212316% |  |
| Class A2 | -14.52% | -11.44% |  | -9.50% |  | -14.18% |  | -11.71% |  | -5.61% |  | 420% |  |  | 203481% |  |
| Class B | -12.30% | -12.31% |  | -12.15% |  | -11.43% |  | -12.31% |  | -11.21% |  | 437% |  |  | 214243% |  |
| Class C | -13.23% | -16.68% |  | -16.88% |  | -12.41% |  | -12.85% |  | -12.90% |  | 351% |  |  | 197379% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -13.59% | -12.47% |  | -14.18% |  | -13.28% |  | -12.71% |  | -12.26% |  | 410% |  |  | 207086% |  |
| Class D | -14.34% | -15.82% |  | -17.40% |  | -11.21% |  | -12.00% |  | -11.59% |  | 342% |  |  | 185469% |  |
| Class F | -8.28% | -11.67% |  | -10.63% |  | -8.37% |  | -11.91% |  | -10.79% |  | 647% |  |  | 97257% |  |
| Class H | #VALUE! | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #DIV/0! |  |  | #DIV/0! |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-7.1 VTM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR |  | V-PSNR |  | Y-MSIM |  | U-MSIM |  | V-MSIM |  | EncT |  |  | DecT CPU |  |
| Class A1 | #VALUE! | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #DIV/0! |  |  | #DIV/0! |  |
| Class A2 | #VALUE! | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #DIV/0! |  |  | #DIV/0! |  |
| Class B | -11.10% | -6.55% |  | -7.45% |  | -11.04% |  | -11.29% |  | -12.22% |  | 413% |  |  | 214332% |  |
| Class C | -12.28% | -13.55% |  | -14.46% |  | -12.60% |  | -13.84% |  | -13.48% |  | 341% |  |  | 197098% |  |
| Class E | -13.31% | -3.89% |  | -9.88% |  | -14.03% |  | -8.57% |  | -11.80% |  | 907% |  |  | 272167% |  |
| Overall | -12.05% | -8.22% |  | -10.40% |  | -12.31% |  | -11.46% |  | -12.53% |  | 472% |  |  | 221253% |  |
| Class D | -13.56% | -12.14% |  | -14.84% |  | -12.38% |  | -12.34% |  | -13.48% |  | 324% |  |  | 184900% |  |
| Class F | -8.83% | -9.53% |  | -10.94% |  | -10.01% |  | -12.18% |  | -15.42% |  | 630% |  |  | 116303% |  |
| Class H | #VALUE! | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #DIV/0! |  |  | #DIV/0! |  |
|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-7.1 VTM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR |  | V-PSNR |  | Y-MSIM |  | U-MSIM |  | V-MSIM |  | EncT |  |  | DecT CPU |  |
| Class A1 | -12.82% | -8.37% |  | -15.39% |  | -14.38% |  | -13.29% |  | -17.63% |  | 512% |  |  | 151639% |  |
| Class A2 | -12.35% | -10.32% |  | -9.22% |  | -13.17% |  | -9.89% |  | -6.44% |  | 352% |  |  | 121892% |  |
| Class B | -11.01% | -10.87% |  | -11.24% |  | -10.94% |  | -11.58% |  | -11.47% |  | 322% |  |  | 114567% |  |
| Class C | -12.13% | -12.55% |  | -15.09% |  | -11.86% |  | -12.10% |  | -14.14% |  | 245% |  |  | 81849% |  |
| Class E | -15.62% | -14.68% |  | -14.86% |  | -15.75% |  | -13.67% |  | -14.67% |  | 350% |  |  | 127857% |  |
| Overall | -12.55% | -11.37% |  | -13.05% |  | -12.89% |  | -12.05% |  | -12.78% |  | 337% |  |  | 114638% |  |
| Class D | -11.72% | -11.38% |  | -15.01% |  | -10.82% |  | -11.27% |  | -12.86% |  | 223% |  |  | 76334% |  |
| Class F | -8.85% | -11.06% |  | -10.75% |  | -8.54% |  | -11.48% |  | -11.55% |  | 212% |  |  | 99222% |  |

Note: Results from Tencent, crosschecked by Oppo.

### NNVC-7.1 VTM mode vs NNVC-7.1 VTM mode + LOP.2

The results reflect LOP only performance.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | LOP2 BD-rate Over NNVC-7.0 == 7. |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT |
| Class A1 | -5.92% | -8.80% | -9.06% |  |  |  | 121% | 12269% |
| Class A2 | -5.59% | -12.12% | -8.92% |  |  |  | 113% | 9036% |
| Class B | -4.99% | -13.19% | -12.07% |  |  |  | 115% | 8850% |
| Class C | -5.12% | -13.71% | -13.20% |  |  |  | 105% | 8032% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -5.33% | -12.24% | -11.14% |  |  |  | 113% | 9245% |
| Class D | -6.01% | -14.08% | -13.94% |  |  |  | 104% | 7464% |
| Class F |  |  |  |  |  |  |  |  |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | LOP BD-rate Over NNVC-7.0 == 7. |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT |
| Class A1 | -4.64% | -9.01% | -8.52% |  |  |  | 118% | 8037% |
| Class A2 | -4.48% | -10.12% | -8.62% |  |  |  | 108% | 6290% |
| Class B | -4.39% | -10.32% | -10.64% |  |  |  | 106% | 5134% |
| Class C | -4.64% | -11.11% | -11.70% |  |  |  | 103% | 3205% |
| Class E | -6.42% | -9.58% | -11.35% |  |  |  | 106% | 4491% |
| Overall | -4.84% | -10.12% | -10.31% |  |  |  | 108% | 5157% |
| Class D | -5.00% | -10.10% | -12.34% |  |  |  | 101% | 3109% |
| Class F |  |  |  |  |  |  |  |  |

Note: Results from Qualcomm, crosschecked by XX.

### NNVC-7.1 VTM mode vs NNVC-7.1 VTM mode + HOP.2

The results reflect HOP only performance.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-7.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -13.58% | -7.16% | -17.71% |  |  |  |  |  |
| Class A2 | -13.47% | -10.84% | -8.82% |  |  |  |  |  |
| Class B | -10.95% | -11.39% | -11.44% |  |  |  |  |  |
| Class C | -11.97% | -15.75% | -15.73% |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -12.25% | -11.60% | -13.32% |  |  |  |  |  |
| Class D | -13.29% | -15.18% | -16.83% |  |  |  |  |  |
| Class F | -7.44% | -10.89% | -9.97% |  |  |  |  |  |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-7.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -9.33% | -4.74% | -12.49% |  |  |  |  |  |
| Class A2 | -9.99% | -7.77% | -6.71% |  |  |  |  |  |
| Class B | -8.38% | -8.06% | -8.01% |  |  |  |  |  |
| Class C | -9.58% | -9.30% | -11.86% |  |  |  |  |  |
| Class E | -11.86% | -10.17% | -10.83% |  |  |  |  |  |
| Overall | -9.66% | -8.09% | -9.86% |  |  |  |  |  |
| Class D | -9.30% | -8.84% | -12.04% |  |  |  |  |  |
| Class F | -7.00% | -9.17% | -8.88% |  |  |  |  |  |

### NNVC-7.1 VTM mode vs NNVC-7.1 VTM + NN intra low complexity

The results reflect NN intra low complexity only performance.

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-7.1 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -3.99% | -3.52% | -3.91% | -4.20% | -3.52% | -3.80% | 165% | 429% |
| Class A2 | -2.81% | -2.37% | -2.88% | -2.81% | -2.15% | -2.53% | 171% | 573% |
| Class B | -2.92% | -2.67% | -2.83% | -2.74% | -2.43% | -2.43% | 172% | 619% |
| Class C | -2.40% | -2.08% | -2.07% | -2.53% | -1.61% | -1.52% | 163% | 596% |
| Class E | -4.14% | -3.91% | -3.51% | -3.99% | -3.45% | -2.21% | 169% | 633% |
| Overall | -3.17% | -2.84% | -2.96% | -3.16% | -2.55% | -2.44% | 168% | 572% |
| Class D | -2.06% | -1.69% | -1.90% | -2.07% | -1.38% | -1.01% | 158% | 627% |
| Class F | -1.74% | -1.41% | -1.35% | -1.81% | -1.26% | -1.18% | 143% | 353% |

Note: Results from InterDigital, crosschecked by Oppo.

## Comparison to previous NNVC - NNVC-6.0 anchor

### NNVC-6.0 anchor vs NNVC-7.1 anchor

The results reflect LOP progress between NNVC 6.0 (as anchor) and NNVC 7.1 (as test).

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC 6.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -0.73% | -5.39% | -4.78% | 0.30% | -7.16% | -5.81% |  |  |
| Class A2 | -0.25% | -7.14% | -4.80% | -0.51% | -7.88% | -4.53% |  |  |
| Class B | -0.29% | -6.01% | -5.47% | -0.85% | -7.19% | -7.04% |  |  |
| Class C | 0.05% | -4.53% | -5.12% | -0.94% | -5.56% | -7.49% |  |  |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.28% | -5.72% | -5.10% | -0.58% | -6.89% | -6.41% |  |  |
| Class D | 0.46% | -5.54% | -5.53% | -0.78% | -6.42% | -7.38% |  |  |
| Class F | -0.66% | -3.76% | -4.60% | -1.21% | -5.62% | -6.47% |  |  |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC 6.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class B | -0.47% | -5.48% | -5.76% | -0.99% | -8.84% | -11.46% |  |  |
| Class C | -0.31% | -2.82% | -6.31% | -1.83% | -5.38% | -13.65% |  |  |
| Class E | -1.29% | -4.85% | -7.30% | -2.27% | -8.32% | -10.72% |  |  |
| Overall | -0.62% | -4.44% | -6.33% | -1.59% | -7.56% | -12.00% |  |  |
| Class D | 0.02% | -2.38% | -5.89% | -1.65% | -5.19% | -11.89% |  |  |
| Class F | -1.11% | -3.84% | -4.37% | -2.13% | -6.69% | -9.37% |  |  |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC 6.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -0.18% | -5.86% | -3.93% | 1.23% | -7.29% | -5.52% |  |  |
| Class A2 | -0.57% | -5.04% | -3.57% | -0.62% | -6.25% | -4.05% |  |  |
| Class B | -0.35% | -4.77% | -5.23% | -0.37% | -6.21% | -6.83% |  |  |
| Class C | -0.21% | -4.39% | -4.86% | -0.35% | -6.55% | -7.42% |  |  |
| Class E | -0.47% | -5.44% | -6.80% | -0.86% | -6.31% | -8.28% |  |  |
| Overall | -0.35% | -5.02% | -4.91% | -0.22% | -6.49% | -6.52% |  |  |
| Class D | -0.08% | -4.81% | -6.03% | -0.32% | -7.18% | -8.29% |  |  |
| Class F | -0.75% | -5.00% | -4.88% | -0.80% | -6.78% | -7.49% |  |  |

### NNVC-6.0 HOP vs NNVC-7.1 HOP

The results reflect HOP progress between NNVC 6.0 (as anchor) and NNVC 7.1 (as test).

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 HOP |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -3.44% | 18.42% | 11.54% | -3.47% | 14.01% | 11.76% |  |  |
| Class A2 | -2.95% | 17.74% | 18.25% | -2.78% | 16.39% | 18.04% |  |  |
| Class B | -1.44% | 20.17% | 17.60% | -1.09% | 19.65% | 18.91% |  |  |
| Class C | -1.65% | 16.96% | 16.42% | -1.47% | 17.49% | 18.43% |  |  |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -2.20% | 18.48% | 16.20% | -2.01% | 17.29% | 17.18% |  |  |
| Class D | -1.55% | 16.41% | 15.29% | -1.01% | 17.69% | 17.88% |  |  |
| Class F | -1.79% | 9.86% | 9.32% | -1.13% | 11.55% | 11.23% |  |  |

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 HOP |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class C | -1.63% | 16.82% | 11.43% | -1.66% | 16.35% | 11.10% |  |  |
| Class E | -1.60% | 15.29% | -2.81% | -2.12% | 10.42% | -3.97% |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class D | -1.82% | 15.20% | 3.94% | -1.79% | 14.20% | 3.02% |  |  |
| Class F | -1.88% | 13.07% | 4.68% | -1.54% | 12.45% | 7.95% |  |  |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 HOP |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -2.25% | 17.82% | 14.03% | -2.24% | 16.11% | 13.06% |  |  |
| Class A2 | -3.14% | 16.68% | 14.74% | -3.18% | 17.17% | 14.67% |  |  |
| Class B | -1.49% | 15.42% | 14.91% | -1.40% | 17.06% | 17.16% |  |  |
| Class C | -1.66% | 12.43% | 12.91% | -1.54% | 15.24% | 16.42% |  |  |
| Class E | -1.53% | 13.98% | 15.33% | -1.20% | 14.27% | 17.38% |  |  |
| Overall | -1.93% | 15.12% | 14.36% | -1.83% | 16.05% | 15.93% |  |  |
| Class D | -1.57% | 12.83% | 13.43% | -1.43% | 17.15% | 18.42% |  |  |
| Class F | -1.85% | 8.35% | 8.89% | -1.51% | 10.59% | 12.97% |  |  |

## Comparison to NNVC-7.1 anchor

### NNVC-7.1 anchor vs NNVC-7.1 HOP.2

The results below reflect LOP/HOP performance differences (LOP set as anchor, HOP as test).

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-7.1 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -8.00% | 2.04% | -8.95% | -8.89% | -3.77% | -10.33% |  |  |
| Class A2 | -8.27% | 1.41% | 0.15% | -7.66% | -0.03% | 1.77% |  |  |
| Class B | -6.15% | 1.99% | 0.66% | -4.97% | 0.79% | 0.77% |  |  |
| Class C | -7.13% | -2.16% | -2.89% | -5.65% | 0.36% | -0.14% |  |  |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -7.21% | 0.78% | -2.31% | -6.47% | -0.40% | -1.49% |  |  |
| Class D | -7.74% | -1.17% | -2.98% | -5.02% | 1.38% | 1.39% |  |  |
| Class F | -4.83% | -2.53% | -1.47% | -4.39% | -2.25% | -1.24% |  |  |

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-7.1 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class C | -7.16% | 0.44% | 0.68% | -5.64% | 2.23% | 4.49% |  |  |
| Class E | -7.63% | 5.46% | 2.64% | -6.71% | 2.24% | 2.00% |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class D | -7.52% | 1.01% | 0.76% | -5.43% | 2.83% | 5.77% |  |  |
| Class F | -5.93% | -0.76% | -2.42% | -6.08% | -2.85% | -2.61% |  |  |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-7.1 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -4.59% | 4.88% | -3.46% | -5.89% | 1.08% | -5.79% |  |  |
| Class A2 | -5.62% | 2.85% | 2.26% | -6.12% | 2.73% | 3.07% |  |  |
| Class B | -3.98% | 2.77% | 2.73% | -3.83% | 2.20% | 2.29% |  |  |
| Class C | -5.02% | 1.56% | -0.72% | -4.26% | 2.48% | 0.37% |  |  |
| Class E | -5.52% | -0.68% | 0.30% | -5.29% | -0.11% | -0.70% |  |  |
| Overall | -4.84% | 2.29% | 0.45% | -4.89% | 1.78% | 0.15% |  |  |
| Class D | -4.42% | 1.62% | -0.04% | -3.63% | 2.86% | 2.88% |  |  |
| Class F | -3.87% | -0.22% | -0.36% | -3.63% | -0.29% | 0.42% |  |  |

### NNVC-7.1 anchor vs NNVC-7.1 NN intra lower complexity

The results below show the difference in performance of NN intra (as anchor) to NN intra lower complexity (as test).

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC 7.1 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | 0.31% | 0.31% | 0.72% | 0.31% | 0.27% | 0.37% | 72% | 90% |
| Class A2 | -0.03% | -0.05% | -0.21% | -0.06% | -0.25% | -0.30% | 71% | 86% |
| Class B | 0.25% | 0.04% | 0.12% | 0.26% | -0.04% | -0.07% | 70% | 82% |
| Class C | 0.70% | 0.61% | 0.85% | 0.63% | 0.65% | 0.73% | 70% | 75% |
| Class E | 0.60% | 0.63% | 0.41% | 0.77% | 0.82% | 0.42% | 70% | 81% |
| Overall | 0.37% | 0.30% | 0.38% | 0.38% | 0.27% | 0.23% | 71% | 82% |
| Class D | 0.83% | 0.64% | 0.87% | 0.76% | 0.29% | 1.23% | 71% | 71% |
| Class F | 0.41% | 0.33% | 0.49% | 0.34% | 0.29% | 0.37% | 81% | 83% |

## Other tools

Other results remain the same as tools were not changed.

# Discussions

## Discussions on optimization level in NNVC

### Overall optimization level

Current optimization level of NNVC is as follow:

- The legacy VTM code is considered less optimized than the ECM part as ECM integrated more SIMD optimization functions.

- The NN based code is mainly executed inside the SADL library, with some parts, non-SIMD optimized, executed on the NNVC side

The complexity at encoder, as measured by the encoding time, is split in half between the VTM part and the NN based part. It is considered that the NN based part is more optimized than the VTM part since almost all the code is SIMD optimized.

The complexity at decoder, as measured by the encoding time, is mainly driven by the NN based part.

### NN based optimization level

The SIMD optimization parts in SADL are not all optimized at the same level.

It was noted that recent switch to AVX2 by default is slowing down NNVC significantly (about 3 or 4 times slower) compared to AVX512. A better handling of some layers for AVX2 is already prepared and solved this issue.

Other SIMD improvement have been proposed and are currently under review.

Compared to a state-of-the-art CPU inference of the same models (using optimized numpy implementation), a slowdown of a factor 2 to 3 can be observed.

# Configurations

The following configurations is used to generate the different NNVC results.

The column “tested” is read as follow:

- Y: the configuration has been tested using the new NNVC-7.1 software

- P: the results are the ones from previous NNVC software basis

- N: not tested.

The column “xcheck” is read as follow:

- Y: the test has been cross-checked

- P: no cross-checked performed but results are consistent with previous version on NNVC

- N: no cross-check available

| Name | Tools | Configuration | Tested | Xcheck |
| --- | --- | --- | --- | --- |
| NNVC-7.1 VTM mode | none | encoder_xxx_vtm.cfg | P |  |
| NNVC 7.1 Anchor/EE1 | Intra Pred + LOP.2 filterset | encoder_xxx_nnvc.cfg | Y | Y |
| NNVC-7.1. HOP | Intra Pred + HOP.2 | encoder_xxx_nnvc.cfg + nn-based/HOP.cfg | Y | Y |
| HOP only | HOP | encoder_xxx_vtm.cfg + nn-based/HOP.cfg | Y | Y |
| Intra lower complexity | NN Intra lower complexity | nn-based/intra2.cfg | Y | Y |
| set0 | Loop filter set #0 | encoder_xxx_vtm.cfg + nn-based/NnlfOption_1.cfg | P |  |
| set1 | Loop filter set #1 | encoder_xxx_vtm.cfg + nn-based/NnlfOption_2.cfg | P |  |
| set0+rdo | Loop filter set #0 + Rdo | nn-based/NnlfOption_1.cfg + --EncNnlfOpt=1 | P |  |
| set1+rdo+intra+temporal filter | Loop filter set #1 + Temporal filter | nn-based/NnlfOption_2.cfg + --NnlfSet1Multiframe=1 | P |  |
| Sr | Super-resolution | nn-based/nnsr.cfg + nn-based/nnsr_classAx.cfg | P |  |
| Pf | Adaptive post-filters | nn-based/nnpf/nnpf_xxx.cfg | P |  |

# Recommendations

The AHG recommends to:

- Continue to develop NNVC software.

- Improve the software documentation.

- Encourage people to report all (potential) bugs that they are finding using GitLab Issues functionality https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/issues

- Encourage people to submit merge requests fixing identified bugs.