| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0024-v1 |
| --- | --- |

| Title: | EE2: Summary Report on Enhanced Compression beyond VVC capability |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Vadim Seregin Jie Chen Roman Chernyak Karam Naser Jacob Ström Fan Wang Martin Winken Xiaoyu Xiu Kai Zhang | Email: | vseregin@qti.qualcomm.com jiechen.cj@alibaba-inc.com chernyak@tencent.com Karam.Naser@InterDigital.com jacob.strom@ericsson.com wangfan6@oppo.com martin.winken@hhi.fraunhofer.de xiaoyuxiu@kwai.com zhangkai.video@bytedance.com |
| Source: | EE coordinators |  |  |

_____________________________

# Abstract

This document provides a summary report of Exploration Experiment on Enhanced Compression beyond VVC capability. The tests are categorized as intra prediction, inter prediction, transform and coefficient coding, in-loop filtering, and entropy coding.

The software basis for this EE is ECM-10.0, released at https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-11.0. ECM-11.0 is used as an anchor in the tests.

Software for EE tests is released in the corresponding branches at https://vcgit.hhi.fraunhofer.de/ecm/jvet-af-ee2/ECM/-/branches.

Test results can be found in input JVET contributions, cross-check results are uploaded to https://vcgit.hhi.fraunhofer.de/ecm/jvet-af-ee2/simulation-results if cross-check reports are not submitted as they are optional for EE tests.

# List of tests

|  | Tests | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1 Intra prediction |  |  |  |
| 1.1a | Decoder derived CCP mode | Y.-J. Chang (Qualcomm) JVET-AG0154 | H.-J. Jhu (Kwai) |
| 1.1b | Test 1.1a with decoder derived CCP fusion modes | Y.-J. Chang (Qualcomm) JVET-AG0154 | J. Lainema (Nokia) |
| 1.2 | IntraTMP with merge candidates | K. Naser (InterDigital) JVET-AG0151 | I.Zupancic (Nokia) JVET-AG0245 |
| 1.3 | SGPM with IntraTMP and IBC | K. Naser (InterDigital) JVET-AG0152 | I.Zupancic (Nokia) JVET-AG0245 |
| 1.4 | IntraTMP extension to DIMD | K. Naser (InterDigital) JVET-AG0146 | K. Cui (Qualcomm) JVET-AG0170 |
| 1.5 | IntraTMP extension to LIC | F. Le Léannec (InterDigital) JVET-AG0136 | K. Cui (Qualcomm) JVET-AG0171 |
| 1.6 | Test 1.4 + Test 1.5 | F. Le Léannec (InterDigital) JVET-AG0137 | M. Abdoli (Xiaomi) JVET-AG0255 |
| 1.7a | Test 1.6 + Test 1.3 | K. Naser (InterDigital) JVET-AG0137 | F. Wang, L. Zhang (OPPO) JVET-AG0274 |
| 1.7b | Test 1.7a + Test 1.2 | F. Le Léannec (InterDigital) JVET-AG0137 | F. Wang, L. Zhang (OPPO) JVET-AG0274 |
| 1.8a | The length of the auto-relocated BVP trace path is 1 (i.e, n=1) | N. Zhang (Bytedance) JVET-AG0091 | D. Ruiz Coll, (Ofinno) JVET-AG0134 Y. Kidani, (KDDI) JVET-AG0225 |
| 1.8b | The length of the auto-relocated BVP trace path is 2 (i.e, n=2) | N. Zhang (Bytedance) JVET-AG0091 | D. Ruiz Coll, (Ofinno) JVET-AG0134 Y. Kidani, (KDDI) JVET-AG0225 |
| 1.8c | No constraint for the length of the auto-relocated BVP trace path | N. Zhang (Bytedance) JVET-AG0091 | D. Ruiz Coll, (Ofinno) JVET-AG0134 Y. Kidani, (KDDI) JVET-AG0225 |
| 1.9 | Intra TMP fusion probing | J.-L. Lin (Qualcomm) JVET-AG0118 | Z. Deng (Bytedance) JVET-AG0236 |
| 1.10 | Bilateral filtering for intra prediction | W. Yin (Bytedance) JVET-AG0123 | X. Li (Alibaba) JVET-AG0248 |
| 1.11a | IntraTMP search area extension with sampling factor proportional to CU distance | D. Ruiz Coll (Ofinno) JVET-AG0131 | G. Verba (Qualcomm) JVET-AG0229 |
| 1.11b | IntraTMP search area extension with sampling factor proportional to search area dimension | K. Naser (InterDigital) JVET-AG0131 | G. Verba (Qualcomm) JVET-AG0229 |
| 1.11c | Test 1.11a + Test 1.11b | D. Ruiz Coll (Ofinno) K. Naser (InterDigital) JVET-AG0131 | G. Verba (Qualcomm) JVET-AG0229 |
| 1.12a | IBC with extended reference area up to the picture’s upper side boundary | Y. Kidani (KDDI) JVET-AG0072 | N. Zhang (Bytedance) JVET-AG0206 |
| 1.12b | IBC with four CTU rows instead of two CTU rows in HD resolution or less | Y. Kidani (KDDI) JVET-AG0072 | N. Zhang (Bytedance) JVET-AG0206 |
| 1.13a | Test 1.11c + Test 1.12a | Y. Kidani (KDDI) D. Ruiz Coll (Ofinno) K. Naser (InterDigital) JVET-AG0199 | N. Zhang (Bytedance) JVET-AG0207 |
| 1.13b | Test 1.11c + Test 1.12b | Y. Kidani (KDDI) D. Ruiz Coll (Ofinno) K. Naser (InterDigital) JVET-AG0199 | N. Zhang (Bytedance) JVET-AG0207 |
| 1.14a | Encoder run-time reduction methods for extrapolation filter-based intra prediction mode only | L. Xu (OPPO) JVET-AG0058 |  |
| 1.14b | Encoder run-time reduction methods for extrapolation filter-based intra prediction and its merge mode | L. Xu (OPPO) JVET-AG0058 |  |
| 1.15a | CCP merge fusion | H. Huang (OPPO) Z. Deng (Bytedance) JVET-AG0059 |  |
| 1.15b | Inheriting LB-CCP flag in CCP merge mode | H. Huang (OPPO) Z. Deng (Bytedance) JVET-AG0059 |  |
| 1.15c | Test 1.15a + Test 1.15b | H. Huang (OPPO) Z. Deng (Bytedance) JVET-AG0059 |  |
| 1.16 | Slope adjustment for IBC LIC | C. Ma (Kwai) JVET-AG0103 | Y. Wang (Bytedance) JVET-AG0221 |
| 1.17a | IBC GPM with block vector difference | C. Ma (Kwai) JVET-AG0104 | Y. Wang (Bytedance) JVET-AG0222 |
| 1.17b | IBC GPM with split mode reordering | C. Ma (Kwai) JVET-AG0104 | Y. Wang (Bytedance) JVET-AG0222 |
| 1.17c | Test 1.17a + Test 1.17b | C. Ma (Kwai) JVET-AG0104 | Y. Wang (Bytedance) JVET-AG0222 |
| 1.18a | DIMD mode derivation from spatial blocks | J. Huo, J. Fan (Xidian Univ.) M. Li (OPPO) JVET-AG0076 |  |
| 1.18b | DIMD mode derivation with reduced complexity | J. Huo, J. Fan (Xidian Univ.) M. Li (OPPO) JVET-AG0076 |  |
| 1.19 | IBC-LIC model merge mode | L. Zhang (OPPO) | Y. Wang (Bytedance) JVET-AG0220 |
| 1.20a | TIMD fusion with non-angular predictor | P. Andrivon (Ofinno) JVET-AG0092 | M. Abdoli (Xiaomi) |
| 1.20b | Test 1.20a + TIMD sample-based fusion | P. Andrivon (Ofinno) JVET-AG0092 | S. Blasi (Nokia) JVET-AG0132 |
| 1.21 | TIMD fusion reference line determination | C. Zhou (vivo) JVET-AG0128 | J. Chen (Alibaba) |
| 1.22a | Test 1.20a + Test 1.21 | P. Andrivon (Ofinno) C. Zhou (vivo) JVET-AG0093 | M. Abdoli (Xiaomi) |
| 1.22b | Test 1.20b + Test 1.21 | P. Andrivon (Ofinno) C. Zhou (vivo) JVET-AG0094 | S. Blasi (Nokia) JVET-AG0133 |
| 1.22c | Test 1.20b + Test 1.21 + Test 1.4 | P. Andrivon (Ofinno) C. Zhou (vivo) K. Naser (InterDigital) JVET-AG0095 | H. Qin (TCL) JVET-AG0127 |
| 2 Inter prediction |  |  |  |
| 2.1 | A LIC flag for inter prediction merge modes | Y. Zhang (Qualcomm) JVET-AG0176 | X. Xiu (Kwai) JVET-AG0270 |
| 2.2 | Enable PU level BDMVR and BDOF for bi-predicted LIC | Y. Zhang (Qualcomm) JVET-AG0176 | A. Robert (InterDigital) X. Xiu (Kwai) JVET-AG0270 |
| 2.3 | LIC with multiple templates | Y. Wang (Bytedance) X. Xiu (Kwai) JVET-AG0099 | L. Zhang (OPPO) JVET-AG0262 |
| 2.4 | LIC with slope adjustment | Y. Wang (Bytedance) JVET-AG0099 | P. Astola (Nokia) JVET-AG0214 |
| 2.5 | Non-local illumination compensation | X. Xiu (Kwai) JVET-AG0176 | Y. Zhang (Qualcomm) JVET-AG0286 |
| 2.6a | Test 2.1 + Test 2.2 | Y. Zhang (Qualcomm) JVET-AG0176 | X. Xiu (Kwai) JVET-AG0270 |
| 2.6b | Test 2.3 + Test 2.4 | Y. Wang (Bytedance) X. Xiu (Kwai) JVET-AG0099 | L. Zhang (OPPO) JVET-AG0262 |
| 2.6c | Test 2.3 + Test2.5 | X. Xiu (Kwai) Y. Wang (Bytedance) | withdrawn |
| 2.6d | Test 2.1 + Test2.2 + Test 2.3 + Test 2.4 + Test 2.5 | Y. Zhang (Qualcomm) | withdrawn |
| 2.6g | Test 2.6e + Test 2.3 | Y. Wang (Bytedance) Y. Zhang (Qualcomm) X. Xiu (Kwai) JVET-AG0276 |  |
| 2.6h | Test 2.6e + Test 2.4 | Y. Wang (Bytedance) Y. Zhang (Qualcomm) X. Xiu (Kwai) JVET-AG0276 |  |
| 2.6i | Test 2.6f + Test 2.3 | Y. Wang (Bytedance) Y. Zhang (Qualcomm) X. Xiu (Kwai) JVET-AG0276 |  |
| 2.6j | Test 2.6f + Test 2.4 | Y. Wang (Bytedance) Y. Zhang (Qualcomm) X. Xiu (Kwai) JVET-AG0276 |  |
| 2.7 | AMVP with SbTMVP mode | R.-L. Liao (Alibaba) JVET-AG0098 | L. Zhao (Bytedance) JVET-AG0234 F. Pu (Dolby) JVET-AG0288 |
| 2.8a | DMVR for non-equal POC distance cases | M. Salehifar (Bytedance) JVET-AG0067 | P. Le Guyadec (InterDigital) |
| 2.8 | DMVR for non-equal POC distance cases with constraint | M. Salehifar (Bytedance) | withdrawn |
| 2.8b | BDOF layer with new subblock sizes including 16×16 | M. Salehifar (Bytedance) JVET-AG0067 |  |
| 2.8c | Mean removed formula for BDOF | M. Salehifar (Bytedance) JVET-AG0067 |  |
| 2.8d | Test 2.8a + Test 2.8b + Test 2.8c | M. Salehifar (Bytedance) JVET-AG0067 | Kwai |
| 2.9 | CIIP with subblock-based motion compensation | L. Zhao (Bytedance) JVET-AG0135 | W.Chen (Kwai) JVET-AG0271 |
| 2.10 | GPM with affine prediction | K. Zhang (Bytedance) JVET-AG0164 | P.Bordes (InterDigital) JVET-AG0247 |
| 2.11a | Regression-based GPM blending | P. Bordes (InterDigital) JVET-AG0112 | L. Zhao (Bytedance) JVET-AG0285 |
| 2.11b | Test 2.11a without modified index coding | P. Bordes (InterDigital) JVET-AG0112 |  |
| 2.11c | Test 2.11a without modified blending | P. Bordes (InterDigital) JVET-AG0112 |  |
| 2.11d | Test 2.10 + Test 2.11a | P. Bordes (InterDigital) K. Zhang (Bytedance) JVET-AG0142 | M. Blestel (Ofinno) JVET-AG0223 |
| 2.11e | Test 2.9 + Test 2.10 | L. Zhao (Bytedance) P. Bordes (InterDigital) JVET-AG0142 | Kwai |
| 2.11f | Test 2.9 + Test 2.10 + Test 2.11a | L. Zhao (Bytedance) P. Bordes (InterDigital) JVET-AG0142 | Kwai |
| 2.12 | Adjusting out-of-boundary prediction samples | P. Astola (Nokia) JVET-AG0097 | Y. Wang (Bytedance) JVET-AG0205 |
| 3 Transform and coefficients coding |  |  |  |
| 3.1a | Intra contexts for intra CUs in inter slices | F. Lo Bianco (InterDigital) JVET-AG0143 |  |
| 3.1b | Test 3.1a + inter contexts for IntraTMP/IBC CUs | F. Lo Bianco (InterDigital) JVET-AG0143 |  |
| 3.1c | Test 3.1b without context switch for CBF | F. Lo Bianco (InterDigital) JVET-AG0143 |  |
| 3.1d | Test 3.1 with retrained context initialization of the affected by the test contexts | F. Lo Bianco (InterDigital) JVET-AG0143 | P. Nikitin (Qualcomm) JVET-AG0147 |
| 3.2a | Transform coefficient coding | P. Nikitin (Qualcomm) JVET-AG0100 | F. Lo Bianco, F. Galpin (Interdigital) JVET-AG0159 |
| 3.2b | Test 3.2a with retrained context initialization | P. Nikitin (Qualcomm) JVET-AG0100 | F. Lo Bianco, F. Galpin (Interdigital) JVET-AG0159 |
| 3.3 | Utilizing LFNST/NSPT for inter coding | F. Wang (OPPO) JVET-AG0061 | S. Puri (InterDigital) JVET-AG0259 M. Koo (LGE) JVET-AG0211 |
| 4 In-loop filtering |  |  |  |
| 4.1a | Adaptive clipping with signalled min/max values from MCTF prefiltered picture | K. Cui (Qualcomm) JVET-AG0145 | F. Le Léannec (InterDigital) JVET-AG0246 |
| 4.1b | Adaptive clipping with signalled min/max values from original picture | K. Cui (Qualcomm) JVET-AG0145 | F. Le Léannec (InterDigital) JVET-AG0246 |
| 4.2a | Fixed filter for chroma ALF | N. Hu (Qualcomm) JVET-AG0157 | C.-W. Kuo (Kwai) JVET-AG0272 |
| 4.2b | Fixed filter for chroma ALF reusing luma filter | N. Hu (Qualcomm) JVET-AG0157 | C.-W. Kuo (Kwai) JVET-AG0272 |
| 4.3 | Coefficient precision adjustment for ALF | W. Yin (Bytedance) N. Hu (Qualcomm) JVET-AG0158 | N. Song (OPPO) JVET-AG0279 |
| 5 Entropy coding |  |  |  |
| 5.1 | Retrain CABAC initialization parameters for all slices | F. Galpin (InterDigital) V. Seregin (Qualcomm) JVET-AG0196 | D. Bugdayci Sansli, J. Lainema (Nokia) JVET-AG0210 |
| 5.2 | Window slice type-dependent offsets initialization | V. Seregin (Qualcomm) F. Galpin (InterDigital) JVET-AG0196 | D. Bugdayci Sansli, J. Lainema (Nokia) JVET-AG0210 |
| 5.3a | Spatial CABAC tuning | J. Lainema (Nokia) JVET-AG0117 | P. Nikitin (Qualcomm) JVET-AG0124 K. Andersson (Ericsson) JVET-AG0119 |
| 5.3b | Spatial CABAC tuning in combination with retrained context initialization for inter slices | J. Lainema (Nokia) JVET-AG0117 | P. Nikitin (Qualcomm) JVET-AG0124 K. Andersson (Ericsson) JVET-AG0119 |

# Description of tests

## Intra prediction

### Test 1.1: Decoder derived CCP mode (JVET-AG0154)

In this method, a candidate list of cross-component prediction (CCP) modes is constructed, and to select the best candidate from the list a template cost is calculated to compare the reconstructed samples and the prediction values generated by the evaluated CCP mode. The template is shown in the next figure.

The template adjacent to the current chroma CU

The CCP mode list is constructed from the already existed in ECM modes by single model CCLM, single model CCCM, multi-model CCCM, single model GLCCCM, single model CCCM applied with LBCCP, and multi-model CCCM applied with LBCCP.

In the second aspect of the method, various decoder-derived CCP fusion candidates are added. A fusion candidate is the combination of two CCP modes selected from the existing CCP mode lists reordered by template costs.

Mode flag and a fusion flag are signalled to indicate the mode usage.

Test 1.1a: Decoder derived CCP mode

Test 1.1b: Test 1.1a with decoder derived CCP fusion mode

### Test 1.2: IntraTMP with merge candidates (JVET-AG0151)

In the test, new candidates, consisting of block vectors from neighbouring PUs coded in IBC or IntraTMP mode, are added into IntraTMP search process. There are up to 25 local and non-local PUs that may be checked during the selection of the neighbouring candidates, those candidates are evaluated together with other candidates, and 30 best candidates (same as in ECM) are selected for further refinement.

The neighbouring candidates may come from outside of IntraTMP search area and such outside candidates are prioritized.

### Test 1.3: SGPM with IntraTMP and IBC (JVET-AG0152)

In the test, spatial GPM (SGPM) mode is extended to additionally consider BVs from IBC and IntraTMP to form a predictor. In the extended SGPM, the predictor may be IBC, IntraTMP or regular intra prediction.

BV are selected as follows:

- Obtain block vectors of all merge candidates

- Select the best block vector according to SATD template cost

- Test up to 6 best block vectors (if available) inside SGPM candidate list construction.

Identifiers intra_pred_0 and intra_pred_1 correspond to be regular or BV based prediction.

### Test 1.4: IntraTMP extension to DIMD (JVET-AG0146)

The method consists in adaptively selecting between planar or block vector based prediction obtained from IntraTMP or IBC mode of neighboring blocks for the blending with an angular intra prediction in DIMD (up to five predictors and the planar mode predictor with the weights derived from the histogram of gradients can be used). The selection is based on the SAD template cost, where the planar mode is compared to all block vector based prediction. The mode with the minimum cost is selected.

### Test 1.5: IntraTMP extension to LIC (JVET-AG0136)

In the test, LIC usage is extended to IntraTMP mode. A CU-level flag is signalled to indicate the use of LIC. For screen content coding, the IntraTMP LIC mode is extended to support top-only and left-only templates to compute LIC model parameters, while for camera-captured coding, only the top-left template is employed. MMLM is also supported for IntraTMP LIC mode in screen content coding, similar to IBC LIC.

When LIC is used for a given CU, the IntraTMP search process employs MRSAD rather than SAD distortion function to search for block vector.

A restricted version of IntraTMP LIC is also tested. The restriction consists in a reduced IntraTMP search process for both LIC and non-LIC IntraTMP coding units. That is, a SAD-based search is performed for non-LIC blocks and a MRSAD-based search is performed for IntraTMP CUs coded in LIC mode.

To limit the amount of BV search process, the sampling of the IntraTMP search area is increased from 3 to 4.

### Tests 1.6-1.7: Combination of IntraTMP related tests (JVET-AG0137)

Test 1.6: Test 1.4 + Test 1.5 (restricted)

Test 1.7a: Test 1.3 + Test 1.4 + Test 1.5 (restricted)

Test 1.7b: Test 1.2 + Test 1.3 + Test 1.4 + Test 1.5 (restricted)

Test 1.7c: Test 1.2 + Test 1.3 + Test 1.4

### Test 1.8: Auto-relocated block vector prediction (JVET-AF0079)

In the test, auto-relocated BV is introduced to IBC merge and AMVP candidate list construction. As shown in the next figure, a guiding block vector BV0,1 associated with the current block B0 points to a reference block B1. If B1 has a BV denoted as BV1,2 pointing to a reference block B2, then BV0,2, given by BV0,2 = BV0,1 +BV1,2, is defined as the auto-relocated BVP, guided by BV0,1. Similarly, BV0,n+1 can be derived by

BV0,n+1 =BV0,n+BVn,n+1 = BV0,1+BV1,2 +…+BVn-1,n +BVn,n+1.

When deriving BVn,n+1 guided by BV0,n, all five positions shown in the next figure including top-left, top-right, center, bottom-left, and bottom-right positions of Bn are checked to find BVn,n+1.

Auto-relocated BV candidates are inserted after the HBVP candidates. The IBC merge and AMVP candidate list sizes are kept unchanged.

Three tests are conducted varying the length of the trace path, n=1, n=2, and unrestricted.

Test 1.8a: Auto-relocated BV with trace path 1.

Test 1.8b: Auto-relocated BV with trace path 2.

Test 1.8c: Auto-relocated BV with unrestricted trace path.

### Test 1.9: IntraTMP fusion probing (JVET-AG0118)

In ECM, IntraTMP candidate list is constructed from candidates derived by L-shape, left or above template based on the template matching cost which is calculated as the SAD between the template of the reference block and the template of current block. For IntraTMP fusion mode, multiple IntraTMP candidates are linearly combined according to the fusion weights derived from the template matching costs or derived by the MSE minimization method. A flag intra_tmp_fusion_weight_type is signalled to indicate which weight derivation method is used and an index intra_tmp_fusion_idx is signalled to indicate which candidate set is selected for IntraTMP fusion.

In the tested method, a fusion candidate with the minimum probing cost is selected from a fusion candidate list without signalling the intra_tmp_fusion_weight_type and intra_tmp_fusion_idx. The probing cost is derived as the SAD between the samples in the probing line of the fused template and the current block’s template. The fusion weights derivation is the same as in ECM-11.0, but the samples in probing line are excluded.

A flag is signalled to indicate the IntraTMP fusion probing mode.

### Test 1.10: Bilateral filtering for intra prediction (JVET-AG0123)

In this test, the generated intra prediction samples are further filtered by bilateral filter to reduce the noise level. The filter is similar to BIF used in the loop-filtering stage. The filter length is kept as 2.

### Test 1.11: IntraTMP search range extension with adaptive sampling (JVET-AG0131)

In ECM, the IBC search region comprises the reconstructed samples of a fixed number of CTUs based on the CTU dimension. This region encompasses all left neighbor CTUs within the same row as the current CTU and extends to two CTU rows above the current CTU.

In the test, IntraTMP search region is extended to align with the one of IBC. To cover the whole IBC reference region, the number of IntraTMP regions is increased by one compared to ECM-11, and exclusively, the boundaries of the farthest original regions (R1, R2, and R3) have been modified to cover the whole IBC search region, as it is depicted in next figure.

The ‘regular search area’ maintains the exact dimensions as in ECM-11 constrained by the SearchRangeWidth and SearchRangeHeight parameters, and the same sample rate of 3.

The searching process in the further away regions (R1 and R7) is subject to pruning if the normalized TM cost of the last BV candidate in the sparse list is less than a threshold, which depends on the current block's horizontal and vertical size (W, H). Furthermore, the search regions R1 and R6 are scanned in the inverse direction, from left to right, checking before the most probable candidates, and the scanning pattern is shifted by one pixel per row. Lastly, for block sizes of 64 samples, the factor parameter ‘a’ is set to 4, preventing the regular search region from exceeding the IBC search region.

Two methods are tested adapting the sampling rate beyond the regular search area to reduce the complexity of the search extension.

In Test 1.11a, sampling rate is adjusted based on stepwise function stored as lookup table based on the block distance ranging from 3 within the regular search region (a*W, a*H) to a maximum sampling factor of 80.

In Test 1.11b, sampling rate is based on IntraTMP regions dimension, where a new subsampling factor is determined for each region (Ri) according to the region dimensions as follows:

SamplingFactorWidth(Ri) = Width(Ri)/ F*BlkW

SamplingFactorHeight(Ri) = Height(Ri)/ F*BlkH

where F is the constant that controls the gain/complexity trade-off. The new sampling rate factors are only applied at the extended region locations.

Test 1.11c is a combination of Test 1.11a and Test 1.11b:

- The multifactor parameter 'a' for block sizes of 8 and 4 pixels is unified to 5.

- The sampling factor rate for both methods is calculated as described in Test 1.11b.

- The most significant sampling factor is selected for the current location:

SamplingFactor= max(SamplingFactor1,SamplingFactor2)

### Test 1.12: IBC with upward-extended reference area (JVET-AG0072)

In the ECM, the current IBC reference area is shown in the next figure.

In Test 1.12a, IBC reference area is extended to the whole causal area of a picture (to the top picture boundary).

In Test 1.12b, IBC search area is increased from 2 to 4 CTU rows above the current CTU for HD resolution or less.

### Test 1.13: Combination of IBC and IntraTMP search area related tests (JVET-AG0199)

Test 1.13a: Test 1.11c + Test 1.12a.

Test 1.13b: Test 1.11c + Test 1.12b.

### Test 1.14: Extrapolation filter-based intra prediction mode (JVET-AG0058)

The extrapolation filter-based intra prediction mode consists of three steps:

- The extrapolation filter coefficients are derived from a neighbouring reconstructed area of the current block or filter shape and coefficients are inherited from a previous block coded with this mode. For the latter, a mode flag is signalled, and candidate list is constructed using spatial and non-adjacent blocks coded with this mode, initial list of 12 candidates is reduced to 6 after reordering based on SAD cost measured on the L-shape template of size 1. An index is further signalled to identify a candidate from the list.

- The extrapolation process generates predicted signals from the top-left to bottom-right corner within the current block.

- An intra prediction angle using DIMD process is derived by analyzing the gradient of the predicted block, and then the corresponding intra mode is used to select MTS, NSPT, and LFNST kernels for transform.

The mode is restricted to blocks with sizes not greater than 32x32 and luma component only.

Three filer shapes are used in this method, a choice of reconstructed area and filter shape shown in the next figure is signalled.

The selected filter shape moves in the selected reconstructed area with a one-sample step either horizontally or vertically to collect input samples and output samples, then the filter coefficients are derived using CCCM solver.

The merge mode is also introduced, where the filter shape and the filter coefficients are inherited from the previous decoded blocks that are coded with the tested extrapolation filter-based intra prediction mode or the merge mode based on this mode. In the merge mode, the positions and inclusion order of the spatial adjacent, temporal, non-adjacent, shifted temporal, and history candidates are the same as those defined in ECM-11.0 for the CCP merge prediction candidates. In addition, the merge candidates are reordered by comparing the SAD cost on an L-shape template with column width and row height equal to 1. In the SAD calculation, predictions of the template area by the mode filters are generated only from reconstructed (neighbouring and template) samples, allowing the filters to be applied in parallel rather than sequentially.

In the current block, the recursive predictor is derived from the top-left to the bottom-right position by a diagonal prediction order as shown in the next figure, where the predicted results of the previous diagonal are used.

The predicted samples are calculated as follows,

predx,y=i=014(ci×tx-offsetXi, y-offsetYi)

where predx,y is the predicted value at (x, y) in the current block, ci is the ith coefficient of the selected filter, the index of the coefficients is from 0 to 14, tx-offsetXi, y-offsetYiis a reconstructed or a predicted value used for the current position’s prediction, offsetXi and offsetYi are the position offsets to the current position along x and y directions, respectively.

At encoder side, SATD is used to compare the costs for the added modes to update the list for full RDO, the number of total RDs is not increased. Additionally, the encoder will conditionally reduce one mode with the worst SATD cost in the full RDO cost when extrapolation-based prediction mode is already included in the full RDO list.

Test 1.14a: Encoder run-time reduction methods for extrapolation filter-based intra prediction mode only

Test 1.14b: Encoder run-time reduction methods for extrapolation filter-based intra prediction and its merge mode

### Test 1.15: Enhancements on CCP merge for chroma intra coding (JVET-AG0059)

In the CCP merge mode of ECM, a candidate list is created, which includes CCP models collected from the previous blocks coded by CCLM, MMLM, CCCM, GLM, chroma fusion, and CCP merge modes. Then one candidate in the list is selected as the CCP model for the current block.

In the test, two aspects are evaluated.

Aspect #1: CCP-merge fusion method is introduced where a fused prediction is generated by a weighted sum of the CCP-merge prediction and either the MM-CCCM prediction or the DIMD prediction. A CCP-merge fusion flag is signalled conditionally under the CCP-merge flag, to indicate whether the fusion mode is applied. A CCP-merge fusion type flag is further signalled if the CCP-merge fusion flag is true, to indicate whether the MM-CCCM prediction or the DIMD prediction is selected and fused with the CCP-merge prediction.

Aspect #2: The local-boosting CCP mode flag is inherited from a CCP candidate in the CCP merge candidate list.

Test 1.15a: CCP merge fusion

Test 1.15b: Inheriting LB-CCP flag in CCP merge mode

Test 1.15c: Test 1.15a + Test 1.15b

### Test 1.16: IBC LIC with slope adjustment (JVET-AG0103)

In this test, the slope adjustment of IBC LIC is introduced, where an offset parameter chosen from the set {-1/2, -1/4, -1/8, -1/16, 1/16, 1/8, 1/4, 1/2} is signalled to update the slope parameter of IBC LIC. The proposed method is only applied in IBC AMVP mode and luma component. In addition, the slope adjustment is not applied to the CUs that apply multi-mode IBC LIC.

### Test 1.17:  IBC GPM with block vector difference and split mode reordering (JVET-AG0104)

In Test 1.17a, IBC GPM with block vector difference is introduced, where one additional BVD, which is selected from the set {1/4-pel, 1/2-pel, 1-pel, 2-pel, 4-pel, 8-pel, 16-pel, 32-pel}, is allowed to be added on top of the BV of one IBC GPM partition which is based on regular IBC merge. A flag is signalled for each IBC partition to indicate whether additional BVD is applied. When the flag is true, the corresponding BVD index is further signalled.

In Test 1.17b, the split modes of IBC GPM with template matching are reordered, the reordering is only applied to the first set of IBC GPM split modes.

Test 1.17c: Test 1.17a + Test 1.17b.

### Test 1.18: DIMD mode derivation from spatial blocks (JVET-AG0076)

In this test, DIMD mode is derived by spatial blocks including non-adjacent spatial candidates. When neighbouring blocks are coded with DIMD mode, the DIMD histograms are combined to form a new DIMD merge histogram for the current block. DIMD merge modes and weights are computed based on this merged histogram.

As shown in the below figure, the distances between non-adjacent candidates and current block are defined based on the width and height of current coding block. When using DIMD merge, the DIMD information extracted from neighbouring blocks and non-adjacent spatial blocks is used to compute the intra prediction for the current block.

In Test 1.18a, when more than one neighbouring block encoded with DIMD or DIMD merge are available, all the spatial blocks are used to generate the DIMD merge mode.

In Test 1.18b, when more than one neighbouring block encoded with DIMD or DIMD merge are available, only the non-adjacent spatial candidates are used to derive the DIMD merge mode.

### Test 1.19: IBC-LIC model merge mode (JVET-AG0060)

In the test, IBC-LIC model parameters are inherited from spatial adjacent and non-adjacent neighbours, history candidates, or default models as follows.

- Construct a model candidate list which consists of model parameters from spatial adjacent and non-adjacent neighbors, history candidates, and default models. The size of the candidate list is 12.

If the list is not full, a default model and the scaled models are then added to the list. To avoid redundant models, a pruning operation is also applied.

- Select an IBC-LIC model from the candidate list and signal its index.

The IBC-LIC model merge mode can be applied to both IBC-merge mode and IBC-AMVP mode. In IBC-merge mode, the selected LIC model is applied to the reference template during merge list reordering. Then the IBC merge list is further sorted by the inherited IBC LIC flags. In the IBC-AMVP mode, the selected LIC model is applied to the reference template during BVD prediction.

A flag is signalled to indicate whether the IBC-LIC merge mode is applied or not. If this flag is true, an index is further signaled to indicate which candidate model is used by the current block.

### Test 1.20: TIMD fusion with non-angular predictor (JVET-AG0092)

In TIMD of ECM, two intra modes providing the smallest template cost are selected from the list of candidates and blended in a fusion process according to their respective template costs.

In Test 1.20a, a third non-angular intra prediction mode (among DC and planar) with the smallest SATD cost is added in TIMD fusion process, the fusion weights are computed based on the SATD costs.

In Test 1.20b, additionally location-dependent sample-based blending of the DIMD fusion process is re-used but the location-dependent criterion applying to amplitudes of the selected predictors is replaced by a SATD cost-based criteria. The location-dependent criterion is determined from a ratio of the normalized SATD of the selected TIMD predictors computed in ABOVE and LEFT templates area.

### Test 1.21: TIMD fusion reference line determination (JVET-AG0128)

For TIMD mode with blending in ECM-11.0, the first mode uses reference line l. While for the second mode, whether to use reference line l or l+1 depends on the following conditions:

- If all of the following conditions are true, l+1 is chosen.

- the current block is not ISP block

- the second mode is angular prediction mode

- the second mode does not represent non-fractional angles

- Otherwise, l is chosen.

In this test, for the second mode, the modified (underscored) conditions of reference line determination are as follows:

- If all of the following conditions are true, l+1 is chosen.

- the current block is not ISP block

- both the first mode and second mode are angular prediction mode

- all of the following conditions are false

- abs(predModeIntra1 – predModeIntra2) is greater than Threshold. The value of Threshold is set to 8 or 4.

- (predModeIntra1 - EXT_HOR_IDX) * (predModeIntra2 - EXT_HOR_IDX) is less than 0.

- (predModeIntra1 - EXT_VER_IDX) * (predModeIntra2 - EXT_VER_IDX) is less than 0.

- Otherwise, l is chosen.

### Test 1.22: Combination test of TIMD related tests

Test 1.22a: Test 1.20a + Test 1.21 (JVET-AG0093)

Test 1.22b: Test 1.20b + Test 1.21 (JVET-AG0094)

Test 1.22c: Test 1.20b + Test 1.21 + Test 1.4 (JVET-AG0095)

- TIMD/DIMD intra prediction modes may select IntraTMP/IBC BV (Test 1.22b + Test 1.4)

- a non-angular intra prediction mode (among DC, Planar or the best IntraTMP/IBC BV for TIMD/DIMD modes) is conditionally introduced (Test 1.20a + Test 1.4)

- location sample-based blending for TIMD is used similarly to DIMD (Test 1.20b and Test 1.22b)

- reference lines for TIMD are adjusted (Test 1.21 and Test 1.22b)

## Inter prediction

### Tests 2.1, 2.2, 2.5, 2.6a, 2.6e, and 2.6f on LIC improvement (JVET-AG0176)

In the tests, 3 aspects are introduced:

- To signal LIC flag for inter-prediction merge modes, instead of inheriting the flag value from a merge candidate. The flag is signalled to indicate if the original inherited LIC flag or the reverse LIC flag value is used for a merge candidate. The flag is signalled for regular, affine, and TM merge modes.

- To enable LIC with PU level BDMVR and BDOF.

- Non-local illumination compensation, where the linear model is derived from the previously coded inter CUs by minimizing the difference between their reconstruction and prediction samples. When constructing the merge lists, up to 16 and 6 non-local candidates (obtained from both spatial adjacent and non-adjacent positions) are inserted to the lists of regular and subblock merge respectively and reordered with the existing merge candidates. The lengths of the output merge lists are kept unchanged. The same pattern used for non-adjacent merge mode is reused to locate the non-adjacent positions in the scheme.

Test 2.1: LIC flag signalling for inter prediction merge modes.

Test 2.2: Enable PU level BDMVR and BDOF for bi-predicted LIC.

Test 2.5: Non-local illumination compensation.

Test 2.6a: Test 2.1 + Test 2.2

Test 2.6e: Test 2.2 + Test 2.5

Test 2.6f: Test 2.1 + Test 2.2 + Test 2.5

### Tests 2.3, 2.4, 2.6b LIC with multiple templates and slope adjustment (JVET-AG0099)

In Test 2.3, LIC with multiple templates is introduced, where besides using a template comprising neighbouring samples both top to and left to the current block, two new LIC modes using top-only or left-only template are added to derive the parameters of LIC. LIC with multiple templates is only applied to AMVP uni-predicted blocks.

In Test 2.4, the slope adjustment like in CCLM is introduced to LIC, in which an adjustment parameter is used to modify parameters of LIC. The adjustment parameter is signalled for AMVP mode. LIC with slope adjustment is not applied to bi-prediction.

Test 2.6b: Test 2.2 + Test 2.3

### Tests 2.6g, 2.6h, 2.6i, 2.6j combination of tests on LIC improvement (JVET-AG0276)

Test 2.6g: Test 2.6e + Test 2.3

Test 2.6h: Test 2.6e + Test 2.4

Test 2.6i: Test 2.6f + Test 2.3

Test 2.6j: Test 2.6f + Test 2.4

### Test 2.7: AMVP with SbTMVP mode (JVET-AG0098)

In AMVP with SbTMVP mode, a CU is predicted in a similar way to SbTMVP in merge mode (CU is split into 4x4 subblocks, and each subblock derives its own motion from a corresponding subblock in the collocated picture) except that the motion shift is signalled in the bitstream instead of being derived from neighbouring blocks. The motion shift is obtained using MVP with a signalled MVD, and the number of MVDs is determined according to the percentage of the area of the blocks coded in the the AMVP with SbTMVP mode in the previous coded picture as follows:

- If the current picture is the first coded picture in a temporal layer, the number of MVD is set to 8, as shown in figure (a).

- Otherwise, if the percentage of the area of the proposed mode is smaller than threshold1, the number of MVD is set to 4, as shown in figure (b).

- Otherwise, if the percentage of the area of the proposed mode is smaller than threshold2, the number of MVD is set to 8, as shown in figure (a).

- Otherwise, the number of MVD is set to 12, as shown in figure (c).

| (a) | (b) | (c) |
| --- | --- | --- |

In Test 2.7a, threshold1 and threshold2 are set to 3% and 6%, respectively.

In Test 2.7b, threshold1 and threshold2 are set to 4% and 7%, respectively.

### Test 2.8: On DMVR Extensions (JVET-AG0067)

In the test, 3 aspects are introduced.

In Test 2.8a, DMVR is extended to non-equal POC distance.

In Test 2.8b, 16x16 BDOF DMVR stage is added.

In Test 2.8c, mean removed equations are used to derive BDOF MV refinement parameters as follows.

(åGx.Gx+R1) * vx + åGx.Gy * vy = ådI . Gx. è (åGx.Gx+R1) * vx + åGx.Gy * vy = ådI . Gx - dM . åGx

åGx.Gy * vx  + (åGy.Gy+R1) * vy= ådI . Gy  è åGx.Gy * vx + (åGy.Gy+R1) * vy = ådI . Gy - dM . åGy

Test 2.8d: Test 2.8a + Test 2.8b + Test 2.8c

### Test 2.9: CIIP with subblock-based motion compensation (JVET-AG0135)

In the test, CIIP with subblock-based motion compensation is presented, where the inter prediction signal of CIIP is generated based on affine or SbTMVP motion compensation. A subblock-based merge candidate is used to generate the inter signal of CIIP, where the same subblock-based merge candidate list used by affine and SbTMVP is utilized.

When CIIP flag is true, a subbock-based CIIP flag is signalled. If subblock-based CIIP flag is true, an index indicating specific candidate in the subblock-based merge list is signalled, and TIMD is used to generate intra signal by default thus no CIIP-PDPC flag is signalled any more.

### Test 2.10: GPM with affine prediction (JVET-AG0164)

In ECM, GPM mode splits a CU into two partitions separated by a straight splitting line. The motion compensation for each GPM partition is performed to the whole CU, and a sample-based weighted blending process is used to generate the final prediction. Each partition of a GPM CU is merge-predicted with its own merge candidate index. In ECM, affine prediction is disallowed to predict a GPM partition.

In the test, GPM is extended to affine motion compensation, so a GPM partition can be predicted by affine motion compensation, non-affine motion compensation or intra prediction. When affine MC is applied, a uni-prediction affine merge candidate list is constructed from the subblock-based merge candidate list after discarding sub-TMVP candidates, similar to the uni-prediction merge candidate list construction for GPM in VVC.

A gpm_affine_flag is signalled for each GPM partition to indicate whether affine MC is applied for the GPM partition.

### Test 2.11: Regression-based GPM blending (JVET-AG0112)

In the test, two changes are introduced to GPM mode: an additional GPM blending mode and GPM partition merge index signalling modification.

In GPM implicit blending mode, two integer blending matrices (W0 and W1) are derived from the template (1 line above, 1 column left). The blending matrices are modelled as an affine linear function of the sample positions (x,y) in the current CU:

W0(x,y) = a.x + b.y + c  		and 	W1(x,y) = 1 - W0(x,y)

The parameters (a,b,c) are derived from the reference template using the same solver (MSE minimization) as the one used for CCCM, GLM or GL-CCCM. A list of pair of candidates is built from the regular GPM candidates and re-ordered with the template cost.

The GPM implicit mode is signalled by a CU-level flag (gpm_implicit_flag). If gpm_implicit_flag is true, a merge_idx is coded to signal the pair of GPM candidates to be used. If gpm_implicit_flag is false, the regular GPM syntax elements are signalled.

Three variants have been tested:

Test 2.11a (same as described above): a list of pair of candidates is built from the regular GPM candidates and re-ordered with the template cost. A single merge_idx is coded to signal the pair of GPM candidates to be used.

Test 2.11b: proposed GPM blending but two merge_idx are coded (one for each partition) without re-ordering, same as regular GPM syntax.

Test 2.11c: the regular GPM blending is used but a single merge_idx is coded to signal the pair of GPM candidates and the split to be used. Both pairs of candidates and split are re-ordered with the template cost.

### Test 2.11: Combination of Test 2.9, Test 2.10 and Test 2.11a (JVET-AG0142)

Test 2.11d: Test 2.10 + Test 2.11a

Test 2.11e: Test 2.9 + Test 2.10

Test 2.11f: Test 2.9 + Test 2.10 + Test 2.11a

### Test 2.12: Adjusting out-of-boundary prediction samples (JVET-AG0097)

In ECM, in bi-directional motion compensation the out of boundary (OOB) prediction samples are discarded and only the non-OOB predictors, when available, are used to generate the final predictor.

If Pi,jL0 is OOB and Pi,jL1 is non-OOB

Pi,jfinal=Pi,jL1

else if Pi,jL0 is non-OOB and Pi,jL1 is OOB

Pi,jfinal=Pi,jL0

else

Pi,jfinal=(Pi,jL0+Pi,jL1+1)>>1.

In the test, the final prediction samples for luma are generated using scales and offsets as follows,

if Pi,jL0 is OOB and Pi,jL1 is non-OOB

Pi,jfinal=(Pi,jL1*1+a1+b1+1)>>1

else if Pi,jL0 is non-OOB and Pi,jL1 is OOB

Pi,jfinal=Pi,jL0*1+a0+b0+1>>1

else

Pi,jfinal=(Pi,jL0+Pi,jL1+1)>>1.

The scales and offsets a0,a1,b0, b1 are derived over the non-OOB parts of PL0 and PL1 using least squares minimization,

mina0,b0 (i,j)∈Ωn(Pi,jfinal-(a0*Pi,jL0+b0))2

and

mina1,b1 (i,j)∈Ωn(Pi,jfinal-(a1*Pi,jL1+b1))2

where Ωn are the non-OOB samples of PL0 and PL1. Scales and offsets are derived using sample covariance and sample variance at integer precision. This modification is applied to luma only, and chroma uses ECM process.

## Transform and coefficient coding

### Test 3.1 CABAC inter/intra model switch for residual coding (JVET-AG0143)

In the current context design, residual information (e.g. residual coefficients, QtCbf, SigCoeffGroup etc.) are coded using context models depending on the slice type, but not on the prediction mode of the current CU.

It is asserted that it is better to use context models depending on the prediction rather than slice type.

In the tests, for intra coded CUs, intra context models are used independently of the slice type. Conversely, CUs which are coded with a mode similar to inter prediction (IBC and IntraTMP), inter context models are used to code residual information independently of the slice type.

The following tests have been conducted.

Test 3.1a: Use intra context models to encode residual information of intra coded CU in inter slices.

Test 3.1b: Test 3.1a and use inter context models to encode residual information of IBC or IntraTMP coded CUs in intra slices.

Test 3.1c: Test 3.1b, but QtCbf flag is excluded from the syntax elements for which a model switch is allowed.

Test 3.1d: as in Tests 3.1b and 3.1c, but all CABAC contexts are retrained.

### Test 3.2 Transform coefficient coding (JVET-AG0100)

In ECM and VVC residual coding, regular coded absolute coefficient levels are represented by sig_coeff_flag(sig), abs_level_gtx_flag[0](gt1), par_level_flag(par), abs_level_gtx_flag[1](gt3) syntax elements along with bypass coded abs_remainder (remLevel).  Associated coefficient level is reconstructed in ECM in the following way:

abs_coeff_level = sig + gt1 + par + 2*(gt3 + remLevel).

In the test, the number gtX flags coded in the regular mode of the arithmetic coding using adaptive context models before remainder coding is changed. There are N+1 flags coded in regular mode: sig_coeff_flag, gt1_flag, gt2_flag, gt3_flag, …, gtN_flag. Parity flag, par_level_flag is coded in bypass mode after the gtN_flag. In the experiments N was set to 7. Absolute coefficient level is reconstructed as follows:

abs_coeff_level = sig +gt1+ gt2 +gt3 +… + gtN + par + 2*remLevel,

The grouping of regular coded bins and by bypass coded bins is maintained.

The flags gt1, gt2, gt3 use separate contexts, flags gt4, gt5, gt6 and gt7 share the same context.

In Test 3.2a the new added contexts are not retrained, in Test 3.2b contexts related to coefficient coding are retrained. In Test 3.2c, all contexts are retrained.

Additional test noted as Test 3.b* was performed with the reduced  bin budget.

### Test 3.3: Utilizing LFNST/NSPT for inter coding (JVET-AG0061)

In the test, the utilization of LFNST/NSPT for inter coding is evaluated. For inter coded block, an intra prediction mode is first derived according to the inter prediction block with a DIMD-like process applied to the prediction. Then the derived intra prediction mode is used to select an LFNST/NSPT transform set and the transform can be processed with the selected LFNST/NSPT kernel, like in the intra coding process.

The signalling of inter LFNST/NSPT index is different from that of intra LFNST/NSPT index. The intra LFNST/NSPT index binarization employs two context coded bins for each symbol, while truncated unary code with different context models for inter LFNST/NSPT index coding is used in the test.

Encoding fast algorithm like inter MTS is applied to reduce the encoding time.

## In-loop filtering

### Test 4.1: Adaptive clipping with signalled lower and upper bounds (JVET-AG0145)

In ECM, a bit-depth based clipping defined as [0, 2^bitdepth-1] is applied in filtering, sampling, interpolation, weighted prediction, weighted combination, reconstruction, and other stages, to ensure that the generated prediction and reconstruction samples remain in the defined dynamic range.

In the test, the range [min, max] of the clipping is signalled as delta values in picture header. For I pictures, the min/max is signalled as delta values compared to [64, 940], for other pictures, the delta values compared to the min/max values of the collocated picture are signalled.

When clipping is performed in LMCS domain, the min/max values are derived by applying forward LMCS look-up table to the signalled min/max values.

At encoder, the min/max values are obtained by scanning the sample values in the original picture. If motion compensated temporal filter (MCTF) is applied to a picture, the min/max values are obtained by scanning the sample values in the MCTF pre-filtered picture. The min/max values are used as the lower and upper bounds for clipping at the reconstruction stage (when adding the residual signal to the prediction) and before saving the reconstructed picture into the decoded picture buffer.

Test 4.1a: Adaptive clipping with signalled min/max values from MCTF prefiltered picture.

Test 4.1b: Adaptive clipping with signalled min/max values from original picture.

### Test 4.2: Fixed filter for chroma ALF (JVET-AG0157)

In the tests, fixed filter for chroma ALF is introduced.

In Test 4.2a, like the classifiers for luma component, a classifier based on Laplacian values and variance is applied to a 2x2 chroma block. The derived class index is then used to select a fixed filter from a chroma filter set, which is trained from BVI video set. A chroma fixed filter is applied to chroma ALF input samples in a 13x13 diamond shape and DBF input samples in a 7x7 diamond shape.

In Test 4.2b, luma ALF fixed filter is reused. The first luma classifier is applied to each 2x2 chroma block, then the derived class index is used to select a fixed filter from the luma fixed filter set related to this classifier. A fixed filter is applied to chroma ALF input sample in a 9x9 diamond shape and DBF input samples in a 9x9 diamond shape.

Finally, in a signalled chroma filter, 5x5 crossing extra taps are introduced, which are applied to the fixed filter output.

### Test 4.3: Adaptive precision for luma ALF coefficients (JVET-AG0158)

In ALF of ECM-11.0, 7 bits are used to represent the fractional part of a coefficient of a signalled ALF filter.

In the test, the number of bits used to represent the fractional part of a luma coefficient is adaptive from 5 to 8, inclusively. For each luma filter set, which contains up to 25 filters, a 2-bit syntax element is signalled in APS to indicate the number of bits used for the coefficients in this set. The value range of a coefficient is not changed.

## Entropy coding

### Tests 5.1-5.2: CABAC context initialization retraining and slice type based window offsets (JVET-AG0196)

In ECM, CABAC has the following parameters: probability states (6 bits), short and long window sizes (4 bits), weights (6 bits), and offsets for short (8 bits) and long (8 bits) windows. In total, (6 bits + 4 bits + 6 bits) * 3 slices + 8 bits * 2 windows = 64 bits is required per context.

In Test 5.1, initialization parameters of all CABAC contexts for all slices are retrained.

In Test 5.2, windows offsets defined per slice type are added and contexts are retrained. It requires 8 bits * 2 windows * 2 slices = 32 bits additional memory per context.

Context training software is provided located at source/App/CabacTraining including readme file describing how to use it.

### Test 5.3: Spatial CABAC tuning (JVET-AG0117)

In ECM, CTUs are scanned in raster scan order, while CUs inside CTUs are scanned in nested tree order. When starting to process the current CTU (CTUC) the processing “jumps” from the bottom-right CU (yellow CU 8) to the top-left CU of the current CTU (blue CU 0) as shown in the next figure. At that stage, the context parameters of the CABAC engine can be expected to be fine tuned for the bottom-right area of the previous CTU instead of the ideal case where the context parameters would be fine tuned for the actual next CU to be processed.

In the tested method, syntax elements bins of the bottom CUs of the above CTU (the green CUs in the figure) are used to tune the CABAC contexts when starting to process a new CTU.

In the implementation, bins for 128 contexts are buffered for each CTU of a CTU line. The number of bin values that can be stored for each of those contexts is limited to 4 in this experiment. Thus, a maximum of 128 * 4 bin values equalling to 512 bin values may be stored for each CTU in one CTU row.

When starting to encode or decode a new CTU, the stored bins from the above CTU are retrieved and fed into the context update mechanism of the shorter window probability estimator (with “window 0”). The bins are fed to context update process in the reverse order with respect to their coding order to simulate encoding or decoding that approaches the top-left corner of the new CTU from the direction of bottom-right corner of the CTU above.

Decoded bins are stored in a buffer, an element of such buffer should hold a 10-bit context ID (for the current ECM where the number of contexts is below 1024), a 4-bit bin storage containing the bin values for that context and a 2-bit counter indicating how many bins are stored for that context summarized in the next table.

| Test | Bits / element | Number of elements | Total bits / CTU | Total bytes / CTU |
| --- | --- | --- | --- | --- |
| Test 5.3 | 10+4+2 = 16 | 128 | 2048 | 256 |

The total memory requirement then depends on the number of CTUs in a CTU row. Considering the CTC settings for different classes of content and the memory elements outlined above, the total amount of memory needed for the Tests 5.3a and 5.3b are as follows:

| Class | CTUs / | Bytes / | Bytes / | Bytes for | Bytes |
| --- | --- | --- | --- | --- | --- |
|  | row | CTU | CTU row | active CTU | Total |
| A1 | 15 | 256 | 3840 | 747 | 4587 |
| A2 | 15 | 256 | 3840 | 747 | 4587 |
| B | 15 | 256 | 3840 | 747 | 4587 |
| C | 7 | 256 | 1792 | 747 | 2539 |
| D | 4 | 256 | 1024 | 747 | 1771 |
| E | 10 | 256 | 2560 | 747 | 3307 |
| F | 15 | 256 | 3840 | 747 | 4587 |
| TGM | 15 | 256 | 3840 | 747 | 4587 |

In Test 5.3b, context initialization from Test 5.1 is used.

# Test results

See the accompanied Excel table.