| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0056-v1 |
| --- | --- |

| Title: | EE1-1.4: Joint design of rotation and flipping on NNLF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhihuang Xie Yue Yu Haoping Yu Dong Wang(OPPO) Renjie Chang Liqiang Wang Xiaozhong Xu Shan Liu(Tencent) | Tel: Email: | xiezhihuang@oppo.com renjiechang@tencent.com liqiangwang@tencent.com xiaozhongxu@tencent.com shanl@tencent.com |
| Source: | OPPO, Tencent |  |  |

_____________________________

# Abstract

This contribution reports the test results of EE1-1.4. In EE1-1.4, both rotation and flipping operations are proposed to be applied to the inputs and output of the NNVC unified in-loop filter to further improve the coding performance. The simulation results are summarized as follows:

Compare with NNVC-7.1 (LOP2+NnIntra):

RA:  {-0.06% (Y), -0.45% (U), -0.41% (V), 100% (EncT), 104% (DecT)}

LDB: {-0.41% (Y), -1.51% (U), -1.86% (V), 98% (EncT), 102% (DecT)}

Compare with NNVC-7.1 (HOP2+NnIntra):

RA:  {-0.13% (Y), -0.13% (U), -0.44% (V), 101% (EncT), 115% (DecT)}

LDB: { x.xx% (Y), x.xx% (U), x.xx% (V), xx% (EncT), xx% (DecT)}

# Introduction

In the current NNVC software, two unified neural-network loop filters are working as additional in-loop filters to improve the coding performance. One is the high operation point (HOP) filter, which provides more coding gain with a higher complexity. The other one is the low operation point (LOP) filter, and it can achieve a good coding gain with only 17.0 KMAC/pixel computational complexity.

The network architectures of HOP and LOP are shown in Fig.1 and Fig.2, respectively. The inputs of both HOP and LOP network contain reconstruction, prediction, boundary strength, sliceQP, baseQP and IPB. The outputs of both HOP and LOP network contain luma and chroma filtered reconstruction samples.

Fig. 1 Network architecture of HOP filter

Fig. 2 Network architecture of LOP filter

# Proposed

## Geotransform to input and output of NNLF

This EE studies the joint design of rotation[1] and flipping[2] methods on top of the unified NNLF. Both rotation and flipping have two types of transform, as shown in Fig. 3. If the inputs of NNLF are rotated 90 degrees clockwise then the output of NNLF will be rotated 90 degrees counter clockwise to restore the correct positions of filtered reconstruction samples. Similarly, if the inputs are rotated 90 degrees counter clockwise firstly then the output should be rotated 90 degrees clockwise. In the case that the inputs of NNLF are flipped vertically, then the output of NNLF will be flipped vertically back to get the correct order of filtered reconstruction samples, and a similar flip-back operation is performed for horizontal flip.

Fig. 3 Rotation and flipping operations

A slice-level flag is signalled to indicate the use of this geotransform. An index of geotransform type is further signalled to indicate which geotransform candidate is applied to all CTUs in the current slice, in case that the use of geotransform flag is true. In addition, this geotransform is only allowed for B slice.

To reduce the encoder complexity, some encoder-only fast algorithms are investigated in this joint design. For example,

- Only partial CTUs are filtered and used to calculate the distortion between the filtered and unfiltered images, while encoder is doing RDO decision on baseQP offsets and geotransform types. As shown in Fig. 4, only shadowed CTUs will be checked.

- If the distortion of an NN based filtered image is worse than the unfiltered image, then the encoder will skip checking geotransforms.

- The encoder searches for at least two geotransform candidates. If there is a geotransform candidate that is working better than the previous best result, then the encoder will skip checking the rest of geotransform candidates.

CTU

Fig. 4 Encoder-side method

## Inference stage

The inference information of HOP case is identical to NNVC-7.1 HOP2 filter.

| ## Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 1.45 M/model |
|  | Total Number of Parameters (All Models) | 1.45 M/model |
|  | Parameter Precision (Bits) | 16 |
|  | Memory Parameter (MB) | 2.8 MB/model |
|  | Multiply Accumulate (kMAC/pixel) | 477 |
|  | Calculation Method | block basis |
|  | Total Conv. Layers | 154 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 144x144, 272x272 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

The inference information of LOP case is identical to NNVC-7.1 LOP2 filter.

| ## Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 51248 |
|  | Total Number of Parameters (All Models) | 51248 |
|  | Parameter Precision (Bits) | 16 |
|  | Memory Parameter (MB) | 0.05 |
|  | Multiply Accumulate (kMAC/pixel) | 17.0 |
|  | Calculation Method | block basis |
|  | Total Conv. Layers | 106 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 128x128, 256x256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

# Experimental results

The proposed method is implemented on top of NNVC-7.1 HOP and LOP, and all of the tests are conducted under the common test condition[3].

For the results in 3.1, both the anchor and test are NNVC-7.1 with NnIntra and NnlfLOP2 enabled.

For the results in 3.2, the test is NNVC-7.1 with NnIntra and NnlfLOP2 enabled, and the anchor is VTM.

For the results in 3.3, both the anchor and test are NNVC-7.1 with NnIntra and NnlfHOP2 enabled.

For the results in 3.4, the test is NNVC-7.1 with NnIntra and NnlfHOP2 enabled, and the anchor is VTM.

## Proposed (LOP2) vs. NNVC-7.1

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over nnvc7.1-LOP2 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.01% | -0.11% | 0.11% | 101% | 100% |
| Class A2 | -0.09% | -0.26% | -0.09% | 99% | 101% |
| Class B | -0.06% | -0.25% | -0.40% | 100% | 107% |
| Class C | -0.10% | -1.09% | -1.04% | 98% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.06% | -0.45% | -0.41% | 100% | 104% |
| Class D | -0.19% | -1.31% | -1.39% | 100% | 107% |
| Class F |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | BD-rate Over nnvc7.1-LOP2 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.30% | -1.51% | -1.83% | 100% | 104% |
| Class C | -0.52% | -2.32% | -3.31% | 100% | 101% |
| Class E | -0.45% | -0.43% | 0.04% | 94% | 99% |
| Overall | -0.41% | -1.51% | -1.86% | 98% | 102% |
| Class D | -0.51% | -3.19% | -3.38% | 101% | 100% |
| Class F |  |  |  |  |  |

## Proposed (LOP) vs. VTM anchor

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -7.90% | -10.19% | -10.51% | 96% | 7199% |
| Class A2 | -6.87% | -12.90% | -9.82% | 94% | 6794% |
| Class B | -6.63% | -14.14% | -13.05% | 169% | 14456% |
| Class C | -6.67% | -15.78% | -15.32% | 141% | 9491% |
| Class E |  |  |  |  |  |
| Overall | -6.94% | -13.54% | -12.50% | 128% | 9665% |
| Class D | -7.34% | -15.94% | -16.17% | 138% | 9527% |
| Class F |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | BD-rate Over VTM-anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -5.58% | -14.66% | -14.84% | 116% | 8439% |
| Class C | -6.02% | -15.70% | -17.75% | 99% | 7695% |
| Class E | -6.56% | -9.30% | -12.06% | 115% | 8587% |
| Overall | -5.97% | -13.67% | -15.11% | 110% | 8219% |
| Class D | -7.01% | -15.90% | -18.32% | 100% | 7559% |
| Class F |  |  |  |  |  |

## Proposed (HOP2) vs. NNVC-7.1 with HOP2 enabled

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over nnvc7.1-HOP2 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -0.07% | 0.17% | -0.24% | 101% | 115% |
| Class A2 | -0.16% | -0.22% | -0.13% | 102% | 115% |
| Class B | -0.12% | 0.10% | -0.43% | 100% | 115% |
| Class C | -0.18% | -0.56% | -0.84% | 102% | 114% |
| Class E |  |  |  |  |  |
| Overall | -0.13% | -0.13% | -0.44% | 101% | 115% |
| Class D | -0.27% | -0.50% | -0.91% | 107% | 116% |
| Class F | -0.07% | -0.31% | -0.39% | 88% | 115% |
|  | Low delay B Main10 |  |  |  |  |
|  | BD-rate Over nnvc7.1-HOP2 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.41% | -1.77% | -2.10% | 100% | 113% |
| Class E | -0.34% | 0.24% | -0.93% | 83% | 114% |
| Overall |  |  |  |  |  |
| Class D | -0.51% | -1.12% | -2.75% | 107% | 115% |
| Class F |  |  |  |  |  |

## Proposed (HOP2) vs. VTM anchor

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -15.34% | -8.03% | -18.84% | 443% | 241697% |
| Class A2 | -14.65% | -11.63% | -9.60% | 425% | 232324% |
| Class B | -12.40% | -12.22% | -12.52% | 432% | 241316% |
| Class C | -13.39% | -17.13% | -17.56% | 354% | 220612% |
| Class E |  |  |  |  |  |
| Overall | -13.70% | -12.58% | -14.54% | 411% | 233903% |
| Class D | -14.57% | -16.21% | -18.12% | 365% | 210643% |
| Class F | -8.34% | -11.91% | -10.94% | 561% | 109521% |
|  | Low delay B Main10 |  |  |  |  |
|  | BD-rate Over VTM-anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -12.64% | -14.98% | -16.22% | 339% | 221989% |
| Class E | -13.62% | -3.68% | -10.72% | 750% | 308395% |
| Overall |  |  |  |  |  |
| Class D | -14.00% | -13.14% | -17.22% | 343% | 208345% |
| Class F |  |  |  |  |  |

# Conclusion

This contribution reports the results of EE1-1.4, which studies the joint design of rotation and flipping operations on the input and output of NNLF. The proposed method provides 0.06%, 0.41%coding gain for RA and LDB compared with NNVC-7.1 LOP2, and provides 0.13%, x.xx% coding gain for RA and LDB compared with NNVC-7.1 with HOP2. Considering the good trade-off performance, it is suggested to adopt this method into the next version of NNVC common software.

# Reference

[1] R. Chang, L. Wang, X. Xu, S. Liu. EE1-1.2.3: Input and output rotation of model for NNVC in-loop filter. JVET-AF0154, 32nd Meeting, Hannover, DE, 13–20 October 2023.

[2] Z. Xie, Y. Yu, H. Yu, D. Wang. EE1-1.2.6: On flipping of input and output of model in NNVC HOP filter. JVET-AF0086, 32nd Meeting, Hannover, DE, 13–20 October 2023.

[3] E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-AF2016, 32nd Meeting, Hannover, DE, 13–20 October 2023.

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).