| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0060-v1 |
| --- | --- |

| Title: | EE2-1.19: IBC-LIC Model Merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lai Zhang Yue Yu Haoping Yu Dong Wang | Tel: Email: | zhanglai@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution reports the EE2-1.19 simulation results for the IBC-LIC model merge mode. The proposed method inherits IBC-LIC model parameters from spatial adjacent and non-adjacent neighbors, history candidates, or default models. On top of ECM-11.0, the performance of EE2-1.19 is reported as follows:

AI: { 0.01% Y, 0.03% U, 0.04% V, 101.3% EncT, 100.4% DecT } for Overall

{ -0.41% Y, -0.49% U, -0.30% V, 100.4% EncT, 100.2% DecT } for class F

{ -0.15% Y, -0.15% U, -0.17% V, 100.6% EncT, 100.5% DecT } for class TGM

RA: { 0.01% Y, -0.03% U, -0.10% V, 100.3% EncT, 100.6% DecT } for Overall

{ -0.30% Y, -0.40% U, -0.29% V, 100.4% EncT, 100.7% DecT } for class F

{ 0.00% Y, -0.07% U, -0.03% V, 100.5% EncT, 100.3% DecT } for class TGM

Introduction

In JVET-AF0084[1], an IBC-LIC model merge mode is proposed to inherit IBC-LIC model parameters from previously coded blocks. More specifically, an IBC-LIC model for an IBC-LIC model merge mode can be obtained as follows:

- Construct a model candidate list which consists of model parameters from spatial adjacent and non-adjacent neighbors, history candidates, and default models. The size of the candidate list is twelve.

Similar to the CCP merge method, IBC-LIC models are collected from the previously coded IBC-LIC and IBC-LIC model merge modes in both adjacent and non-adjacent positions. When an IBC-LIC model candidate is a multi-model IBC-LIC coded block, the parameters from two linear models and the classification threshold are inherited accordingly.

Besides, a history IBC-LIC model table with a size of twelve is maintained. The IBC-LIC models from spatial neighbors and the history IBC-LIC model table are added to the IBC-LIC model merge candidate list. If the list is not full, a default model and the scaled models are then added to the list. To avoid redundant models, a pruning operation is also applied.

- Select an IBC-LIC model from the candidate list and signal its index in the bitstream.

The IBC-LIC model merge mode can be applied to both IBC-merge mode and IBC-AMVP mode. In IBC-merge mode, the selected LIC model is applied to the reference template during merge list reordering. Then the IBC merge list is further sorted by the inherited IBC LIC flags. The proposed mode is used only if there is an adjacent block coded as IBC-LIC or the proposed mode. In the IBC-AMVP mode, the selected LIC model is applied to the reference template during BVD prediction.

A flag is signaled to indicate whether the proposed IBC-LIC merge mode is applied or not. If this flag is true, an index is further signaled to indicate which candidate model is used by the current block.

Simulation Results

EE2-1.19 was implemented on top of ECM-11.0 [2] and tested based on the common test condition as specified in the Exploration Experiments 2 document [3]. The test result is summarized below.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | -0.03% | 0.01% | 101.2% | 100.5% | 100.9% | 101.6% |
| Class A2 | 0.00% | 0.02% | 0.07% | 101.2% | 100.6% | 101.0% | 101.8% |
| Class B | 0.01% | 0.04% | 0.00% | 101.3% | 100.9% | 101.5% | 101.0% |
| Class C | 0.01% | -0.05% | 0.06% | 101.2% | 99.4% | 100.1% | 100.5% |
| Class E | 0.04% | 0.18% | 0.11% | 101.3% | 100.5% | 100.6% | 100.6% |
| Overall | 0.01% | 0.03% | 0.04% | 101.3% | 100.4% | 100.9% | 101.1% |
| Class D | 0.01% | -0.09% | 0.07% | 100.9% | 100.4% | 100.4% | 100.4% |
| Class F | -0.41% | -0.49% | -0.30% | 100.4% | 100.2% | 99.7% | 100.7% |
| Class TGM | -0.15% | -0.15% | -0.17% | 100.6% | 100.5% | 100.7% | 100.8% |

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.01% | -0.16% | -0.12% | 99.9% | 100.3% | 103.5% | 104.3% |
| Class A2 | 0.02% | 0.11% | 0.00% | 100.2% | 101.5% | 104.1% | 104.0% |
| Class B | 0.02% | -0.01% | -0.24% | 100.3% | 100.4% | 102.7% | 102.7% |
| Class C | -0.01% | -0.05% | 0.02% | 100.7% | 100.6% | 100.1% | 101.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | 0.01% | -0.03% | -0.10% | 100.3% | 100.6% | 102.4% | 102.8% |
| Class D | -0.01% | -0.29% | 0.23% | 100.3% | 100.7% | 100.9% | 100.5% |
| Class F | -0.30% | -0.40% | -0.29% | 100.4% | 100.7% | 101.6% | 101.6% |
| Class TGM | 0.00% | -0.07% | -0.03% | 100.5% | 100.3% | 102.1% | 102.2% |

Conclusions

This contribution reports the simulation results of EE2-1.19. It is reported that the proposed method in EE2-1.19 provides attractive coding performance on top of ECM-11.0 for class F and class TGM with negligible complexity change. It is suggested to adopt this method into next version of ECM.

Reference

- L. Zhang, et. al., “Non-EE2: Update on IBC-LIC Model Merge mode” JVET-AF0084, October 2023

- ECM-11.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-11.0

- V. Seregin, et. al., “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-AF2024, October 2023

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).