| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0062-v1 |
| --- | --- |

| Title: | Non-EE2: Multiple Transform Set Selection for LFNST/NSPT |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fan Wang Yue Yu Haoping Yu Dong Wang | Tel: Email: | wangfan6@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution proposes a multiple transform set selection method for intra blocks coded with LFNST/NSPT. In the proposed method, CUs coded with DIMD, TIMD, MIP, SGPM and ITMP modes can choose an alternative LFNST/NSPT transform set. The transform set selection is signaled in the bit-stream. On top of ECM-11.0, the performance of the proposed method is reported as follows:

AI {xx% Y, xx% U, xx% V, xx% EncT, xx% DecT}

RA {xx% Y, xx% U, xx% V, xx% EncT, xx% DecT}

Introduction

LFNST (low frequency non-separable transform) is a transform coding tool in VVC. In ECM, LFNST is further enhanced with more transform sets for a finer granularity of directions, i.e., from 4 to 35, and more transform kernels in each set, i.e., from 2 to 3. In addition, larger kernels are introduced for large blocks.

NSPT (non separable primary transform) is introduced to replace the separable DCT-II plus LFNST transform combinations for the block shapes of 4x4, 4x8/8x4, 4x16/16x4, 8x8, 8x16/16x8, 4x32/32x4, and 8x32/32x8 in ECM. The kernel mapping method for intra prediction mode is the same as LFNST.

The LFNST/NSPT transform set is implicitly decided by an intra prediction mode (IPM). For a block coded with PLANAR, DC or directional modes, the IPM is set as the intra prediction mode itself. For a DIMD coded block, the IPM is set as the best mode derived by the DIMD process with neighboring reconstructed pixels. For a TIMD coded block, the IPM is set as the best mode derived by the TIMD process. For a MIP or ITMP coded block, the IPM is set as the best mode derived by the DIMD process with the prediction. For an SGPM coded block, the IPM is set as the mode corresponding to the split mode of SGPM.

The prediction strategies of DIMD, TIMD, SGPM, MIP and ITMP are different from normal intra prediction modes like PLANAR, DC and directional modes. DIMD, TIMD, SGPM and ITMP allow multiple predictions combination. MIP makes the prediction using matrix calculation. ITMP references the reconstructed region in the current picture. The prediction residual may illustrate different feature from the derived IPM.

Proposed method

It is proposed to allow CUs coded with DIMD, TIMD, MIP, SGPM and ITMP to select one LFNST/NSPT transform set out of 2 candidate sets. If the current block is coded with DIMD, TIMD, MIP, SGPM, ITMP and LFNST/NSPT is used, one more bin is employed to indicate whether the 1st or the 2nd candidate transform set is selected. The 1st candidate transform set remains the same as the current ECM. The 2nd candidate is derived by the DIMD process with neighboring reconstructed pixels. Specially, if a TIMD coded block applies fusion, the 2nd TIMD IPM is first considered to derive the 2nd candidate set. The difference between the 1st and 2nd IPM should be larger than 3 to keep the diversity.

Simulation results

The proposed method is implemented on top of the ECM-11.0 software [1] and tested based on the common test condition [2]. The test results are summarized below.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over anchor |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.05% | 0.10% | 0.04% |  |  |  |  |
| Class C | -0.06% | 0.00% | 0.06% |  |  |  |  |
| Class E | -0.06% | 0.00% | 0.06% |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.08% | -0.02% | 0.22% |  |  |  |  |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

Conclusions

This contribution proposes a multiple transform set selection method for intra blocks coded with DIMD, TIMD, MIP, SGPM, ITMP and LFNST/NSPT. The proposed method brings -xx% coding gain in AI. It is recommended to investigate the proposed method in EE2.

References

- ECM-11.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-11.0

- M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing” JVET-AF2017, October 2023

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).