| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0065-v1 |
| --- | --- |

| Title: | Non-EE2: Adaptive coefficient precision for CCALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Song, Yue Yu, Haoping Yu, Dong Wang | Email: | songnan@oppo.com |
| Source: | OPPO |  |  |

_____________________________

# Abstract

This contribution proposes a method of using adaptive coefficient precision for CCALF to replace the fixed 7-bit precision currently used in ECM. The proposed method is implemented on top of ECM-11.0 software, and its performance is reported as follows:

AI: { 0.00% Y, -0.20% U, -0.25% V, 100.2% EncT, 100.5% DecT }

RA: { 0.00% Y, -0.26% U, -0.39% V, 99.9% EncT, 100.0% DecT }

# Introduction

In Cross Component Adaptive Loop Filter (CCALF) of ECM-11.0, the coefficient precision used in training and filtering is a fixed value, i.e., the precision value for both Cb and Cr is set to 7-bit. In the training process, the coefficients of each filter are calculated by solving the corresponding Weiner-Hopf equation. Those coefficients are floating-point numbers during the calculation, and then they are mapped to 0,±1,±2,±4,±8,±16,±32,±64 by multiplication with 128 and along with a rounding process. In the filtering process, because the coefficients have been multiplied by 128 previously, the filter outputs would right-shift 7 bits to get the final correct results.

# Proposed methods

This contribution proposes to apply an adaptive coefficient precision in CCALF.

In the training process, there are four coefficient precision candidates, and the candidate list is set to be {7, 8, 9, 10}. Firstly, each frame traverses those precision candidates to get the corresponding filter coefficients, then the corresponding distortion and bit cost for each precision are calculated. The coefficient precision corresponding to the minimum RD cost is selected as the best coefficient precision bestFilterCoeffPrec for the current frame. The best coefficient precision bestFilterCoeffPrec together with the integerized filter coefficients calculated with the selected bestFilterCoeffPrec, CTU filter index and the number of filters are used for the filtering process and therefore are coded in the bitstream. It is noted that all the filters for the same chroma component in one frame share the same coefficient precision bestFilterCoeffPrec.

During the filtering process, the coefficient precision bestFilterCoeffPrec instead of a fixed 7-bit precision value will be used. For chroma pixel at location (xc,yc), the output of the filter is calculated as follows:

offset(xc,yc)=ifui,viinteger.IxL+ui,yL+vi+1≪bestFilterCoeffPrec -1≫bestFilterCoeffPrec

where bestFilterCoeffPrec is the best coefficient precision selected in the training process; fui,viintegeris the integerized coefficient calculated with bestFilterCoeffPrec; Ix+ui,y+viis the corresponding luma pixel.

The filtered pixel RecC(xc,yc)' after CCALF is:

RecC(xc,yc)'=RecC(xc,yc)+offset(xc,yc)

where RecC(xc,yc) is the chroma pixel value at location (xc,yc) before CCALF.

There are two bestFilterCoeffPrec syntax elements for each frame, one for Cr and one for Cb, and they would be coded individually in APS.

# Experimental results

The proposed method is implemented on top of ECM-11.0 [1]. The test results are reported as follows by following the common test conditions [2]:

Table 1. The proposed method on top of ECM-11.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.09% | -0.09% | 100.3% | 100.7% |
| Class A2 | 0.00% | -0.17% | -0.10% | 100.3% | 100.4% |
| Class B | 0.00% | -0.24% | -0.41% | 100.2% | 101.0% |
| Class C | 0.01% | -0.17% | -0.25% | 100.1% | 99.8% |
| Class E | 0.01% | -0.30% | -0.32% | 100.1% | 100.4% |
| Overall | 0.00% | -0.20% | -0.25% | 100.2% | 100.5% |
| Class D | 0.02% | -0.26% | -0.47% | 100.3% | 99.7% |
| Class F | 0.01% | -0.42% | -0.47% | 100.3% | 100.3% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-11.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.22% | -0.23% | 99.7% | 100.0% |
| Class A2 | 0.02% | -0.10% | -0.22% | 99.6% | 99.8% |
| Class B | 0.00% | -0.39% | -0.62% | 100.0% | 99.9% |
| Class C | 0.01% | -0.25% | -0.35% | 100.0% | 100.1% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.26% | -0.39% | 99.9% | 100.0% |
| Class D | 0.02% | -0.43% | -0.81% | 100.2% | 100.1% |
| Class F | 0.01% | -0.35% | -0.52% | 100.1% | 99.6% |

# Conclusion

This contribution proposes using an adaptive coefficient precision instead of a fixed precision in CCALF. The results show that the proposed method can improve the coding efficiency of ECM with minor complexity increases. It is suggested to further study this method in EE2.

# References

- ECM11.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-11.0

- M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, October, 2023.

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).