| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by Teleconference, 17–26 January 2024 | Document: JVET-AG0070-v1 |
| --- | --- |

| Title: | AHG9: Comments on Source Picture Timing Information Message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande Jonatan Samuelsson-Allendes | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

Some modifications are proposed to the Source Picture Timing Information SEI Message. Following is proposed:

- Proposal 1: It is asserted that the current signalling in source picture timing SEI message includes only an integer scale factor for temporal sublayer interval. It is proposed to signal the scale factor for temporal sublayer interval as a numerator and a denominator.

- Proposal 2: It is proposed to signal spti_num_units_in_elemental_interval with a minus 1 coding to avoid signalling the value 0.

Alternatively, it is proposed to add a constraint to prevent signalling the value 0 for spti_num_units_in_elemental_interval.

Proposal 3: An asserted bug-fix is proposed for the derivation of the variable temporalReversalFlag.

# Introduction

The Source Picture Timing Information (SPTI) SEI message is included in the Technology under Consideration (TuC) [1]. The source picture timing information (SPTI) SEI message indicates the temporal distance between source pictures associated with the corresponding decoded output pictures prior to encoding.

This document proposes modifications to the SPTI message in TuC [1].

# Proposals

## Proposal 1

It is asserted that the current signaling in source picture timing SEI message includes only an integer scale factor for temporal sublayer interval signalled by the syntax element spti_sublayer_interval_scale_factor[ i ].

As a result the source picture interval associated with output pictures having TemporalId less than or equal to i, denoted by the variable SourcePictureInterval[ i ], in units of seconds can only be an integer multiple of ElementalSourcePictureInterval. It is asserted that for the most general and flexible signaling to cater to all current and future use cases, the scale factor for sublayers should be signalled as a fractional number, by signalling a numerator and a denominator.

For example, in a scenario where the syntax elements spti_num_units_in_elemental_interval and spti_time_scale are signalled such that ElementalSourcePictureInterval corresponds to the elemental source picture interval of a very high speed camera, the current signalling allows only integer scale factors for signaling source picture interval of temporal sublayers. Now consider an example case where the high speed camera generated X pictures per second and ElementalSourcePictureInterval is signalled as 1/X seconds (i.e. the syntax elements spti_num_units_in_elemental_interval and spti_time_scale are signalled such that spti_num_units_in_elemental_interval/ spti_time_scale = 1/X). If picture rate upsampling is then later on applied on the high speed captured content to double the overall picture rate (i.e. the overall picture rate is 2X), then the scale factor at the highest temporal sublayer is ½ (since the source picture interval is half of the ElementalSourcePictureInterval which is 1/X) which cannot be signalled with the current signalling since spti_sublayer_interval_scale_factor[ i ] can only be integer.

It is thus proposed to signal the scale factor for temporal sublayers as a fraction, by signalling a numerator and a denominator. To avoid signalling a 0 value for the denominator, it is proposed to signal denominator syntax element with a minus1 coding. The proposed modifications are shown below compared to VSEI TuC (JVET-AF2032) [1].

  - Source picture timing information SEI message

    - Source picture timing information SEI message syntax

| source_picture_timing_info( payloadSize ) { | Descriptor |
| --- | --- |
| spti_cancel_flag | u(1) |
| if( !spti_cancel_flag ) { |  |
| spti_persistence_flag | u(1) |
| spti_source_timing_equals_output_timing_flag | u(1) |
| if( !spti_source_timing_equals_output_timing_flag ) { |  |
| spti_source_type_present_flag | u(1) |
| if( spti_source_type_present_flag ) |  |
| spti_source_type | u(16) |
| spti_time_scale | u(32) |
| spti_num_units_in_elemental_interval | u(18) |
| if( spti_persistence_flag ) |  |
| spti_max_sublayers_minus_1 | u(3) |
| for( i = 0; i <= spti_max_sublayers_minus1; i++ ) { |  |
| spti_sublayer_interval_scale_factor_numerator[ i ] | ue(v) |
| spti_sublayer_interval_scale_factor_denominator_minus1[ i ] | ue(v) |
| spti_sublayer_interval_scale_factor[ i ] | ue(v) |
| spti_sublayer_synthesized_picture_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |

    - Source picture timing information SEI message semantics

The source picture timing information (SPTI) SEI message indicates the temporal distance between source pictures associated with the corresponding decoded output pictures prior to encoding, e.g., for camera-captured content, the temporal distance between source pictures is the difference between the time at which an image sensor was exposed to produce a source picture associated with the current decoded picture and the time at which the image sensor was exposed to produce the source picture associated with a previous decoded picture in output order.

spti_cancel_flag equal to 1 indicates that the SPTI SEI message cancels the persistence of any previous SPTI SEI message in output order that applies to the current layer. spti_cancel_flag equal to 0 indicates that source picture timing information follows.

…

spti_time_scale specifies the number of time units that pass in one second. The value of spti_time_scale shall not be equal to 0. For example, a time coordinate system that measures time using a 27 MHz clock has an spti_time_scale of 27,000,000.

spti_num_units_in_elemental_interval specifies the number of time units of a clock operating at the frequency spti_time_scale Hz that corresponds to the indicated elemental source picture interval of consecutive pictures in output order in the CLVS.

The indicated elemental source picture interval, also to be denoted by the variable ElementalSourcePictureInterval, in units of seconds, is equal to the quotient of spti_num_units_in_elemental_interval divided by spti_time_scale. For example, to represent an elemental source picture interval equal to 0.04 seconds, spti_time_scale may be equal to 27,000,000 and spti_num_units_in_elemental_interval may be equal to 1,080,000.

spti_max_sublayers_minus_1 plus 1 specifies the maximum number of temporal sublayers for which picture interval scale factor (spti_sublayer_interval_scale_factor[ i ]) and synthesized flag (spti_sublayer_synthesized_picture_flag[ i ]) information is signalled. When spti_max_sublayers_minus_1 is not present, it is inferred to be equal to TemporalId..

spti_sublayer_interval_scale_factor_numerator[ i ] when present, specifies a numerator of the scale factor used in determining the source picture interval of corresponding consecutive pictures in output order in the CLVS having TemporalId less than or equal to i. The value 0 may be used to indicate that the source picture corresponding to the current decoded output picture is identical to the source picture corresponding to the previous decoded output picture. spti_sublayer_interval_scale_factor_numerator[ i ] shall be in the range of 0 to 65535, inclusive.

spti_sublayer_interval_scale_factor_denominator_minus1[ i ] plus 1 when present, specifies a denominator of the scale factor used in determining the source picture interval of corresponding consecutive pictures in output order in the CLVS having TemporalId less than or equal to i.  spti_sublayer_interval_scale_factor_denominator_minus1[ i ] shall be in the range of 0 to 65534, inclusive.

spti_sublayer_interval_scale_factor[ i ], when present, specifies a scale factor used in determining the source picture interval of corresponding consecutive pictures in output order in the CLVS having TemporalId less than or equal to i. The value 0 may be used to indicate that the source picture corresponding to the current decoded output picture is identical to the source picture corresponding to the previous decoded output picture.

The indicated source picture interval associated with output pictures having TemporalId less than or equal to i, denoted by the variable SourcePictureInterval[ i ], in units of seconds, is derived as follows:

SourcePictureInterval[ i ] = ElementalSourcePictureInterval * spti_sublayer_interval_scale_factor[ i ] (spti_sublayer_interval_scale_factor_numerator[ i ] ÷ (spti_sublayer_interval_scale_factor_denominator[ i ]+1) * ( 1 – 2 * temporalReversalFlag )				(8-X)

The variable temporalReversalFlag is equal to ( spti_source_type & 0x10 )? 1 : 0.

spti_sublayer_synthesized_picture_flag[ i ], when present, equal to 1 indicates that decoded output pictures belonging to the ith temporal sublayer are synthesized and do not correspond to unmodified original source pictures. spti_sublayer_synthesized_picture_flag[ i ] equal to 0 provides no such indication. When not present, the value of spti_sublayer_synthesized_picture_flag[ i ] is inferred to be equal to 0.

Note 1 – When the TemporalId of an SPTI SEI message is greater than 0, and the SPTI SEI message persists for one or more pictures with lower TemporalId, an encoder can repeat the information of the SPTI SEI message by including it in one or more SPTI SEI messages with lower TemporalId, in order to avoid loss of information when pictures in temporal sublayer(s) are lost or removed.

## Proposal 2

It is asserted that spti_num_units_in_elemental_interval shall not be equal to 0. Two alternative options are proposed related to the syntax element spti_num_units_in_elemental_interval.

### Option 1

It is proposed to signal spti_num_units_in_elemental_interval with a minus 1 coding to avoid signalling the value 0. This also allow signaling one more value for number of units in elemental interval compared to the TuC since the syntax element spti_num_units_in_elemental_interval is coded as u(18).

The proposed modifications are shown highlighted below compared to JVET-AF2032 [1].

  - Source picture timing information SEI message

    - Source picture timing information SEI message syntax

| source_picture_timing_info( payloadSize ) { | Descriptor |
| --- | --- |
| spti_cancel_flag | u(1) |
| if( !spti_cancel_flag ) { |  |
| spti_persistence_flag | u(1) |
| spti_source_timing_equals_output_timing_flag | u(1) |
| if( !spti_source_timing_equals_output_timing_flag ) { |  |
| spti_source_type_present_flag | u(1) |
| if( spti_source_type_present_flag ) |  |
| spti_source_type | u(16) |
| spti_time_scale | u(32) |
| spti_num_units_in_elemental_interval_minus1 | u(18) |
| if( spti_persistence_flag ) |  |
| spti_max_sublayers_minus_1 | u(3) |
| for( i = 0; i <= spti_max_sublayers_minus1; i++ ) { |  |
| spti_sublayer_interval_scale_factor[ i ] | ue(v) |
| spti_sublayer_synthesized_picture_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |

    - Source picture timing information SEI message semantics

…

spti_time_scale specifies the number of time units that pass in one second. The value of spti_time_scale shall not be equal to 0. For example, a time coordinate system that measures time using a 27 MHz clock has an spti_time_scale of 27,000,000.

spti_num_units_in_elemental_interval_minus1 plus 1 specifies the number of time units of a clock operating at the frequency spti_time_scale Hz that corresponds to the indicated elemental source picture interval of consecutive pictures in output order in the CLVS.

The indicated elemental source picture interval, also to be denoted by the variable ElementalSourcePictureInterval, in units of seconds, is equal to the quotient of spti_num_units_in_elemental_interval_minus1 plus 1 divided by spti_time_scale. For example, to represent an elemental source picture interval equal to 0.04 seconds, spti_time_scale may be equal to 27,000,000 and spti_num_units_in_elemental_interval_minus1 plus 1 may be equal to 1,080,000.

### Option 2

Alternatively, it is proposed to add a constraint to prevent signalling the value 0 for spti_num_units_in_elemental_interval.

spti_num_units_in_elemental_interval specifies the number of time units of a clock operating at the frequency spti_time_scale Hz that corresponds to the indicated elemental source picture interval of consecutive pictures in output order in the CLVS. The value of spti_num_units_in_elemental_interval shall not be equal to 0.

The indicated elemental source picture interval, also to be denoted by the variable ElementalSourcePictureInterval, in units of seconds, is equal to the quotient of spti_num_units_in_elemental_interval divided by spti_time_scale. For example, to represent an elemental source picture interval equal to 0.04 seconds, spti_time_scale may be equal to 27,000,000 and spti_num_units_in_elemental_intervalmay be equal to 1,080,000.

## Proposal 3

It is asserted that the derivation of SourcePictureInterval[ i ] uses a derived variable temporalReversalFlag which utilizes the syntax element spti_source_type which may not be present (when spti_source_type_present_flag is equal to 0) and spti_source_type has no inferred value when not present. A bug-fix is proposed to the derivation of temporalReversalFlag when spti_source_type_present_flag is equal to 0. The proposed text below derives the temporalReversalFlag variable to be equal to 0 when spti_source_type_present_flag is equal to 0 and uses the current derivation in TuC when spti_source_type_present_flag is equal to 1.

The proposed modifications are shown highlighted below compared to JVET-AF2032 [1].

spti_sublayer_interval_scale_factor[ i ], when present, specifies a scale factor used in determining the source picture interval of corresponding consecutive pictures in output order in the CLVS having TemporalId less than or equal to i. The value 0 may be used to indicate that the source picture corresponding to the current decoded output picture is identical to the source picture corresponding to the previous decoded output picture.

The indicated source picture interval associated with output pictures having TemporalId less than or equal to i, denoted by the variable SourcePictureInterval[ i ], in units of seconds, is derived as follows:

SourcePictureInterval[ i ] = ElementalSourcePictureInterval * spti_sublayer_interval_scale_factor[ i ] * ( 1 – 2 * temporalReversalFlag )				(8-X)

The variable temporalReversalFlag is equal to spti_source_type_present_flag ? (( spti_source_type & 0x10 )? 1 : 0) : 0.

Alternatively the modification may be specified as follows:

If spti_source_type_present_flag is equal to 1, tThe variable temporalReversalFlag is equal to ( spti_source_type & 0x10 )? 1 : 0. Otherwise (spti_source_type_present_flag is equal to 0), the variable temporalReversalFlag is equal to 0.

# Conclusion

This document proposes modifications to source picture timing information SEI. It is proposed to adopt the proposals in the next version of the text which includes SPTI SEI.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- JVET-AE2032, M. M. Hannuksela, J. Chen, S. Deshpande, S. McCarthy, Technologies under consideration for future extensions of VSEI (draft 2) October 2023

Acknowledgement
These research results were obtained from the commissioned research (No.05101) by National Institute of Information and Communications Technology (NICT), Japan.