| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0072 |
| --- | --- |

| Title: | EE2-1.12: IBC with upward-extended reference area |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Haruhisa Kato Kei Kawamura (KDDI) 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | yo-kidani@kddi.com |
| Source: | KDDI Corporation (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution reports the experimental results of Test 1.12 (IBC with upward-extended reference area) in the context of Exploration Experiments 2 (JVET-AF2024).

In Test 1.12, the reference area of IBC is further extended in an upward direction. The following two extension methods are tested for camera-captured content and screen contents: Test 1.12a is IBC with the extended reference area to the picture's upper side boundary; Test 1.12b with four CTU rows instead of the existing two CTU rows above the current CTU in HD resolution or less.

The experimental results reported show the best performance in Test 1.12a over ECM-11.0 as follows:

AI:

Overall: {0.00%	, 0.03%, 0.00%, 100.3%, 99.9%};

Class F: {-0.16%, -0.19%, -0.12%, 100.4%, 99.7%};

Class TGM: {-1.29%, -1.31%, -1.31%, 99.5%, 99.3%};

RA:

Overall: {0.00%	, -0.04%, -0.05%, 100.1%, 99.7%};

Class F: {-0.10%, 0.08%, -0.25%, 100.0%, 99.5%};

Class TGM: {-0.66%, -0.68%, -0.64%, 100.1%, 101.2%}.

Introduction

In the current ECM design, the reference area for IBC is extended to two CTU rows above current CTU, as shown in Figure 1. This extension was proposed by JVET-Z0153 [1], considering the extra memory required new intra coding tools in ECM such as IntraTMP.

Figure 1 Reference area for IBC when CTU (m, n) is coded. The blue block denotes the current CTU; green blocks denote the reference area; and the white blocks denote invalid reference area.

In the 32nd JVET meeting, JVET-AF0115 [2] investigated an IBC with upward-extended reference area only for screen content to achieve further coding performance of ECM. This technology is studied as Test 1.12 in this JVET meeting. Note that the proponent of JVET-AF0115 was requested to report the experimental results in which this technology is applied for camera-captured content in the 32nd JVET meeting, so Test 1.12 reports the results of both camera-captured content and screen content.

Test description

In Test 1.12, the two following extension methods on IBC-reference areas are tested:

Test 1.12a is IBC with the extended reference area to the picture's upper side boundary;

Test 1.12b with four CTU rows instead of the existing two CTU rows above the current CTU in HD resolution or less.

Experimental results

The proposed methods were implemented on top of ECM-11.0 [3] and tested according to the common test condition of AI and RA configuration as specified in the JVET-AF2017 [4] and JVET-AF2024 [5].

Experimental results of Test 1.12a and Test 1.12b are summarized below. More detailed results can be found in the attached spreadsheets.

Table 1 Test 1.12a results over ECM-11.0 for screen content.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class F | -0.16% | -0.19% | -0.12% | 100.4% | 99.7% | 98.2% | 100.8% |
| Class TGM | -1.29% | -1.31% | -1.31% | 99.5% | 99.3% | 98.9% | 101.8% |
|  | Random Access Intra Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class F | -0.10% | 0.08% | -0.25% | 100.0% | 99.5% | 100.2% | 100.6% |
| Class TGM | -0.66% | -0.68% | -0.64% | 100.1% | 101.2% | 99.3% | 101.2% |

Table 2 Test 1.12a results over ECM-11.0 for camera-captured content.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | 0.06% | -0.02% | 100.6% | 99.9% | 101.4% | 105.1% |
| Class A2 | 0.00% | 0.05% | 0.07% | 100.6% | 100.0% | 100.6% | 105.0% |
| Class B | 0.00% | 0.00% | -0.05% | 100.2% | 99.9% | 95.2% | 102.0% |
| Class C | 0.00% | 0.00% | 0.00% | 100.1% | 100.0% | 100.6% | 100.2% |
| Class E | 0.00% | 0.05% | 0.03% | 100.1% | 99.8% | 100.0% | 100.7% |
| Overall | 0.00% | 0.03% | 0.00% | 100.3% | 99.9% | 99.1% | 102.4% |
| Class D | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% | 100.0% | 99.9% |
|  | Random Access Intra Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.01% | -0.20% | -0.09% | 100.1% | 99.6% | 100.3% | 103.0% |
| Class A2 | -0.02% | 0.12% | 0.00% | 100.2% | 99.5% | 100.1% | 103.0% |
| Class B | 0.01% | -0.03% | -0.04% | 100.1% | 99.8% | 99.9% | 101.4% |
| Class C | 0.00% | -0.05% | -0.09% | 100.0% | 100.0% | 100.0% | 100.2% |
| Class E |  |  |  |  |  |  |  |
| Overall | 0.00% | -0.04% | -0.05% | 100.1% | 99.7% | 100.1% | 101.7% |
| Class D | 0.00% | 0.00% | 0.00% | 99.7% | 100.0% | 100.2% | 99.9% |

Table 3 Test 1.12b results over ECM-11.0 for screen content.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class F | -0.11% | -0.10% | -0.08% | 100.0% | 100.1% | 100.9% | 101.5% |
| Class TGM | -1.21% | -1.26% | -1.25% | 99.2% | 99.6% | 98.9% | 101.8% |
|  | Random Access Intra Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class F | -0.06% | 0.21% | -0.07% | 99.9% | 99.5% | 100.3% | 101.1% |
| Class TGM | -0.61% | -0.63% | -0.56% | 100.0% | 101.1% | 100.9% | 101.7% |

Table 4 Test 1.12b results over ECM-11.0 for camera-captured content.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | 0.00% | 0.00% | 99.8% | 100.4% | 100.0% | 102.2% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99.8% | 100.5% | 100.3% | 102.2% |
| Class B | 0.00% | 0.01% | -0.03% | 99.9% | 100.3% | 98.0% | 101.9% |
| Class C | 0.01% | 0.00% | 0.01% | 99.8% | 100.3% | 102.0% | 101.1% |
| Class E | 0.00% | 0.04% | 0.03% | 99.8% | 100.4% | 99.1% | 101.5% |
| Overall | 0.00% | 0.01% | 0.00% | 99.9% | 100.4% | 99.8% | 101.8% |
| Class D | 0.00% | 0.00% | 0.00% | 99.7% | 100.5% | 100.2% | 100.7% |
|  | Random Access Intra Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | 0.00% | 0.00% | 100.1% | 100.1% | 100.3% | 101.3% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100.1% | 100.0% | 99.9% | 101.3% |
| Class B | 0.00% | -0.07% | -0.05% | 100.0% | 100.1% | 99.9% | 101.5% |
| Class C | 0.00% | -0.05% | -0.09% | 99.9% | 99.9% | 100.2% | 101.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | 0.00% | -0.04% | -0.04% | 100.0% | 100.0% | 100.1% | 101.3% |
| Class D | 0.00% | 0.00% | 0.00% | 99.5% | 99.9% | 100.4% | 100.7% |

Conclusion

This contribution reports two test results of IBC with upward-extended reference area: Test 1.12a is IBC with the extended reference area to the picture's upper side boundary; Test 1.12b with four CTU rows instead of the existing two CTU rows above the current CTU in HD resolution or less. For screen content, some coding gains with negligible complexity increases are observed, and the Test 1.12a’s trade-off between the coding gain and complexity is better than that of Test 1.12b. It is recommended to adopt the Test 1.12a into the next ECM only for screen content.

Reference

- J. Xu and N. Zhang, “EE2-3.2: IBC Reference Area Extension,” JVET-Z0153, by teleconference, April 2022.

- Y. Kidani, H. Kato, and K. Kawamura, “EE2-3.2: IBC Reference Area Extension,” JVET-AF0115, Hannover, October 2023.

- ECM-11.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-11.0

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, October. 2023.

- V. Seregin, et al., “Exploration experiment on enhanced compression beyond VVC capability (EE2)”, JVET-AF2024, October 2023.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).