| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0073 |
| --- | --- |

| Title: | Non-EE2: Chained motion vector prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Haruhisa Kato Kei Kawamura (KDDI) 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | yo-kidani@kddi.com |
| Source: | KDDI Corporation (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution proposes a chained motion vector (MV) prediction as a new inter merge candidate list construction. Inspired by auto-relocated block vector (BV) prediction proposed in JVET-AF0129, the chained MV prediction candidates can be derived as the sum of the recursively traced MVs and BVs based on the pre-derived MVs. The chained MV prediction candidates are inserted after the HMVP prediction candidates.

On top of the ECM-11.0, simulation results {Y, U, V, EncT, DecT} of the proposed method for overall test sequences in CTC are reported below:

The length of the chained MV prediction trace depth is 1:

RA: {-0.10%, -0.20%, -0.27%, 99.9%, 100.5%};

LDB: {-0.42%, -0.20%, -0.32%, 100.1%, 102.1%};

LDP: {-0.28%, -0.46%, -0.54%, 100.1%, 101.7%};

No constraint for the length of the chained MV prediction trace path:

RA: {-0.12%, -0.19%, -0.20%, 99.7%, 100.7%};

LDB: {-0.38%, -0.04%, -0.16%, 100.1%, 102.5%};

LDP: {-0.28%, -0.43%, -0.25%, 100.1%, 102.1%}.

Introduction

In the 32nd JVET meeting, JVET-AF0129 [1] proposed the auto-relocated block vector (BV) prediction that derives a new BV prediction candidate for the IBC merge/AMVP candidate list construction to achieve further coding performance of ECM. The auto-relocated BV prediction candidates can be derived as the sum of the recursively traced BVs based on the pre-derived BVs. The auto-relocated BV prediction candidates are inserted after the history-based BV prediction candidates.

However, the current ECM does not have any MV prediction candidates derived by recursively tracing MVs based on pre-derived MVs for the inter merge candidate list construction. Therefore, there is room to further improve the coding performance of ECM in inter coding.

Proposed method

This contribution introduces a chained MV prediction (CMVP) into inter merge candidate list construction.

As shown in Figure 1, CMVP candidates can be derived as the sum of the recursively traced MVs and BVs based on the pre-derived MVs for the inter merge candidate list. For instance, a CMVP candidate, a set of motion vector MVk/m and reference picture RefPick/m can be derived by

MVk/m = MVk(0) + BVk(0) + MVk(1) +MVk(2) + … + MVk(m),

RefPick/m = RefPick(m),

where k and m indicate the number of merge index and trace depths of the CMVP.

Figure 1 An example of derivation of CMVP candidate.

When deriving MVk/m, MVk(m) is found by checking the existence of MVs or BVs in MV/BV storage corresponding to all five position of the current block as shown in Figure 2 (i.e., the center, top-left, top-right, bottom-left, and bottom-right of the current block).

Figure 2 Operation of referencing source and destination of tracing MVs in CMVP.

When pre-derived merge candidates targeting CMVP candidates has two MVs, a MVk/m is derived for each list (i.e., L0 and L1) and each trace depth. Up to two MVs can be derived for each list and each trace depth, and the MV set is sequentially inserted into inter merge candidate list.

The traceable reference pictures are only within the reference picture list.

CMVP candidates are inserted after HMVP candidates for the regular merge and TM merge.

When deriving CMVP candidates, hpelIfIdx, bcwIdx, licFlag, and mhpFlag are not inherited.

CMVP candidates are not derived when the TMVP is disabled.

Experimental results

The proposed method has been implemented on top of ECM-11.0 [2]. The simulations were conducted based on the common test condition and evaluation procedures for enhanced compression tool testing in RA, LDB, and LDP configuration [3]. The two following test results are summarized in Table 1. More detailed results can be found in the attached spreadsheets.

- Test 1: The length of the CMVP trace depth is 1 (i.e., m = 1):

- Test 2: No constraint for the length of the CMVP trace path

Table 1 Test 1 results over ECM-11.0

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | -0.16% | -0.11% | 99.8% | 100.6% | 100.4% | 100.0% |
| Class A2 | -0.11% | -0.20% | -0.35% | 99.8% | 100.6% | 100.2% | 99.7% |
| Class B | -0.12% | -0.27% | -0.34% | 100.0% | 100.4% | 99.7% | 99.9% |
| Class C | -0.13% | -0.15% | -0.25% | 100.1% | 100.5% | 100.0% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.10% | -0.20% | -0.27% | 99.9% | 100.5% | 100.0% | 99.9% |
| Class D | -0.09% | 0.02% | -0.04% | 100.1% | 100.7% | 99.9% | 100.0% |
| Class F | -0.05% | -0.06% | -0.11% | 100.1% | 101.1% | 99.9% | 99.9% |
| Class TGM | -0.18% | -0.20% | -0.14% | 100.1% | 103.9% | 99.6% | 100.0% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.25% | -0.21% | -0.07% | 100.0% | 101.5% | 100.0% | 99.9% |
| Class C | -0.37% | -0.04% | -0.67% | 100.3% | 102.4% | 100.0% | 100.0% |
| Class E | -0.78% | -0.37% | -0.28% | 100.2% | 102.9% | 99.3% | 100.0% |
| Overall | -0.42% | -0.20% | -0.32% | 100.1% | 102.1% | 99.8% | 99.9% |
| Class D | -0.48% | -0.32% | -1.51% | 99.6% | 101.4% | 100.1% | 100.0% |
| Class F | -0.96% | -0.69% | -0.76% | 99.4% | 104.5% | 99.7% | 100.2% |
| Class TGM | -0.45% | -0.57% | -0.50% | 100.1% | 102.1% | 100.3% | 100.1% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.28% | -0.37% | -0.60% | 99.9% | 100.8% | 100.0% | 100.1% |
| Class C | -0.32% | -0.03% | -0.26% | 100.1% | 102.4% | 100.0% | 100.1% |
| Class E | -0.24% | -1.18% | -0.82% | 100.3% | 102.4% | 100.0% | 99.8% |
| Overall | -0.28% | -0.46% | -0.54% | 100.1% | 101.7% | 100.0% | 100.0% |
| Class D | -0.36% | -0.87% | 0.20% | 100.2% | 101.3% | 99.9% | 100.0% |
| Class F | -0.29% | -0.19% | 0.29% | 99.8% | 101.5% | 99.9% | 99.8% |
| Class TGM | -0.18% | -0.34% | -0.50% | 100.3% | 102.2% | 100.8% | 100.2% |

Table 2 Test 2 results over ECM-11.0

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.06% | -0.30% | -0.14% | 99.6% | 100.8% | 100.0% | 99.9% |
| Class A2 | -0.08% | -0.17% | -0.28% | 99.6% | 100.8% | 100.0% | 99.9% |
| Class B | -0.14% | -0.20% | -0.29% | 99.8% | 100.4% | 99.7% | 100.0% |
| Class C | -0.15% | -0.11% | -0.07% | 99.9% | 100.9% | 100.0% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.12% | -0.19% | -0.20% | 99.7% | 100.7% | 99.9% | 100.0% |
| Class D | -0.04% | -0.07% | -0.07% | 100.1% | 101.1% | 100.1% | 100.0% |
| Class F | -0.10% | -0.11% | -0.27% | 100.0% | 101.8% | 100.1% | 99.8% |
| Class TGM | -0.18% | -0.23% | -0.22% | 100.0% | 103.5% | 100.0% | 100.1% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.31% | 0.05% | 0.13% | 100.0% | 101.8% | 100.1% | 99.8% |
| Class C | -0.36% | -0.17% | -0.89% | 100.2% | 102.8% | 100.1% | 100.1% |
| Class E | -0.53% | -0.01% | 0.33% | 100.3% | 103.4% | 99.6% | 100.2% |
| Overall | -0.38% | -0.04% | -0.16% | 100.1% | 102.5% | 99.9% | 100.0% |
| Class D | -0.34% | -0.63% | -0.71% | 99.6% | 101.6% | 99.9% | 100.0% |
| Class F | -0.58% | -0.77% | -0.82% | 99.5% | 103.7% | 99.5% | 100.1% |
| Class TGM | -0.49% | -0.46% | -0.55% | 99.9% | 103.4% | 100.6% | 100.2% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.26% | -0.58% | -0.52% | 100.0% | 101.3% | 100.0% | 100.3% |
| Class C | -0.36% | -0.34% | -0.43% | 100.3% | 102.8% | 100.0% | 100.1% |
| Class E | -0.22% | -0.29% | 0.44% | 100.1% | 102.7% | 100.0% | 99.9% |
| Overall | -0.28% | -0.43% | -0.25% | 100.1% | 102.1% | 100.0% | 100.1% |
| Class D | -0.27% | -0.91% | -0.05% | 100.6% | 101.9% | 99.9% | 100.0% |
| Class F | -0.17% | -0.36% | -0.27% | 99.9% | 101.1% | 100.0% | 99.8% |
| Class TGM | -0.25% | -0.42% | -0.52% | 100.2% | 104.2% | 100.3% | 100.3% |

Conclusion

This contribution proposes a chained motion vector prediction. The proposed method brings some coding gains with increasing minor decoder runtime. It is recommended to adopt the proposed methods into the next EE2.

Reference

- N. Zhang, K. Zhang, and L. Zhang, “Non-EE2: Auto relocated block vector prediction,” JVET-AF0129, Hannover, October 2023.

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-11.0

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AF2017, Hannover, October 2023.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).