| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0087-v1 |
| --- | --- |

| Title: | AHG9: On the generative face video SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Francesco Cricri, Honglei Zhang Nokia Technologies, Tampere, Finland | Email: | {miska.hannuksela, francesco.cricri, honglei.1.zhang}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes the following asserted improvements and fixes to the generative face video (GFV) SEI message proposed in JVET-AF0234:

- It is proposed to avoid the mandatory definition of TranslatorNN in each base picture and instead gate the definition of the TranslatorNN with a flag. The definition is required to be present in the first GFV SEI message that is included in or follows an IRAP picture unit in output order.

- It is proposed to avoid dummy pictures and instead allow multiple GFV SEI messages in a picture unit, e.g., carried within suffix SEI NAL units. Additional ingredients of this item include the following:

  - Each new instance of a GFV SEI message will invoke the generative neural network (NN) inference.

  - To differentiate between repetitions and new instances of GFV SEI messages, it is proposed to add a counter syntax element (gfv_cnt) in the syntax.

- It is proposed to enable referencing the same drive picture multiple times. This item includes the following ingredients:

  - It is proposed to indicate whether the current picture is a drive picture with a new flag, gfv_drive_pic_flag,

  - The semantics of gfv_drive_pic_fusion_flag continue to indicate if the drive picture is used as input to GenerativeNN.

- It is proposed to remove:

NOTE 5 – When current decoded picture corresponds to a driving picture, it should be marked as not for output purpose.

# Avoiding unnecessary repetitive definitions of TranslatorNN

## Problem statement

The definition of the TranslatorNN is present in each base picture. The definition re-uses the modes specified for the neural-network post-filter characteristics SEI message, i.e., a definition through a Uniform Resource Identifier (URI) or a Neural Network Representation (NNR, ISO/IEC 15938-17) bitstream. Both modes have relatively large byte count, typically tens or hundreds of bytes (for URI and NNR updates, respectively). If the base picture is not a random access point picture, it seems wasteful to require the presence of the definition of TranslatorNN.

## Overview of the proposal

It is proposed to gate the definition of the TranslatorNN with a flag. The definition is required to be present when the GFV SEI message is the first GFV SEI message that is included in or follows an IRAP picture unit in output order.

## Proposed specification text changes relative to JVET-AF0234

| generative_face_video ( payloadSize ) { | Descriptor |
| --- | --- |
| gfv_id | ue(v) |
| gfv_base_pic_flag /*indicate if current decoded output picture is a base picture*/ | u(1) |
| if( gfv_base_pic_flag ) { /*specify TranslatorNN( )*/ |  |
| gfv_translator_nn_flag | u(1) |
| if( gfv_translator_nn_flag ) { |  |
| gfv_nn_base_flag | u(1) |
| gfv_nn_mode_idc | ue(v) |
| if( gfv_nn_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| gfv_nn_reserved_zero_bit_a | u(1) |
| gfv_nn_tag_uri | st(v) |
| gfv_nn_uri | st(v) |
| } |  |
| } |  |
| } else /* current decoded output picture is a driving picture*/ |  |
| gfv_drive_pic_fusion_flag /*indicate if DrivePicture is input to GenerativeNN( )*/ | ue(v) |
| … |  |
| if( gfv_translator_nn_flag gfv_base_pic_flag && !gfv_key_present_flag) |  |
| if( gfv_nn_mode_idc = = 0 ) { |  |
| while( !byte_aligned( ) ) |  |
| gfv_nn_reserved_zero_bit_b | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| gfv_nn_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

gfv_translator_nn_flag equal to 1 specifies that the syntax elements defining the TranslatorNN are present in this SEI message. gfv_translator_nn_flag equal to 0 specifies that the syntax elements defining the TranslatorNN are not present in this SEI message. When gfv_translator_nn_flag is not present, it is inferred to be equal to 0.

When a GFV SEI message is the first GFV SEI message with a particular gfv_id value that is included in or follows an IRAP picture unit in output order within a CLVS, gfv_translator_nn_flag shall be present and equal to 1.

When gfv_translator_nn_flag is equal to 0 and TranslatorNN is referenced in the semantics of this SEI message, the applicable TranslatorNN is defined by the previous GFV SEI message with the same gfv_id value and gfv_translator_nn_flag equal to 1 in output order.

# Avoiding the use of dummy pictures

## Problem statement

The generative face video (GFV) SEI message proposed in JVET-AF0234 makes use of dummy pictures, e.g., pictures of the minimum allowed resolution, that are only used to contain GFV SEI messages.

This contribution asserts that dummy pictures seem to be redundant and cause unnecessary bitrate overhead.

## Overview of the proposal

It is proposed to allow multiple GFV SEI messages with different payload in the same picture unit. Each new instance of a GFV SEI message will invoke the generative neural network (NN) inference.

Instead of a dummy picture containing a GFV SEI message, it is proposed that an encoder may use a suffix SEI NAL unit containing a GFV SEI message in the picture unit that contains the base or drive picture referenced by the GFV SEI message.

It is a design principle to allow repetition of SEI messages within a picture unit. To differentiate between repetitions and new instances of GFV SEI messages, it is proposed to add a counter syntax element (gfv_cnt) in the syntax.

## Proposed specification text changes relative to JVET-AF0234

| generative_face_video ( payloadSize ) { | Descriptor |
| --- | --- |
| gfv_id | ue(v) |
| gfv_cnt | ue(v) |
| … |  |

gfv_cnt specifies a GFV SEI message instance count value for this gfv_id value within a picture unit.

The gfv_cnt of the first GFV SEI message, in decoding order, with a particular value of gfv_id within picture unit shall be equal to 0. When gfv_cnt assigned to currGfvCnt is greater than 0, a GFV SEI message with the same gfv_id value and gfv_cnt equal to currGfvCnt − 1 shall precede the current GFV SEI message in decoding order in the same picture unit.

The value of gfv_cnt shall be in the range of 0 to 65 535, inclusive.

If there are multiple GFV SEI messages that have gfv_cnt equal to 0, gfv_nn_base_flag present, and the same values of gfv_id and gfv_nn_base_flag and are present in the same picture unit, the SEI messages shall have the same SEI payload content.

Otherwise, if there are multiple GFV SEI messages with the same values of gfv_id and gfv_cnt present in the same picture unit, the SEI messages shall have the same SEI payload content.

gfv_base_pic_flag equal to 1 indicates the current decoded output picture corresponds to a base picture. gfv_base_pic_flag equal to 0 indicates the current decoded output picture does not correspond to a base picture or this SEI message does not specify syntax elements for a base picture. When gfv_cnt is greater than 0, gfv_base_pic_flag shall be equal to 0.

…

For a particular gfv_id value, the following process is used in increasing order of gfv_cnt to generate a video picture per each GFV SEI message that has gfv_base_pic_flag equal to 0 and a unique value of gfv_cnt within a picture unit:

…

# Enabling referencing the drive picture multiple times

## Problem statement

The generative face video (GFV) SEI message proposed in JVET-AF0234 defines the following types of pictures:

- Base picture, i.e., decoded output picture that may be used by a generative network to generate a novel face picture.

- Dummy picture, e.g., a picture of the minimum allowed resolution and only used to contain a GFV SEI message.

- Drive picture (also called primary coded picture) that can be fused by a generative neural network to improve background texture and facial details.

JVET-AF0234 specifies the inputs to GenerativeNN as follows:

- When gfv_base_pic_flag is equal to 0 and gfv_drive_pic_fusion_flag is equal to 0 and ChromaFormatIdc is equal to 0: inputBaseY, inputBaseKeyPoint, inputBaseMatrix, inputDriveKeyPoint, inputDriveMatrix

- When gfv_base_pic_flag is equal to 0 and gfv_drive_pic_fusion_flag is equal to 0 and ChromaFormatIdc is not equal to 0: inputBaseY, inputBaseCb, inputBaseCr, inputBaseKeyPoint, inputBaseMatrix, inputDriveKeyPoint, inputDriveMatrix

- When gfv_base_pic_flag is equal to 0 and gfv_drive_pic_fusion_flag is equal to 1 and ChromaFormatIdc is equal to 0: inputBaseY, inputDriveY, inputBaseKeyPoint, inputBaseMatrix, inputDriveKeyPoint, inputDriveMatrix

- When gfv_base_pic_flag is equal to 0 and gfv_drive_pic_fusion_flag is equal to 1 and ChromaFormatIdc is not equal to 0: inputBaseY, inputBaseCb, inputBaseCr, inputDriveY, inputDriveCb, inputDriveCr , inputBaseKeyPoint, inputBaseMatrix,, inputDriveKeyPoint, inputDriveMatrix

In our understanding, the sample arrays of the drive picture (inputDriveY, inputDriveCb, inputDriveCr) are used in GenerativeNN only when gfv_drive_pic_fusion_flag is equal to 1, i.e., only for generating a picture that coincides with the drive picture in output order and are not used for generating a picture that coincides with a dummy picture in output order.

It is asserted that the background represented by the drive picture may be applicable to generating pictures coinciding with dummy pictures.

## Overview of the proposal

It is proposed to indicate whether the current picture is a drive picture with a new flag, gfv_drive_pic_flag, The semantics of gfv_drive_pic_fusion_flag continue to indicate if the drive picture is used as input to GenerativeNN.

## Proposed specification text changes relative to JVET-AF0234

| generative_face_video ( payloadSize ) { | Descriptor |
| --- | --- |
| gfv_id | ue(v) |
| gfv_base_pic_flag /*indicate if current decoded output picture is a base picture*/ | u(1) |
| if( gfv_base_pic_flag ) { /*specify TranslatorNN( )*/ |  |
| gfv_nn_base_flag | u(1) |
| gfv_nn_mode_idc | ue(v) |
| if( gfv_nn_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| gfv_nn_reserved_zero_bit_a | u(1) |
| gfv_nn_tag_uri | st(v) |
| gfv_nn_uri | st(v) |
| } |  |
| } else { /* current decoded output picture is a driving picture*/ |  |
| gfv_drive_pic_flag | u(1) |
| if( gfv_drive_pic_flag = = 0 ) |  |
| gfv_drive_pic_fusion_flag /*indicate if DrivePicture is input to GenerativeNN( )*/ | u(1) ue(v) |
| } |  |
| … |  |

gfv_drive_pic_flag equal to 1 specifies that the current decoded picture is a driving picture. gfv_drive_pic_flag equal to 0 specifies that the current decoded picture is not a driving picture.

gfv_drive_pic_fusion_flag, when present, equal to 1 indicates the current decoded picture, which corresponds to a driving picture that may be used for fusion, the latest driving picture in output order for this gfv_id value may be input to GenerativeNN( ). gfv_drive_pic_fusion_flag equal to 0 indicates the current decoded picture the driving picture should not be input to GenerativeNN( ).

NOTE 3 – A gfv_drive_pic_fusion_flag value of 1 can be used, for example, to indicate that the current decoded picture can be used to improve face details or handle background changes.

…

When gfv_drive_pic_fusion_flag is equal to 1, the DrivePicture luma sample array inputDriveY, inputDriveCb and input DriveCr are derived as follows:

…

# Removing a contradictory NOTE

## Problem statement

The following NOTE is asserted to be contradictory:

NOTE 5 – When current decoded picture corresponds to a driving picture, it should be marked as not for output purpose.

GeneratorNN is performed as a post-processing operation and has access to the decoded driving picture only if it is an output picture. If the driving picture is marked not for output (i.e., ph_pic_output_flag for the driving picture is equal to 0), the decoder would not output the decoded driving picture and it would not be available as input to GeneratorNN.

The impact of GFV SEI message must be considered in any case for displaying the decoded and post-processed sequence as intended by the encoder.

## Proposal

It is proposed to remove:

NOTE 5 – When current decoded picture corresponds to a driving picture, it should be marked as not for output purpose.

# References

- B. Chen, J. Chen, Y. Ye (Alibaba), S. Wang (CityU), S. McCarthy, P. Yin, G.-M. Su, A. K. Choudhury, W. Husak, G. J. Sullivan (Dolby), "AHG9: Common text for proposed generative face video SEI message", JVET-AF0234.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).