| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0098-v3 |
| --- | --- |

| Title: | EE2-2.7: AMVP with SbTMVP mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao Jie Chen Yan Ye Xinwei Li | Tel: Email: | {ruling.lrl, jiechen.cj, yan.ye, sid.lxw}@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

# Abstract

This contribution reports the results of EE2 Test 2.7 which studies the AMVP with SbTMVP mode. Given a CU coded in AMVP with SbTMVP mode, the CU is predicted in a similar way as that of SbTMVP in merge mode except that the motion shift is signaled in the bitstream instead of being derived from neighboring blocks. The motion shift is obtained using MVP with a signaled MVD, and the number of MVDs is determined according to the percentage of the area of the blocks coded in the the AMVP with SbTMVP mode in the previous coded picture. Two tests with different setting for MVDs are evaluated. It is reported that on top of ECM-11.0, the overall coding performance impact for {Y, U, V, EncT, DecT} are:

1st test: RA is {-0.07%, -0.03%, -0.06%, 100.3%, 99.6%}, LB is {-0.36%, 0.05%, -0.01%, 100.4%, 98.8%}

2nd test: RA is {-0.09%, -0.02%, -0.07%, 100.4%, 99.8%}, LB is {-0.38%, -0.13%, -0.08%, 101.1%, 98.9%}

# Introduction

This contribution presents the simulation results of EE2 Test 2.7 which studies the AMVP with SbTMVP mode. Two tests with different setting for MVDs are evaluated.

# Proposed methods

In the proposed AMVP with SbTMVP mode, a CU is split into 4x4 subblocks, and each subblock derives its own motion from a corresponding subblock in the collocated picture. One collocated picture is used for non-low delay pictures, whereas two collocated pictures are used for low delay pictures. The corresponding subblock is obtained using MVP with a signaled MVD. The MVP is derived in the same way as that of AMVP mode. The number of MVD is determined according to the percentage of the area of the blocks coded in the proposed mode in the previous coded picture with the same temporal layer as follows:

- If the current picture is the first coded picture in a temporal layer, the number of MVD is set to 8, as shown in Figure 1(a).

- Otherwise, if the percentage of the area of the proposed mode is smaller than threshold1, the number of MVD is set to 4, as shown in Figure 1(b).

- Otherwise, if the percentage of the area of the proposed mode is smaller than threshold2, the number of MVD is set to 8, as shown in Figure 1(a).

- Otherwise, the number of MVD is set to 12, as shown in Figure 1(c).

In the 1st test, threshold1 and threshold2 are set to 3% and 6%, respectively. In the 2nd test, threshold1 and threshold2 are set to 4% and 7%, respectively.

When deriving the motion for subblocks, the reference pictures are fixed to the one with the reference picture index equal to 0. Then, the temporal motion vector scaling is applied to the motion vectors of the corresponding subblocks to obtain motion vectors for the subblocks.  When the corresponding subblock in collocated picture is non-inter coded, the temporal motion information is considered unavailable and the motion of center subblock of the CU is used instead. When the motion of the center subblock of the CU is also unavailable, the MVP with the signaled MVD is used instead.

When combining the proposed mode with other inter prediction modes, LIC, MHP are not applied. The OBMC mode is always enabled for the proposed mode. The AMVR is enabled for picture resolution larger than or equal to 3840x2160 luma samples. When the AMVR is enabled for a AMVP with SbTMVP coded block, the MVD magnitudes are increased from {4, 8, 12}-pel to larger values. In the 1st test, MVD magnitudes are {16, 32, 48}-pel when AMVR is enabled, whereas in the 2nd test, MVD magnitudes are {16, 24, 32}-pel.

To reduce the encoder complexity, when the POC distance between collocated pictures and current picture is larger than threshold3, the proposed mode is disabled for the current picture. The threshold3 is set to 2 and 4 for 1st and 2nd tests, respectively. Besides, in the 2nd test, the AMVP affine ME process is skipped if current best coding mode is SbTMVP or the proposed AMVP with SbTMVP modes at the encoder.

| (a) | (b) | (c) |
| --- | --- | --- |

Figure 1. Illustration of possible MVs of the proposed mode

# Simulation results

The proposed methods are implemented on top of ECM-11.0 and are tested under CTC for enhanced compression tool testing [4] with random access and low delay B configurations. Table 1 shows the simulation results of 1st test, where the threshold1, threshold2 and threshold3 are set to 3%, 6% and 2, respectively.

Table 1. Simulation results of 1st test

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | 0.00% | 0.00% | 100.4% | 100.2% | 100.0% | 100.2% |
| Class A2 | -0.06% | -0.05% | -0.07% | 100.4% | 99.8% | 100.4% | 99.8% |
| Class B | -0.04% | -0.02% | -0.03% | 99.9% | 99.2% | 100.1% | 100.1% |
| Class C | -0.16% | -0.06% | -0.16% | 100.6% | 99.5% | 100.3% | 100.3% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.07% | -0.03% | -0.06% | 100.3% | 99.6% | 100.2% | 100.1% |
| Class D | -0.08% | -0.03% | -0.05% | 100.3% | 99.6% | 100.5% | 100.3% |
| Class F | -0.09% | -0.11% | -0.08% | 100.0% | 99.7% | 100.3% | 100.4% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.16% | 0.48% | 0.37% | 99.7% | 98.8% | 100.3% | 99.4% |
| Class C | -0.11% | 0.05% | -0.36% | 100.6% | 98.8% | 100.8% | 100.3% |
| Class E | -1.04% | -0.67% | -0.18% | 101.3% | 98.9% | 102.5% | 100.2% |
| Overall | -0.36% | 0.05% | -0.01% | 100.4% | 98.8% | 101.0% | 99.9% |
| Class D | -0.75% | -0.69% | -1.09% | 101.3% | 97.1% | 100.8% | 100.3% |
| Class F | -0.08% | 0.23% | 0.18% | 99.7% | 99.5% | 99.5% | 99.9% |

Table 2 shows the simulation results of 2nd test, where the threshold1, threshold2 and threshold3 are set to 4%, 7% and 4, respectively.

Table 2. Simulation results of 2nd test

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | -0.07% | 0.00% | 100.6% | 99.4% | 100.2% | 100.1% |
| Class A2 | -0.06% | -0.02% | -0.11% | 100.6% | 99.7% | 100.0% | 100.0% |
| Class B | -0.06% | 0.03% | -0.06% | 100.2% | 99.7% | 100.1% | 100.2% |
| Class C | -0.18% | -0.05% | -0.13% | 100.3% | 100.4% | 99.9% | 100.3% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.09% | -0.02% | -0.07% | 100.4% | 99.8% | 100.0% | 100.2% |
| Class D | -0.06% | -0.03% | -0.07% | 100.7% | 100.6% | 100.6% | 100.4% |
| Class F | -0.08% | -0.06% | -0.02% | 100.3% | 101.5% | 100.0% | 100.2% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.16% | 0.26% | 0.25% | 101.4% | 99.3% | 100.1% | 99.6% |
| Class C | -0.09% | -0.19% | -0.36% | 100.8% | 99.2% | 100.2% | 100.6% |
| Class E | -1.11% | -0.68% | -0.26% | 100.7% | 97.9% | 99.3% | 100.1% |
| Overall | -0.38% | -0.13% | -0.08% | 101.1% | 98.9% | 100.0% | 100.1% |
| Class D | -0.61% | -0.38% | -1.17% | 100.7% | 98.5% | 100.8% | 100.3% |
| Class F | -0.31% | -0.06% | -0.11% | 100.9% | 99.6% | 100.5% | 99.7% |

# Conclusion

The contribution reports the results of the AMVP with SbTMVP mode. Two tests are evaluated. In the 1st test, it achieves 0.07% and 0.36% BD-rate reduction for RA and LB, respectively. In the 2nd test, it achieves -0.09% and -0.38% BD-rate reduction for RA and LB, respectively. Both tests have minor impact on encoder and decoder runtime. It is recommended to adopt one of the proposed tests into the next version of ECM.

# References

- R.-L. Liao, J. Chen, X. Li and Y. Ye, “Non-EE2: AMVP mode with subblock-based temporal motion vector prediction,” JVET-AC0103, Jan. 2023.

- R.-L. Liao, J. Chen, X. Li and Y. Ye, “EE2-2.2: AMVP with SbTMVP mode,” JVET-AD0152, Apr. 2023.

- R.-L. Liao, J. Chen, Y. Ye and X. Li, “AHG12: AMVP with SbTMVP mode,” JVET-AF0134, Oct. 2023.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AF2017, Oct. 2023.

# Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).