| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0110-v1 |
| --- | --- |

| Title: | AhG12: 16 States TCQ with State Exchange |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammet Balcilar, Karam Naser Ya Chen Franck Galpin Fabrice Le Léannec | Author(s) or Contact(s): | muhammet.balcilar@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes to use enhanced number of trellis code quantization (TCQ) states in dependent quantization (DQ). We use 16 states, where half of them are “dummy states”. These “dummy states” need to exchange their values with “original states” if their rate distortion (RD) cost is lower than RD cost of  the “original states”.

The following results {Y, U, V; EncT, DecT, VmPeak} are obtained on top of ECM-11.0:

- RA: {-0.07%, -0.07%,	-0.04%	; 100.6%, 99.6%, 101.2%}.

- LDB:{ -0.14%,	0.61%,	-0.19%	; 100.8%, 99.7%, 101.5%}.

# Introduction

TCQ maps transform coefficients to reconstruction points of two predefined scalar quantizers while taking RD cost into account. The selection of used scalar quantizer for current transform coefficient depends on the current state. The next coefficient’s state depends on current state and parity of the current coefficient’s quantization index. Quantization always starts with state=0 and state to state transitions are carefully designed to optimize RD cost. Currently 8 states TCQ is used.

It is known that using high number of states increases the search capability of TCQ, thus, it results better RD cost theoretically. Under the conditions that the source is i.i.d and has known distribution, designing new transition table with higher number of states may be trivial task. However, it is not the case for realistic task such as improving the performance of state-of-the-art video codecs. In this proposal, we introduce 16-states TCQ with its new state transition table.

# Proposed Method

We propose to use 16 states and following state to state transition table where default scalar quantizers are 
Q0,Q1. The transition depends on current state and the parity of the quantization index to be either even or odd shown by pn=0 or pn=1 respectively.

| state sn | quantizer used | next state sn+1 |  |
| --- | --- | --- | --- |
|  |  | pn=0 | pn=1 |
| 0 | Q0 | 0 | 2 |
| 1 | Q1 | 5 | 7 |
| 2 | Q0 | 1 | 3 |
| 3 | Q1 | 6 | 4 |
| 4 | Q0 | 10 | 8 |
| 5 | Q1 | 12 | 14 |
| 6 | Q0 | 11 | 9 |
| 7 | Q1 | 15 | 13 |
| 8 | Q0 | 8 | 10 |
| 9 | Q1 | 13 | 15 |
| 10 | Q0 | 9 | 11 |
| 11 | Q1 | 14 | 12 |
| 12 | Q0 | 2 | 0 |
| 13 | Q1 | 4 | 6 |
| 14 | Q0 | 3 | 1 |
| 15 | Q1 | 7 | 5 |

We call the first half of the states original states, while the second half of the states are called dummy states. Each original state has its own dummy state. Thus, (i+8)th state is dummy state of ith original state where i<8. After cost calculation of each coefficient, we exchange the value of dummy state and its original one if the dummy state’s RD cost is better. We call this exchange process as state exchange mechanism.

# Simulations

Proposed method is implemented on top of ECM-11.0 and are tested under CTC with random access and low delay B configurations.

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.05% | -0.18% | 0.05% | 101.3% | 98.2% | 101.7% |
| Class A2 | -0.05% | -0.10% | -0.09% | 99.8% | 99.8% | 102.2% |
| Class B | -0.11% | 0.08% | 0.06% | 100.2% | 99.9% | 100.4% |
| Class C | -0.06% | -0.14% | -0.18% | 101.0% | 100.0% | 100.9% |
| Class E |  |  |  |  |  |  |
| Overall | -0.07% | -0.07% | -0.04% | 100.6% | 99.6% | 101.2% |
| Class D | -0.08% | -0.30% | -0.42% | 101.8% | 100.8% | 100.5% |
| Class F | -0.02% | 0.00% | -0.24% | 102.2% | 103.5% | 101.1% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #DIV/0! |

|  | Low delay B Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.16% | 0.66% | 0.32% | 100.4% | 100.1% | 100.6% |
| Class C | -0.05% | 0.36% | -0.58% | 102.8% | 99.7% | 100.9% |
| Class E | -0.22% | 0.86% | -0.51% | 99.0% | 98.9% | 104.1% |
| Overall | -0.14% | 0.61% | -0.19% | 100.8% | 99.7% | 101.5% |
| Class D | -0.22% | 0.10% | -0.91% | 101.8% | 100.4% | 99.1% |
| Class F | -0.29% | -0.04% | 0.31% | 102.9% | 98.8% | 100.8% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #DIV/0! |

# Conclusion

This contribution proposes to use 16 state TCQ in DQ with state exchange mechanism. Considering the coding efficiency improvement, we recommend investigating the proposed method in next round of EE2 in the ECM-12.0.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).