| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0110-v4 |
| --- | --- |

| Title: | AhG12: 16 States TCQ with State Exchange |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammet Balcilar, Karam Naser Ya Chen Franck Galpin Fabrice Le Léannec | Author(s) or Contact(s): | muhammet.balcilar@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes to increase the number of trellis code quantization (TCQ) states in dependent quantization (DQ). The number of states is increased from 8 to 16, where the 8 new states use similar transition rule as the original ones.

The following results {Y, U, V; EncT, DecT, VmPeak} are obtained on top of ECM-11.0:

- RA: { -0.08%, -0.07%,	-0.04%	; 100.9%, 99.3%, 101.2%}.

- LDB:{ -0.14%,	0.61%,	-0.19%	; 101.0%, 100.1%, 101.6%}.

# Introduction

TCQ maps transform coefficients to reconstruction points to two predefined scalar quantizers while taking RD cost into account. The selection of the scalar quantizer used for current transform coefficient depends on the current state. The next coefficient’s state depends on the current state and the parity of the current coefficient’s quantization index. Quantization always starts with state 0 and state to state transitions are carefully designed to optimize RD cost. Currently 8 states TCQ are used.

It is known that using higher number of states increases the search capability of TCQ, thus, it results in better RD cost theoretically. Under the conditions that the source is i.i.d and has known distribution, designing new transition table with higher number of states may be a trivial task. However, it is not the case for realistic task such as improving the performance of state-of-the-art video codecs. In this proposal, we introduce 16-states TCQ with a new state transition table.

# Proposed Method

We propose to use 16 states and following state to state transition table where default scalar quantizers are 
Q0,Q1. The transition depends on the current state and the parity of the quantization index to be either even or odd shown by pn=0 or pn=1 respectively.

| state sn | quantizer used | next state sn+1 |  |
| --- | --- | --- | --- |
|  |  | pn=0 | pn=1 |
| 0 | Q0 | 0 | 2 |
| 1 | Q1 | 5 | 7 |
| 2 | Q0 | 1 | 3 |
| 3 | Q1 | 6 | 4 |
| 4 | Q0 | 10 | 8 |
| 5 | Q1 | 12 | 14 |
| 6 | Q0 | 11 | 9 |
| 7 | Q1 | 15 | 13 |
| 8 | Q0 | 8 | 10 |
| 9 | Q1 | 13 | 15 |
| 10 | Q0 | 9 | 11 |
| 11 | Q1 | 14 | 12 |
| 12 | Q0 | 2 | 0 |
| 13 | Q1 | 4 | 6 |
| 14 | Q0 | 3 | 1 |
| 15 | Q1 | 7 | 5 |

We call the first half of the states original states, while the second half of the states are called dummy states. New dummy states use the same transition rule as the original ones.

At encoder the value of each dummy state is exchanged with original ones based on the rdcost values in order to improve coding performance.

# Simulations

Proposed method is implemented on top of ECM-11.0 and is tested under CTC with random access and low delay B configurations. AI configuration results do not show good trade-off and the tool is disabled for it.

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.05% | -0.18% | 0.05% | 101.3% | 98.2% | 101.7% |
| Class A2 | -0.09% | -0.11% | -0.09% | 101.4% | 98.3% | 102.5% |
| Class B | -0.11% | 0.08% | 0.06% | 100.2% | 99.9% | 100.4% |
| Class C | -0.06% | -0.14% | -0.18% | 101.0% | 100.0% | 100.9% |
| Class E |  |  |  |  |  |  |
| Overall | -0.08% | -0.07% | -0.04% | 100.9% | 99.3% | 101.2% |
| Class D | -0.08% | -0.30% | -0.42% | 101.8% | 100.8% | 100.5% |
| Class F | -0.02% | 0.00% | -0.24% | 102.2% | 103.5% | 101.1% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #DIV/0! |

|  | Low delay B Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B* | -0.17% | 0.66% | 0.32% | 100.7% | 101.2% | 100.6% |
| Class C | -0.05% | 0.36% | -0.58% | 102.8% | 99.7% | 100.9% |
| Class E | -0.22% | 0.86% | -0.51% | 99.0% | 98.9% | 104.1% |
| Overall | -0.14% | 0.61% | -0.19% | 101.0% | 100.1% | 101.6% |
| Class D | -0.22% | 0.10% | -0.91% | 101.8% | 100.4% | 99.1% |
| Class F | -0.29% | -0.05% | 0.30% | 102.9% | 98.6% | 100.8% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #DIV/0! |

* 1 data point is replaced with anchor data

# Conclusion

This contribution proposes to use 16 states TCQ in DQ with state exchange mechanism. Considering the coding efficiency improvement, we recommend investigating the proposed method in next round of EE2 in ECM-12.0.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).