| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0111-v1 |
| --- | --- |

| Title: | EE1-2.2: Content-adaptive LOP filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ruiying Yang Maria Santamaria Francesco Cricri Miska M. Hannuksela Honglei Zhang Jani Lainema Antti Hallapuro Döne Bugdayci Sansli Nokia Technologies, Tampere, Finland | Email: | {ruiying.yang, maria.santamaria_gomez, francesco.cricri, miska.hannuksela, honglei.1.zhang, jani.lainema, antti.hallapuro, done.bugdayci_sansli}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution reports the results for the EE1-2.2 test which studies the loop filter in JVET-AF0056 with content adaptation per random access segment and application to the LOP1 loop-filter. A new type of Adaption Parameter Set (APS) called Neural Network Filter Update (NNFU) APS was designed and implemented to signal the content-adaptive parameters. It is reported that the average coding gains of this approach in the RA configuration are X.XX% (Y), X.XX(%) (Cb) and X.XX% (Cr) with respect to NNVC-7.1 anchor (NN intra tool ON and LOP1 tool ON).

# Introduction

The content-adaptive approach for neural network loop-filter, as detailed in JVET-AF0056 [1], is achieved by means of overfitting one of the models of the default LOP1 filter in NNVC-7.1. The generated weight-updates are coded with the MPEG-NNR standard [2, 3]. In this EE1 test the content adaptation is done independently for each RA segment and a new NNFU APS is designed to seamlessly carry the MPEG-NNR bitstream and other related data within the video bitstream. Both the training code and inference code are available in [4].

# Content adaptation via NN overfitting

The content-adaptive LOP filter is obtained by overfitting multiplier parameters that are applied after each convolutional block.

σW*x+b∙m,
where W is the kernel, * is the convolution operator, x is the input, b is the bias, m is the multiplier and σ is the activation function.

The network architecture is based on JVET-AD0156 [5] (Figure 1) and the difference is that the multiplier layers are added after each convolutional block.

3x3 conv 3x3x10xM

Leaky ReLu

3x3 conv 3x3xKxL

1x1 conv 1x1xMxM

Leaky ReLu

3x3 conv 3x3xKxK

1x1 conv 1x1xMxK

1x1 conv 1x1xKxM

Leaky ReLu

3x3 conv 3x3xKxK

1x1 conv 1x1xMxK

BS info

Qstep

Yx4+U+V 72x72x10

NxN

1x1 conv 1x1xKxM

Leaky ReLu

3x3 conv 3x3xKxK

1x1 conv 1x1xMxK

Y’x4+U’+V‘ 64x64x6

N’xN’

n Hidden Layers

1x1 conv 1x1xKxR

3x1 Sep conv 3x1xRxR

3x1xRxR

1x1 conv 1x1xRxK

1x3 Sep conv 1x3xRxR

CP decomposition

Layers approximated with CP decomposition

Separable Convolution layers

1x1 conv 1x1xMxR

1x1 conv 1x1xRxM

Figure 1. NN architecture from JVET-AD0156 [2].

The content adaptation is done per RA segment independently, and the weight-update is signalled within the first intra slice of such segment. In order to lower the overhead of the weight-update, coded with MPEG-NNR, only a subset of multiplier layers is overfitted. The multiplier layers to overfit are selected by L1 norm. Specifically, first, all multiplier layers are overfitted for a 10 iterations; then, the top K multiplier layers with highest L1 norm will be overfitted from scratch, where K denotes the number of trainable layers for each RA segment of a certain video sequence. The number K is determined by bitrate overhead percentage, as the following steps show:

- Let Vi be the bitrate of a coded video sequence (without the content adaptive technique), a ratio  ri is computed iteratively to find out how “large” the NNR bitstream should be to get coding gains:

Vi=Vi*ri+Vi
Vi will be the test bitrate used to compute the BD-rate. Starting from ri=0.1, at each step ri is increased by 0.1 until the BD-rate is no longer negative. Then, the second to last ri value is selected.

- From the sequence-wise overfitting, the average bitrate of a NNR bitstream with one layer multiplier S is computed. Let SiNNR be the bitrate of the NNR bitstream of one sequence, N the number of sequences and L the number of multiplier layers, S is calculated as:

S=i=1NSiNNRNL

- Let Pi be the number of RA segments in a video sequence, the number of layers to overfit in each segment is given by

K=ri*ViSPi

Thus, the number Kof trainable multiplier layers is dependent on the specific video sequence.

As the margin for any bitrate overhead is very small for class D, the proposed approach was not applied to this class. Nevertheless, for class D a more parameter-efficient approach was considered. All multiplier layers are overfitted, but only a subset of different values for the multipliers in a layer is overfitted. Let C be the number of channels in the multiplier layer, the C multipliers are grouped into M < C groups. During overfitting, all multipliers in a group are constrained to have the same value. M was chosen by following steps:

- Let Vi be the bitrate of a coded video sequence (without the content adaptive technique), a ratio  ri is computed iteratively to find out how “large” the NNR bitstream should be to get coding gains:

Vi=Vi*ri+Vi
Vi will be the test bitrate used to compute the BD-rate. Starting from ri=0.1, at each step ri is increased by 0.1 until the BD-rate is no longer negative. Then, the second to last ri value is selected.

- From the sequence-wise overfitting, the average bitrate of a NNR bitstream with one multiplier S is computed. Let SiNNR be the bitrate of the NNR bitstream of one sequence, N the number of sequences and MT the total number of multipliers, S is calculated as:

S=i=1NSiNNRNMT

- Let Pi be the number of RA segments in a video sequence, L the number of multiplier layers, the number of multipliers of each layer to overfit in each segment is given by

M=ri*Vi(S*L )Pi

Table 1 shows the details of NN information during the overfitting stage. As it can be seen in the training times, we have significantly reduced the time to overfit the filter on the video sequence, for example (class A) from 1 day to 1 hour. This time reduction is attributed to an improved implementation of the data storage and loading pipeline.

Table 1. Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Overfitting Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU NVIDIA A100-SXM-80GB |
|  | Framework: | PyTorch 1.12.1 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 100 epochs |
|  | Batch size: | 64 |
|  | Loss function: | Weighted-MSE |
|  | Training time (for 1 model): | A1 & A2: 1 hour (previous: 1 day) B: 15 mins (previous: 4 hours) C: 10 mins (previous: 20 mins) D: 5 mins F: 15 mins (previous 4 hours) |
|  | Training data information: | JVET CTC RA mandatory sequences |
|  | Training configurations for generating compressed training data (if different to VTM CTC): |  |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 72x72 |
|  | Learning rate: | 1e-3 |
|  | Learning rate update strategy | Constant learning rate |
|  | Optimizer: | ADAM |
|  | Preprocessing: | Convert 144x144 YUV420 signal to 6 72x72 blocks. Normalize to 0~1 |
|  | Mini-batch selection process: |  |
|  | Training data update strategy: |  |
|  | Other information: |  |

The estimated average time per sequence of encoding weight updates is about 30 minutes for class A, 15 minutes for class B & F, 3 minutes for class C, 1 minute for class D. At decoder side, the average time per sequence for NNR decoding and reconstructing the overfitted model is 5 seconds.

# Quantization

The floating-point overfitted models, obtained after decoding the NNR weight updates, were quantized to int16 using data-driven quantization, which falls under the category of post-training quantization and involves calibration of sequence data that was previously utilized in overfitting. In the quantization process, the naïve approach is applied for both parameters and activations, followed by adjustments to the model quantization parameters (model QPs) which are determined based on the statistical characteristics of the output values obtained from each layer within the neural networks. The estimated time for determining the model QPs is around 15 minutes for class A, 5 minutes for class B, less than 1 minute for class C, D & F. The determined model QPs are signalled to the decoder together with the MPEG-NNR bitstream representing the weight update.

# Neural network update signalling

The MPEG-NNR bitstream and the model QPs are carried within a newly designed NN filter update APS NAL unit, which is described in this section.

## Proposed specification text

### Adaptation parameter set

| adaptation_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| aps_params_type | u(3) |
| aps_adaptation_parameter_set_id | u(5) |
| aps_chroma_present_flag | u(1) |
| if( aps_params_type = = ALF_APS ) |  |
| alf_data( ) |  |
| else if( aps_params_type = = LMCS_APS ) |  |
| lmcs_data( ) |  |
| else if( aps_params_type = = SCALING_APS ) |  |
| scaling_list_data( ) |  |
| else if( aps_params_type = = NN_UPDATE_APS ) |  |
| nn_filter_update_data( ) |  |
| aps_extension_flag | u(1) |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

aps_params_type specifies the type of APS parameters carried in the APS as specified in Table 2. The value of aps_params_type shall be in the range of 0 to 2, inclusive, in bitstreams conforming to this version of this Specification. Other values of aps_params_type are reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this version of this Specification shall ignore APS NAL units with reserved values of aps_params_type.

Table 2. APS parameters type codes and types of APS parameters

| aps_params_type | Name of aps_params_type | Type of APS parameters |
| --- | --- | --- |
| 0 | ALF_APS | ALF parameters |
| 1 | LMCS_APS | LMCS parameters |
| 2 | SCALING_APS | Scaling list parameters |
| 3 | NN_UPDATE_APS | Neural-network filter update |

All APS NAL units with a particular value of aps_params_type, regardless of the nuh_layer_id values and whether they are prefix or suffix APS NAL units, share the same value space for aps_adaptation_parameter_set_id. APS NAL units with different values of aps_params_type use separate values spaces for aps_adaptation_parameter_set_id.

aps_adaptation_parameter_set_id provides an identifier for the APS for reference by other syntax elements.

When aps_params_type is equal to ALF_APS or SCALING_APS, the value of aps_adaptation_parameter_set_id shall be in the range of 0 to 7, inclusive.

When aps_params_type is equal to LMCS_APS, the value of aps_adaptation_parameter_set_id shall be in the range of 0 to 3, inclusive.

When aps_params_type is equal to NN_UPDATE_APS, the value of aps_adaptation_parameter_set_id shall be in the range of 0 to 3, inclusive.

### Neural network filter update data syntax

| nn_filter_update_data( ) { | Descriptor |
| --- | --- |
| for( i = 0; i < NumNNFilterLayers; i++ ) { |  |
| nnfu_biasq_diff_minus1[ i ] | se(v) |
| nnfu_convq_kernel_diff[ i ] | se(v) |
| nnfu_mulq_minus14[ i ] | se(v) |
| } |  |
| nnfu_bitstream_size_minus1 | u(15) |
| while( !byte_aligned( ) ) |  |
| nnfu_alignment_zero_bit | b(1) |
| for( i = 0; i <= nnfu_bitstream_size_minus1; i++ ) |  |
| nnfu_payload_byte[ i ] | u(8) |
| } |  |

nnfu_biasq_diff_minus1[ i ] plus 1 specifies the difference of the quantizer of the bias values of the i-th NN layer with index i relative to the respective quantizer of the base neural network.

nnfu_convq_kernel_diff[ i ] specifies the difference of the quantizer of the convolutional kernels of the i-th NN layer with index i relative to the respective quantizer of the base neural network.

nnfu_mulq_minus14[ i ] plus 14 specifies the quantizer for the multipliers of the i-th NN layer.

nnfu_alignment_zero_bit shall be equal to 0.

# Experimental results

Table 3 describes the loop-filter information at inference stage.

Table 3. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | SADL |
|  | Number of GPUs per Task |  |
|  | Number of Parameters (Each Model) | 53724 |
|  | Total Number of Parameters (All Models) | 107448 |
|  | Parameter Precision (Bits) | Int16 |
|  | Memory Parameter (MB) | 0.21 |
|  | Multiply Accumulate (kMAC/pixel) | 16.5 |
|  | Calculation Method | on a block basis |
| Optional |  |  |
|  | Total Conv. Layers | 46 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: |  |
|  | Patch size | 72x72 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

Table 4 shows the coding performance of the proposed approach in the RA configuration.

Table 4. Coding performance

|  | Random access Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-7.1 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |
| Class C | -0,47 % | -0,16 % | 0,46 % | 0,90 % | 1,16 % | 1,11 % | 97 % | 78 % |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |
| Class D | 0,07 % | 0,14 % | 0,75 % | 0,66 % | 0,72 % | 0,75 % | 97 % | 78 % | 100 % |
| Class F |  |  |  |  |  |  |  |  |  |
| Class H |  |  |  |  |  |  |  |  |  |

Please note that it is proposed not to use the approach for classes with small resolution, such as class D. Therefore, the results of class D are only reported for information purposes.

# Recommendation

Adopt the content-adaptive LOP filter described in this document into the next version of NNVC software.

# References

- R. Yang, M. Santamaria, F. Cricri, H. Zhang, J. Lainema, M. M. Hannuksela, A. Hallapuro. EE1-1.2.2: Content-adaptive LOP filter. JVET-AF0056. October 2023.

- ISO/IEC FDIS 15938-17. Information technology – Multimedia content description interface – Part 17: Compression of neural networks for multimedia content description and analysis.

- Test Model of Incremental Compression of Neural Networks for Multimedia Content Description and Analysis (INCTM), N0179. February 2022.

- https://vcgit.hhi.fraunhofer.de/jvet-af-ee1/VVCSoftware_VTM/-/tree/EE1-2.2

- J. N. Shingala, A. Shyam, A. Suneja, S. P. Badya, T. Shao, A. Arora, P. Yin, F. Pu, T. Lu, S. McCarthy. EE1-1.1: Complexity Reduction on Neural-Network Loop Filter. JVET-AD0156. April 2023.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).