| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0121-v2 |
| --- | --- |

| Title: | Non-EE2: Block vector guided LUT for chroma prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junyan Huo, Xue Hao, Minzhe Chen, Nengfu Qiu, Zhenyao Zhang, Yanzhuo Ma, Fuzheng Yang, Ming Li, Fan Wang, Jing Ren | Email: | jyhuo@mail.xidian.edu.cn xhao@stu.xidian.edu.cn mzchen@stu.xidian.edu.cn nfqiu@stu.xidian.edu.cn zhyzhang@stu.xidian.edu.cn yzma@mail.xidian.edu.cn fzhyang@mail.xidian.edu.cn myron.li@oppo.com wangfan6@oppo.com renjing1@oppo.com |
| Source: | Xidian University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

- Abstract

In this contribution, a new chroma intra prediction mode, block vector guided look-up table (BVG-LUT) mode, is proposed for screen content. This method is used for chroma components when dual tree is activated in intra slice. BVG-LUT uses block vectors of the co-located IBC or intraTMP coded luma blocks to identify the reference area for deriving the cross-component LUT, and uses the co-located luma samples for chroma prediction. For non 4:4:4 sequences, the luma samples are derived by multiple down-sampling filters. It is reported that the coding performance of ECM 11.0 is as below:

For AI configuration:

F: -0.02%, -0.49 %, and -0.44 %, with 103.4 % EncT, 101.5 % DecT.

TGM: -0.59%, -1.01 %, and -1.35 %, with 102.6 % EncT, 98.8 % DecT.

For RA configuration:

F: -0.04%, -0.22 %, and -0.20 %, with 101.0 % EncT, 99.8 % DecT.

TGM: xxx%, xxx %, and xxx %, with xxx % EncT, xxx % DecT.

Introduction

ECM 11.0 includes cross component prediction models such as CCLM, CCCM, GLM, to learn linear or nonlinear models, but these models may not be suitable for screen content. Generally, the number of color is limited in the screen content, which is more suitable for discrete mapping. In dual tree partition, the prediction process of palette is applied for luma and chroma components separately, ignoring cross-component correlations.

Proposed Method

In this contribution, a method of block vector guided look-up table (BVG-LUT) prediction is proposed for screen content coding. For the dual tree partition, BVG-LUT method uses block vectors of the co-located luma blocks, coded in IBC or intraTMP mode, to construct the BV’s list. Then, the cross-component LUT is derived from the reference area, which is determined by the BV’s list. The chroma prediction is obtained by mapping the down-sampled luma samples.

The five locations, as shown in Figure 1, in co-located luma area are scanned for determining the block vector to construct the BV’s list, similar to Direct Block Vector (DBV) mode in ECM-11.0.

Figure 1: Locations from co-located luma area.

The extra down-sampling filters used for BVG_LUT are shown in Figure 2 for the YCbCr 4:2:0 color format. The encoder tries the candidate down-sampling filters and signals an index of the best filter to the decoder.

Figure 2: The extra luma down-sampling positions for BVG_LUT method

Experimental results

The proposed method is implemented based on ECM-11.0 under common test conditions [2]. Table 1 summarizes the results with all intra and random access configurations.

Table 1. Simulation results of the proposed method

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class F | -0.02% | -0.49% | -0.44% | 103.4% | 101.5% | 100.9% | 100.0% |
| Class TGM | -0.59% | -1.01% | -1.35% | 102.6% | 98.8% | 99.9% | 99.9% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class F | -0.04% | -0.22% | -0.20% | 101.0% | 99.8% | 99.4% | 100.2% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |

Conclusions

This contribution proposes the BVG-LUT method to improve the coding efficiency of chroma intra prediction for screen content. The experimental results reportedly show that the proposed method achieves coding performance for screen content under AI and RA case. Therefore, it is suggested to investigate in EE.

- 5 References

- ECM-11.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

- M. Karczewicz, et. al., “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, February 2022.

- Patent rights declaration(s)

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).