| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0122-v1 |
| --- | --- |

| Title: | EE1-3.1: Deep Reference Frame Generation for Inter Prediction Enhancement |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Chen, Weijie Bao, Jianghao Jia, Zhenzhong Chen Wuhan University Zizheng Liu, Xiaozhong Xu, Shan Liu Tencent | Email: | {cinched, baoweijie, jiajh2021, zzchen} @whu.edu.cn {zizhengliu, xiaozhongxu, shanl} @tencent.com |
| Source: | Wuhan University, Tencent |  |  |

_____________________________

- Abstract

This contribution reports the EE1-3.1 results of NN-based inter prediction. Based on JVET-AF0208, this contribution has further simplified the network architecture to reduce computational complexity and utilized the license-free dataset HAA500 for training. Implemented on the VTM-11.0_NNVC-7.1, this method can achieve -0.64%/-0.33%/-0.31% bitrate savings for the Y/U/V components under the NNVC-RA configuration (with LOP and NN-intra enabled).

Introduction

JVET-AF0208 [1] proposes a deep reference frame generation method to enhance VVC inter prediction, which simplify the network and accelerate float32 SADL inference. During the encoding and decoding process, the network receives two reconstructed frames from the decoded picture buffer as inputs and generates a new frame. The generated frame will be put into the reference picture lists as an additional reference for the current encoding picture, which enhances the inter prediction.

In this contribution, the network architecture is further simplified and the license-free dataset HAA500 is utilized for training. The main modifications are summarized as:

- Replace the GridNet module with convolution operations and add one residual connection.

- Downsample the inputs of the small IFRNet module and subsequently upsample the resulting optical-flows.

- Reduce the channel number of some convolution operations and remove some convolution operations.

- Utilize the license-free dataset HAA500 [2] for training.

Proposed Method

Framework

Fig. 1: The framework of the proposed DRF method

The framework of the proposed DRF method is depicted in Figure 1. In this approach, two reconstructed frames chosen from the DPB are inputted into the deep reference frame generation network to synthesize a new frame. This synthesized frame is then added to both reference picture list0 and list1, using the same POC as the current encoding frame. It serves as an additional reference frame for subsequent encoding processes.

Network architecture

Fig. 2: The architecture of the DRF network

Figure 2 illustrates the architecture of the proposed DRF network. In this contribution, architectural simplifications have been made to reduce the number of network parameters. Specifically, the GridNet module are replaced with convolution operations and one residual connection is added. To reduce the complexity of optical-flow calculations, the inputs of the small IFRNet module are downsampled, and then the resulting optical-flows are upsampled. Besides, the channel number of some convolution operations is decreased and some convolution operations are removed. These modifications have resulted in a reduction in the number of parameters from 3800K to 2900K and a decrease in the multiply-accumulate operations from 504kMAC/pixel to 69 kMAC/pixel.

Dataset

The training dataset is generated from the license-free dataset HAA500, which is a manually annotated human-centric atomic action dataset for action recognition on 500 classes. All video frames are reformatted and cropped into image triplets with resolution of 256x256. Some selecting metrics including PSNR, TI, and SI are employed to avoid scene changes and excessively large or small movements within one triplet. Finally, the dataset is compressed by VTM-11.0_NNVC-6.0. Through the above processing, the training dataset contains 64, 500 triplets.

Network information in training stage

Table 1: Network information for the NN-based video coding tool testing in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GTX 3090 x 2 x 24GB |
|  | Framework | PyTorch v1.9.0 |
|  | Number of GPUs per Task | 2 |
|  | Epoch | 50 |
|  | Batch size | 3Kx24 |
|  | Loss function | L1 |
|  | Training time (for 1 model) | 24h |
|  | Training data information | HAA500 |
|  | Training configurations for generating compressed training data (if different to VTM CTC) | VTM-11.0_NNVC-6.0, QP{22, 27, 32, 37, 42} |
| Optional | Number of iterations |  |
|  | Patch size | 256x256 |
|  | Learning rate | 2e-4 (1e-4 for IFRNet) |
|  | Learning rate update strategy | ReduceLROnPlateau |
|  | Optimizer | ADAM |
|  | Preprocessing |  |
|  | Mini-batch selection process |  |
|  | Training data update strategy |  |
|  | Other information |  |

Network information in inference stage

Table 2: Network information for the NN-based video coding tool testing in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment |  |
|  | GPU Type | CPU only |
|  | Framework | SADL |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 2900K |
|  | Total Number of Parameters (All Models) | 2900K |
|  | Parameter Precision (Bits) | 32 (F) |
|  | Multiply Accumulate (kMAC/pixel) | 69 |
| Optional | Total Conv. Layers | 63 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size | 1 |
|  | Patch size | 256x256 (with 16 boundary padding) |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information |  |

Results

The method has been implemented on the VTM-11.0_NNVC-7.1, and the experimental results of the proposed DRF method under NNVC-RA configuration (with LOP and NN-intra available) are presented in Table 3.

Table 3: Performance of the proposed method (NNVC-RA, full length)

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_NNVC-7.1 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -0.77% | -0.15% | -0.51% | -1.18% | -0.82% | -0.95% | 113% | 294% |
| Class A2 | -0.97% | -0.96% | -0.12% | -1.47% | -1.62% | -0.79% | 109% | 295% |
| Class B | -0.41% | 0.03% | -0.37% | -0.74% | -0.71% | -1.25% | 112% | 314% |
| Class C | -0.57% | -0.44% | -0.25% | -1.25% | -1.22% | -1.21% | 108% | 304% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.64% | -0.33% | -0.31% | -1.11% | -1.05% | -1.09% | 111% | 304% |
| Class D | -0.23% | 0.29% | 0.62% | -0.89% | -0.84% | -0.49% | 107% | 297% |
| Class F | -0.22% | 0.08% | 0.25% | -0.52% | -0.15% | -0.48% | 121% | 778% |

Conclusions

This contribution presents a deep reference frame generation (DRF) method for VVC inter prediction enhancement. Implemented on the VTM-11.0_NNVC-7.1, the proposed method can bring -0.64%/-0.33%/-0.31% coding efficiency improvements under NNVC-RA configuration (with LOP and NN-intra enabled).

Reference

- W. Bao, X. Chen, J. Jia, Y. Zhang, Z. Chen, Z. Liu, X. Xu, and S. Liu. “EE1-2.1: Deep Reference Frame Generation for Inter Prediction Enhancement”. JVET-AF0208, 32nd Meeting, Hannover, DE, 13–20 October 2023.

- J. Chung, C. -H. Wuu, H. -R. Yang, Y. -W. Tai and C. -K. Tang, "HAA500: Human-Centric Atomic Action Dataset with Curated Videos," 2021 IEEE/CVF International Conference on Computer Vision (ICCV), Montreal, QC, Canada, 2021, pp. 13445-13454, doi: 10.1109/ICCV48922.2021.01321.

Patent rights declaration(s)

Wuhan University does not have any current or pending patent rights relating to the technology described in this contribution.

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).