| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0123_v0 |
| --- | --- |

| Title: | EE2-1.10: Bilateral Filtering for Intra Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenbin Yin1, Kai Zhang2, Yang Wang1, Zhipin Deng1, Lei Zhao1, Na Zhang1, Mehdi Salehifar2, Li Zhang2 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | yinwenbin.hit@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

Abstract

In this contribution, the results of EE2-1.10 are reported. In EE2-1.10, the Bilateral Filtering (BF) is applied for intra prediction samples to reduce the noise level. The intra prediction samples are filtered before they are used in later process.

On top of ECM-11.0, simulation results of EE2-1.10 are reported as below:

AI:  -0.12%, -0.03%, -0.02%, %, %.

RA: -0.02%, 0.07%, 0.05%, %, %.

Introduction

The intra prediction samples are generated from the intra reference samples. In general, the generated intra prediction samples may contain noises. The Bilateral Filtering (BF) is suitable to reduce coding noise inside prediction since it can achieve edge-protection.

Proposed Method

In EE2-1.10, the generated intra prediction samples are further filtered by BF to reduce the noise level. The BF used in EE2-1.10 is similar as the BF used in the loop-filtering stage. The filter length is kept as 2.

Simulation Results

The EE2-1.10 was implemented on top of the ECM-11.0 [1]. Simulations were performed following the common test conditions [2].

More detailed results could be found in the attached excel sheets. The results compared to ECM-11.0 are shown in Table 1.

Table 1 Performance of EE2-1.10

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.10% | -0.05% | -0.05% |  |  |  |  |
| Class A2 | -0.14% | 0.05% | 0.03% |  |  |  |  |
| Class B | -0.08% | -0.03% | -0.02% |  |  |  |  |
| Class C | -0.18% | -0.16% | -0.14% |  |  |  |  |
| Class E | -0.11% | 0.07% | 0.10% |  |  |  |  |
| Overall | -0.12% | -0.03% | -0.02% |  |  |  |  |
| Class D | -0.18% | -0.05% | -0.14% |  |  |  |  |
| Class F | -0.15% | -0.48% | -0.20% |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.05% | -0.05% | -0.07% |  |  |  |  |
| Class A2 | -0.03% | 0.11% | 0.08% |  |  |  |  |
| Class B | 0.01% | 0.24% | 0.02% |  |  |  |  |
| Class C | -0.02% | -0.10% | 0.17% |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.02% | 0.07% | 0.05% |  |  |  |  |
| Class D | -0.06% | -0.01% | 0.04% |  |  |  |  |
| Class F | 0.02% | -0.02% | -0.01% |  |  |  |  |

Conclusions

In this contribution, results of EE2-1.10 are reported. It is observed that EE2-1.10 can achieve promising coding performance with negligible encoding and decoding time increase. It is recommended to adopt EE2-1.10 into the next version of ECM.

Reference

- ECM-11.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-11.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-AF2017, DE, October 2023.

Patent Rights Declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).