| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0138-v1 |
| --- | --- |

| Title: | Non-EE2: Chroma intra prediction mode reordering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xinwei Li Ru-Ling Liao Jie Chen Yan Ye | Tel: Email: | {sid.lxw, ruling.lrl, jiechen.cj, yan.ye}@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

This contribution proposes to reorder the non-CCP chroma intra prediction modes. First, a chroma intra prediction mode list is constructed. Then the modes in the list are applied to predict the collocated luma block and reordered based on SAD cost. The first N modes in the list are allowed to predict the current chroma block. It is reported that on top of ECM-11.0, the overall coding performance impact for {Y, U, V, EncT, DecT} is {-x.xx%, -x.xx%, -x.xx%, xxx.x%, xxx.x%} for AI and {-x.xx%, -x.xx%, -x.xx%, xxx.x%, xxx.x%} for RA.

Introduction

In ECM-11.0, there are 7 non-CCP chroma intra prediction modes: chroma DBV mode, DM mode, chroma DIMD mode, and four default modes (i.e. planar mode, vertical mode, horizontal mode and DC mode).

For the chroma DBV mode, the first available BV found among the five positions {C, TL, TR, BL, BR} in the collocated luma block as shown in Figure 1 is scaled and used as the BV of the current chroma block.

For the DM mode, the intra prediction mode of the luma block covering the center position C in the collocated luma block as shown in Figure 1 is used to predict the current chroma block.

Figure 1 The collocated luma block and adjacent chroma block

For the DIMD chroma mode, a histogram of gradients (HoG) is build based on the luma samples in the second and third lines in the collocated luma block and the chroma samples in the second neighboring row and column. Then the intra prediction mode with the largest histogram amplitude value in the HoG is used for performing chroma intra prediction of the current chroma block.

A CCP flag is parsed to indicate whether a CCP mode or a non-CCP mode is used for the current chroma block. If the CCP flag is false, a fixed order is used to parse the non-CCP modes as shown in Figure 2. First, a chroma DBV flag is signaled to indicate whether the chroma DBV mode is used if at least one of the luma blocks in five locations of the collocated luma block is coded with IBC or intraTMP mode. Then a DM flag is signaled to indicate whether the DM mode is used if the chroma DBV flag is false. And if the DM flag is false, a chroma DIMD flag is signaled to indicate whether the chroma DIMD mode is used. Finally, a 2-bit fix-length coding is parsed to indicate which default mode is used if the chroma DIMD flag is false. Moreover, a fusion index is parsed if one of the DM mode, chroma DIMD mode and the four default modes is used to indicate whether to do fusion prediction with a CCP mode.

Figure 2 The parsing order of the non-CCP chroma modes in ECM

Proposed methods

In this contribution, it is proposed to reorder the non-CCP chroma intra prediction modes to further improve the coding efficiency of the intra chroma coding.

First, a non-CCP chroma intra prediction mode list is constructed in the following order:

- The 7 non-CCP chroma intra prediction modes in the current ECM;

- The collocated luma modes: the intra prediction modes from luma blocks in the positions of TL, TR, BL and BR in the collocated luma block as shown in Figure 1;

- The adjacent chroma modes: the intra prediction modes from adjacent chroma blocks in the positions of L', T', BL', TR' and TL' as shown in Figure 1;

- The collocated luma BVs: the available BVs from luma blocks in the positions of C, TL, TR, BL and BR in the collocated luma block as shown in Figure 1;

Then, the non-CCP chroma intra prediction modes in the list are used to predict the collocated luma block and the list is reordered based on the SAD cost between predicted samples and reconstructed samples in the collocated luma block. Only the first N modes in the list can be allowed for the current chroma block. The value of N is decided by the same condition of the chroma DBV mode. That is, if at least one of the luma blocks in five locations of the collocated luma block is coded with IBC or intraTMP mode, N is equal to 7, otherwise, N is equal to 6.

The syntax elements to parse the non-CCP chroma intra prediction modes are kept unchanged, however the meanings are changed as shown in Figure 3. Moreover, the proposed reordering method is only applied to dual tree.

Figure 3 The parsing order of the non-CCP chroma modes in the proposed method

Simulation results

The proposed methods are implemented on top of ECM-11.0 and are tested under CTC for enhanced compression tool testing [1]. Table 1 shows the simulation results of the proposed method.

Table 1. Simulation results of proposed method

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class C | -0.07% | -0.08% | -0.07% | 100.5% | 101.5% | 100.4% | 100.2% |
| Class E | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | -0.07% | -0.15% | 0.02% | 100.4% | 101.3% | 100.4% | 100.1% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | 0.00% | -0.06% | -0.11% | 99.2% | 99.6% | 100.2% | 100.1% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |

Conclusion

This contribution proposes a method to reorder the non-CCP chroma intra prediction mode. It is reported that -x.xx% BD-rate reduction can be achieved in all intra condition with . It is recommended to further study this proposal in the next round of EE2.

References

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AF2017, Oct. 2023.

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).