| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0149-v2 |
| --- | --- |

| Title: | Non-EE2: Improvements to Subblock Merge Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Chen Ru-Ling Liao Yeping Zheng Xinwei Li Yan Ye | Tel: Email: | jiechen.cj@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

In this contribution, it is proposed to improve the subblock merge mode by adding more subblock temporal motion vector prediction (SbTMVP) candidates to the subblock merge list and extending the template matching (TM) based refinement for SbTMVP candidates and affine candidates in subblock merge mode. It is reported that on top of ECM-11.0, the proposed improvements overall achieve { } BD-reduction in RA and { } BD-reduction in LD.

In the second version, more experimental results are provided.

Introduction

In ECM-11.0, TM is applied on the subblock merge mode to refine the motion for the current block. There are two different kinds of candidates in subblock merge list: SbTMVP candidate and affine candidate. For SbTMVP candidate, a CU is split into 4x4 subblocks of which the motion is derived based on the motion information of the motion field located by a motion shift in a collocated picture. Thus, each subblock may have different motion vectors (MVs) and reference indices. There are at most four SbTMVP candidates with different motion shifts and collocated pictures. The first candidate will be refined by TM and added as an additional candidate in the subblock merge list. Thus, totally there are at most five SbTMVP candidates in the subblock merge list. For affine candidate, based on the CPMVs inherited, subblock MVs are derived according to the affine model defined by CPMVs. Each subblock may have different MVs but shares the same reference index.

In ECM-11.0, when TM is applied on SbTMVP candidate, it only refines the motion shift which is used to locate the reference block in the collocated picture from which the subblock motion is derived. The subblock MVs which are actually used in the motion compensation are not refined.

When TM is applied on affine candidate, only base MV (denoted as base_mvx and base_mvy in equation (1)) of the affine model is refined. That is, all the CPMVs have the same MV offset after the refinement, and consequently all the subblocks have the same MV offset, too. Moreover, the current affine TM design is restricted to only apply to uni-prediction candidate and the bi-prediction affine candidate cannot be refined by TM although the majority of the affine blocks are bi-predicted.

mvx=a(x-x0)+b(y-y0)+base_mvxmvy=c(x-x0)+d(y-y0)+base_mvy				(1)

In this contribution, it is proposed to add more SbTMVP candidates to subblock merge list and extend the TM refinement for the subblock merge mode.

Proposed methods

For SbTMVP candidate, it is proposed to add the candidates for which the reference picture indices of all subblocks are equal to 0, and apply TM process to refine the subblock MVs. In this newly added subblock MV TM process, all the subblock MVs share the same MV offset. For the template, one top row and one left column are used. And the reference template consists of sub-templates which are fetched from the reference picture with neighboring boundary subblock MVs. And this template is also applied in the ARMC for SbTMVP candidate reordering. As SbTMVP candidate can have bi-prediction, we do TM refinement for reference picture list 0 first and then do it for reference picture list 1. For TM refinement for each reference picture list, the search process and the search pattern are the same with that of affine TM in ECM-11.0.

For affine candidate, it is proposed to extend the current affine TM to bi-prediction affine candidate and add a non-translation parameter TM process after the current base MV TM process to uni-predicted affine candidate. For bi-prediction affine TM, as there are two affine models, one for reference picture list 0 and the other for reference picture list 1, the one with larger initial TM cost is refined first. Then, the refined one is fixed and the other one is refined. For refinement of each affine model, only base MV is refined and the refining process is the same as that of affine TM in ECM-11.0 for uni-prediction affine candidate. For non-translation parameter TM process, one CPMV is fixed as the base MV and the four non-translation affine parameters a, b, c and d as in equation (1) of affine model are refined. With the refined affine parameters, the CPMVs can be calculated and then updated. The parameter search process is the same as that of affine DMVR in ECM-11.0 with only bilateral matching being replaced by template matching. And non-translation parameter TM refinement is only applied to uni-prediction affine candidate.

Experimental results

The proposed methods are implemented on top of ECM-11.0 and are tested under CTC for enhanced compression tool testing [1] with random access and low delay B configurations. Table 1 summarizes the simulation results of the proposed method.

Table 1. Simulation results of the proposed method

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.22% | -0.30% | -0.37% | 103.9% | 103.8% | 100.2% | 100.5% |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.16% | -0.21% | -0.19% | 104.4% | 104.5% | 100.3% | 100.6% |
| Class C | -0.22% | -0.21% | -0.19% | 104.2% | 106.3% | 99.8% | 100.9% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.11% | -0.08% | -0.27% | 103.4% | 106.1% | 100.8% | 101.1% |
| Class F | -0.14% | 0.02% | -0.12% | 102.3% | 105.5% | 99.9% | 100.9% |

|  | Low delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -0.12% | 0.30% | -0.23% | 102.2% | 103.8% | 101.0% | 100.7% |
| Class E | -0.41% | -0.96% | -0.08% | 101.3% | 102.8% | 100.5% | 100.6% |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.32% | -1.03% | -1.58% | 101.4% | 103.2% | 100.5% | 101.0% |
| Class F |  |  |  |  |  |  |  |

Conclusion

This contribution proposes to improve the subblock merge mode by adding more SbTMVP candidates to the subblock merge list and extending TM-based refinement for the SbTMVP candidates and affine candidates in subblock merge mode. By doing this, it is reported that { } overall BD-rate reduction is achieved in RA and { } in LB conditions. Thus, it is suggested to study this contribution in next round of EE2.

References

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AF2017, Jul. 2023.

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).