| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0151-v2 |
| --- | --- |

| Title: | EE2-1.2: IntraTMP with Merge Candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Naser, M. Radosavljević, F. Le Léannec, Y. Chen, T. Dumas, T. Poirier | Email: | karam.naser@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution reports the results for EE2 Test 1.2 proposed in JVET-AE0137. The method proposes to expand the list of candidates for IntraTMP with additional merge candidate block vectors. Merge candidate block vectors (similar to IBC merge) are acquired from the neighboring PUs and are tested in the IntraTMP search process. They are put in competition with other IntraTMP candidates They are ranked together with usual Intra TMP candidates, before the BV refinement phase.

The impact on coding efficiency and runtimes of the test over ECM-11.0 are summarized below {Y, U, V, EncT, DecT}.

Test 1.2:

AI-CTC: -0.07% -0.04% -0.05% 99.0%	100.3%

RA-CTC: -0.04% -0.03% -0.01% 99.5%	100.4%

AI-SCC: -0.13% -0.21% -0.03% 98.7%	101.6%

RA-SCC: -0.16% -0.31% -0.25% 100.4% 99.2%

# Introduction

IntraTMP mode search area consists of 6 regions R4, R5, R6, R1, R2, and R3 (see Figure 1). To build the candidate list, a sparse search is utilized first in which search region is subsampled by a factor of 3. Sparse candidates are ranked in ascending order based on their template cost, and top 30 candidates are held for the second stage. In the second stage, refinement process is carried out. The refinement is performed using a second template matching search around each of the top 30 candidates. A candidate list of up to 19 template matching block vectors is obtained at the end. They are ranked in ascending order according to their template cost. Finally, an index is signaled in the bit-stream to indicate which of the 19 BV candidates is actually being used for the current block.

Figure 1: IntraTMP search regions.

# Proposed method

IntraTMP template matching search does not take into consideration block vector information of the neighboring PUs. It is proposed to enrich the IntraTMP search with new candidates, consisting of block vectors from  neighboring PUs coded in IBC or IntraTMP mode. These new candidates are called IntraTMP merge candidates. The purpose is to improve IntraTMP by further including probable candidates during the IntraTMP search process. Previously, the readily available block vectors from neighboring blocks were not explored in IntraTMP mode despite being more correlated with the properties of the current block.

With respect to the usual IntraTMP mode, this means that not only BV coming from IntraTMP sparse search and refinement search may be tested, but also BVs coming from outside typical IntraTMP search range depicted in Figure 1.

Note, during IBC merge mode, the IntraTMP block vectors are used as IBC merge candidates, but not the other way around. This is a limitation of IntraTMP process.

The same template matching cost process to rank the new candidates among the already computed candidates such as those coming from the sparse search is used.

There are up to 25 local and non-local PUs that may be checked during the selection of a new (merge) candidate. Processing order of neighboring PUs is the same as for IBC merge mode. Whenever a block vector is found within the local or non-local neighborhood of the current PU, it is added to the current IntraTMP candidate list.

Their template cost is obtained in the same way as for other IntraTMP candidates. Merge candidates are ranked together with other candidates. Same as before, top 30 candidates are selected for further refinement.

# Test 1.2 - IntraTMP with Merge Candidates

After addressing comments during the last meeting and reducing the complexity of the ee2 test-1.2, the following is included in the final solution:

- If neighboring PU is coded in bi-IBC mode, include both BVs in the IntraTMP merge list. Otherwise, include just uni-IBC BV in the IntraTMP merge list.

- If neighboring PU is coded in IntraTMP mode and intra_tmp_idx > 0, besides a BV that is selected for that PU, include also best by cost IntraTMP candidate in the IntraTMP merge list. If intra_tmp_idx is equal to 0, selected and best by cost  BVs are the same so only one candidate is included in the IntraTMP list.

- Use only N out of M available merge candidates if M>N and prioritize merge candidates outside the ITMP search range.

By doing so, up to 50 IntraTMP merge candidates can be selected (M can be up to 50). However, in ee2 test-1.2 up to 10 candidates are used at the end (N=10). Hence only 10 candidates are tested and compared with other candidates.

# Experimental results

The proposal was implemented in ECM-11.0 and tested with CTC-AI and CTC-RA conditions. Results for the screen content are also included. The following results are obtained.

Table 1: performances of proposed test EE2-1.2 over ECM-11.0

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.01% | -0.08% | 0.01% | 100.4% | 99.4% | 100.0% | 100.7% |
| Class A2 | -0.08% | -0.01% | -0.02% | 100.8% | 101.7% | 100.2% | 102.6% |
| Class B | -0.09% | 0.03% | -0.16% | 97.7% | 100.3% | 98.0% | 101.1% |
| Class C | -0.02% | -0.11% | 0.09% | 98.7% | 99.2% | 100.5% | 100.2% |
| Class E | -0.13% | -0.04% | -0.11% | 98.6% | 101.1% | 101.5% | 100.6% |
| Overall | -0.07% | -0.04% | -0.05% | 99.0% | 100.3% | 99.8% | 101.0% |
| Class D | -0.05% | 0.05% | 0.07% | 98.1% | 99.6% | 99.7% | 100.1% |
| Class F | -0.08% | -0.22% | 0.13% | 99.3% | 101.5% | 97.2% | 100.4% |
| Class TGM | -0.18% | -0.19% | -0.19% | 98.2% | 101.6% | 99.8% | 100.8% |
| Overall SC | -0.13% | -0.21% | -0.03% | 98.7% | 101.6% | 98.5% | 100.6% |

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.01% | -0.13% | 0.03% | 101.2% | 103.0% | 99.8% | 100.0% |
| Class A2 | -0.02% | 0.00% | -0.03% | 98.8% | 98.1% | 100.4% | 100.0% |
| Class B | -0.04% | 0.04% | -0.15% | 99.8% | 98.8% | 100.3% | 100.0% |
| Class C | -0.08% | -0.05% | 0.16% | 98.4% | 102.2% | 100.0% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.04% | -0.03% | -0.01% | 99.5% | 100.4% | 100.2% | 100.0% |
| Class D | -0.05% | 0.10% | 0.06% | 98.5% | 98.7% | 99.9% | 100.0% |
| Class F | -0.07% | -0.15% | -0.06% | 100.3% | 99.5% | 100.0% | 99.2% |
| Class TGM | -0.25% | -0.47% | -0.43% | 100.5% | 98.9% | 99.5% | 100.2% |
| Overall SC | -0.16% | -0.31% | -0.25% | 100.4% | 99.2% | 99.8% | 99.7% |

# Conclusion

The proposed method enriches the IntraTMP search process with new BVs that are not previously checked by IntraTMP process. This is because it includes IBC block vectors, whose search region is larger than IntraTMP, and also because it possibly includes some candidates that are not tested in the first pass of template matching (sparse search with subsampled regions). It is also noted that the added BVs are correlated with the current block statistics as they were selected previously for predicting nearby blocks.

The method provides additional coding gain on top of ECM-11.0 with reasonable cost. It is proposed to include this method in the next version of ECM reference software.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).