| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0160-v1 |
| --- | --- |

| Title: | AHG9/AHG13: FGS Extension SEI message useful descriptors |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Stephan Wenger, Arianne Hinds, Gilles Teniou | Tel: Email: | {swenger, ahinds, teniou} @global.tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

While defining an FGS extension SEI message primarily addressing the functionality of spatial adaptation, it was felt useful to also include descriptive parameters of the film grain synthesis such as the type of grain that is meant to be reproduced, the purpose of the film grain, such as creative artistic intent or artefact masking, and its essentiality from a service viewpoint.

# Proposed parameters

## Type of grain

The parameters defined in the Film Grain Characteristics SEI message or in the proposed Film Grain Extension SEI message generally intend to reproduce film physical properties. The most popular grains are:

- 8mm films with coarse grains that was mostly used for private film recordings in 70s and 80s,

- 16mm films with noticeable grain that was used for documentaries and TV programs,

- 35mm films with a finer grain recognizable as the historical movie effect

- and 65-70mm with the finest grain that was used for major movie productions, particularly those requiring VFX.

Signalling the type of grain is meant to inform the client of the type of grain to be added to the video.

## Purpose of the film grain

As explained in the Technical Report on Film grain synthesis [1], applying noise on the video may address different objectives. The most obvious is for artistic intent. In this category there are 2 cases: either the film grain has been removed prior to encoding and the Film Grain SEI messages aims at reproducing the original version, or the grain is selected to provide an arbitrary artistic intent, like a video effect. Another major scenario in which film grain may be applied is for masking visual artefacts and bring back some textural effect (this is particularly true in cases with important loop filtered areas).

## Essentiality of the film grain

While describing the film grain, it may be beneficial to signal how important the post processing of FGS is for the targeted service. The level of importance may cover:

- high importance to preserve the original artistic intent,

- moderated importance as the subjectively preferred version.

- low importance because limited to only mask artefacts

It is proposed to define 4 levels of essentiality from 0 (the least essential) to 3 (the most essential).

# Proposal

## VSEI

The proposed implementation assumes the definition of the spatial adaptation parameters as part of the FGS Extension SEI message, alternatively a Film grain info SEI message could be also isolated.

8.X	Film grain extension SEI message

### 8.X.1	Film grain extension SEI message syntax

| film_grain_extension( payloadSize ) { | Descriptor |
| --- | --- |
| fge_cancel_flag | u(1) |
| if( !fge_cancel_flag ) { |  |
| [Film grain Characteristics + Spatial adaptation information…] |  |
| fge_description_information_present_flag | u(1) |
| if( fge_description_information_present_flag ) { |  |
| fge_film_grain_type | u(8) |
| fge_film_grain_purpose | u(4) |
| fge_film_grain_essentiality | u(2) |
| } |  |
| fge _persistence_flag | u(1) |
| } |  |
| } |  |

### 8.X.2	Film grain extension SEI message semantics

This SEI message provides the decoder with a parameterized model for a film grain synthesis process. The film grain synthesis process should be applied to the decoded pictures prior to their display.

Compared to the FGC SEI message defined in 8.5, this SEI message also provides the decoder with parameters defining how to apply locally the film grain synthesis as well as descriptive parameters of the film grain synthesis model and purpose.

[Film Grain Characteristics + Spatial adaptation information…]

fge_description_information_present_flag equal to 1 indicates the presence of the description of film grain characteristics.

fge_film_grain_type specifies the type of grain that is meant to be generated by the active FGC SEI message as specified in Table N. fge_film_grain_type shall be in the range of 0 to 6, inclusive. The values greater than 6 are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification.

Table N – fge_film_grain_type values

| Value | Description |
| --- | --- |
| 0 | Soft |
| 1 | Organic |
| 2 | Living |
| 3 | 8mm |
| 4 | 16mm |
| 5 | 35mm |
| 6 | 65-70mm |
| 7..255 | Reserved |

fge_film_grain_purpose indicates the purpose of the film grain characteristics as specified in table M. fge_film_grain_purpose shall be in the range 0 to 4, inclusive. The values greater than 4 are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification.

Table M – fge_film_grain_purpose values

| Value | Description |
| --- | --- |
| 0 | Original grain simulation |
| 1 | Artistic creative effect - movie |
| 2 | Artistic creative effect - video |
| 3 | Artistic creative effect - gaming |
| 4 | Visual artefact masking |
| 5..15 | Reserved |

fge_film_grain_essentiality indicates the essentiality of the film grain, ranging from 0 to 3, respectively from the least to the most important level.

NOTE – The level of essentiality is subject to interpretation by the entities in charge of setting and reading this parameter.

fge_ persistence_flag specifies the persistence of the film grain extension SEI message for the current layer.

fge_persistence_flag equal to 0 specifies that the film grain extension SEI message applies to the current decoded picture only.

fge_persistence_flag equal to 1 specifies that the film grain extension SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with a film grain extension SEI message is output that follows the current picture in output order.

# References

- "Text of ISO/IEC DTR 23002-9 Film grain synthesis technology for video applications", MDS22874_WG05_N00223

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).